# cf2023 colorForth

Contents

Summary	3

Keypad	3

Colour	5

Token Colours	6

Actions, not Words	7

Using colorForth	7

The colorForth Editor	9

colorForth and ASCII	12

Useful Commands	14

History	15

Micro Forth	15

polyForth and chipForth	15

Slowing down for C	15

ANS Forth and Windows	15

colorForth	16

The Future	16

Philosophy	17

Keep it simple	17

Chuck Moore from Fireside chat Nov 2020	17

The Maze Effect	17

Operating Systems and the Free Market Economy	18

Metadata and Files	19

The Library Trap	19

The Polarizing Effect	20

An anecdote :	20

colorForth Under the Hood	22

BIOS disk access	22

Video Display	22

Keyboard	22

Keypad	22

Appendix A Chuck Moore’s colorForth Primer	23

Words	23

Numbers	23

Definitions	23

Loops	24

Conditions	24

Compiler	24

Program	25

Appendix B NASM Source Code	26

Appendix C colorForth Source Code	149

Appendix D “Coloring Forth”	196

# Summary

colorForth is a dialect of the Forth programming language, both languages were invented by Charles H. “Chuck” Moore ; - Forth around 1968, and colorForth in the late 1990’s.

Click here to skip straight to the “Using colorForth” section to start having fun!

colorForth uses numbered “blocks” to store and edit its source code, rather than files. Each block is 1024 bytes in size and is paired with “shadow block”, on even and odd numbered blocks respectively.

The cf2023 system is created using the file cf2023.nasm assembled by the NASM Netwide ASSembler, together with colorForth blocks contained in the file cf2023Ref.img .

The resulting image file cf2023.img can be run by copying it onto a USB drive (e.g.using Rufus), or in a virtual machine such as Bochs, by running the Windows batch file go.bat.

Note that running colorForth in Bochs (version 2.6) has some differences to running natively :

- the Processor clock counter does not work

- the hardware Random Number Generator is not emulated

- the Programmable Interrupt Controller cannot be re-programmed. If you run the Interrupt demo on block 256, please do not try to save the system – power down Bochs instead.

The display of the source code uses a 16 x 24 x 16 bit colour, fixed-width font, in colours that indicate the function of each word, on a 1024 x 768 pixel black background.

Keypad entry uses a 27 key subset of a normal keyboard, these key functions are described by the “keypad display” mnemonic, seen in the bottom right-hand corner of the display. Some function keys are also used to provide compatibility with conventional systems.

Figure 1 The keypad display

With the index fingers of each hand placed on the ‘F’ and ‘J’ keys, the keys used are that row and the rows above and below, plus the ‘N’, ‘space’ and ‘AltGr’ keys :

27 Keys

3 for each finger - up center down

3 for right thumb - up center right

colorForth is a concise language. The code to produce the startup logo screen pictured above is :

The three red words 5* , cf and logo are new words defined in terms of the other (green) words. Newly defined words can then be used as green words in later definitions. Literal numbers may be decimal or hexadecimal, the latter being marked by a $ symbol.

show sets the display task to execute the code following, using a cooperative multitasker. This allows dynamic displays which show the changing state of variables and hardware registers.

This is the documentation for the cf2023 distribution of colorForth that I maintain, distribute and publicise.

The source code and complete system (for Windows) is available from : https://www.inventio.co.uk/colorforth

This is a snapshot of Work In Progress and may be subject to change in the future.

Feedback welcome : howerd@inventio.co.uk

Enjoy!

Howerd Oakford  2023 Apr 04

# Colour

While the name “colorForth”, the coloured representation colorForth and the colourful appearance of the display all emphasise colour (spelled “color” in the USA), in fact the fundamental principles in colorForth go way beyond colour. Colour in this context is just one way of conveying meta-information about a computer program. For example, conventional Forth uses ‘:’ to indicate the definition of a new Forth word, colorForth uses the colour red together with starting the definition on a new line.

While conventional Forth can have coding style standards that usually specify that colon definitions start on a new line, this not required. In colorForth, red tokens (that start a new word definition) are displayed on a new line automatically. There are some special blue tokens that modify this default behaviour, and this can in any case be changed, if desired, in the NASM source code.

In the cf2023 distribution of colorForth, pressing the F4 function key toggles between colorForth mode and a more conventional Forth display. This is easy to do because the information and meta-information (information about the information) are stored in 32 bit tokens, and can be displayed in any desired way. The F4 function also makes it easier for people who are colour-blind to read the code.

To illustrate this, here is the code for displaying in “colour-blind” mode in a version of the editor written in colorForth. The first 9 lines define the colours and additional text to display for each change of token “colour” ( i.e. state). The word state! is called immediately before each token is displayed, and compares the current and previous token colours and jumps to the correct action in the .old colour and .new colour tables defined by ‘jump’.

The same block in “colour-blind” mode looks like this:

If you are familiar with conventional Forth you will recognize the ‘:’ (red) as the start of a new word definition, the ‘(‘ and ‘)’ brackets to define (white) comments and the ‘[‘ and ‘]’ square brackets to  wrap “immediate” (yellow) words.

The ‘mvar’ word represents a magenta variable, an interesting feature that is easy to implement in colorForth. When executed, a magenta variable returns the address of the next 32 bit cell in the source code block. If a value is stored into a magenta variable the source code is effectively changed, and due to the dynamic update of the display task the new value will be seen immediately on the screen.

## Token Colours

The following colours and their meaning is described below, from file cf2023.nasm line 4024 :

actionColourTable:          ;    * = number

dd colour_orange        ; 0     extension token, remove space from previous word, do not change colour

dd colour_yellow        ; 1     yellow "immediate" word

dd colour_yellow        ; 2  *  yellow "immediate" 32 bit number in the following pre-parsed cell

dd colour_red           ; 3     red forth wordlist "colon" word

dd colour_green         ; 4     green compiled word

dd colour_green         ; 5  *  green compiled 32 bit number in the following pre-parsed cell

dd colour_green         ; 6  *  green compiled 27 bit number in the high bits of the token

dd colour_cyan          ; 7     cyan macro wordlist "colon" word

dd colour_yellow        ; 8  *  yellow "immediate" 27 bit number in the high bits of the token

dd colour_white         ; 9     white lower-case comment

dd colour_white         ; A     first letter capital comment

dd colour_white         ; B     white upper-case comment

dd colour_magenta       ; C     magenta variable

dd colour_silver        ; D

dd colour_blue          ; E     editor formatting commands

dd colour_black         ; F

# Actions, not Words

I strongly recommend that you run cf2023 as a program on a suitable computer. There are two ways of doing this :

- Copy the binary image file cf2023.img directly onto a USB drive, and boot the computer using this drive.

- Run cf2023 in a bochs environment under Windows. Double click on the file go.bat in the cf2023 distribution to do this.

This is because “the map is not the territory” – both Forth and colorForth provide an interactive environment that is best experienced, rather than discussed or thought about.

# Using colorForth

Note : The space bar is used in colorForth as if it is the Enter key.

The system starts up in the Editor - press the space bar to exit, then the space bar again to enter numeric mode.

Enter a number and press the space bar again. Press the space bar again and enter the next number. After entering two numbers e.g. ‘1 1 ‘press the AltGr key to see the Alternative alpha keypad, and press ‘+’:

1 1 +

Above shows the state just before the final space bar. The ‘123’ represents the current word being typed, in this case ‘+’ . The two ‘1’s are on the stack, with the current word being typed, and the stack is shown in the bottom left of the screen. The orange ‘64’ in the top right is the current block number.

Pressing the space bar now will execute ‘+’ which adds the top two stack items, in this case ‘1’ and ‘1’, and will return ‘2’.

On a running cf2023 system :

Press F1 for the help screen. Press repeatedly to cycle round the main load block and its documentation shadow block.

Press F2 to toggle decimal and hexadecimal display of numbers.

Press F3 to toggle display or hiding of blue token words. These words may be added or deleted like any other word in the editor. The following special blue words are detected and acted on by the editor display :

| Blue word | Function |
| --- | --- |
| cr | one CR |
| , | one CR |
| -tab | move to the next 24 multiple column, disabling a CR for a red word |
| tab | move to the next 24 multiple column |
| br | two CRs |
| -cr | disable a CR for the next red or magenta word |
| cr+ | one CR and indent 3 spaces |
| blue | no action |
| tab3 | align to next 3 space column |
| . | add one space |
| .. | add two spaces |
| ... | add three spaces |
| .... | add four spaces |

Press F4 to toggle normal and “colour-blind” display mode, also runs the editor.

Press F6 to toggle between the current and last edited block.

# The colorForth Editor

When the editor is run by typing e ,  256 edit , or by pressing F4, the keypad looks like this :

The mnemonics mean :

| Mnemonic | Function | Qwerty key (UK) | Qwertz key (German) |
| --- | --- | --- | --- |
| S | White text CAPITALS | W | W |
| C | White text first letter only Capital | E | E |
| t | White text lower case | R | R |
| y | Yellow – immediate actions, not compiled | U | U |
| r | Red token – create a new word | I | I |
| g | Green – compiled token | O | O |
| * | Toggle main and shadow blocks | P | P |
| j | Jump to last edited block | F | F |
| l u d r | Cursor arrows, left up down right | J K L ; | J K L Ö |
| a | Silver (gray) token | Y | Y |
| b | Blue token | X | X |
| - | Decrease block number by 2 | M | M |
| m | Magenta variable token | , | , |
| c | Cyan | . | . |
| + | increase block number by 2 | / | - |
| x | Delete token | N | N |
| . | Exit the editor | Space bar | Space bar |
| i | Insert previously deleted token | AltGr | AltGr |
| ¦-left-arrow | Find previous token name RSN | A | A |
| f | Find token name RSN | S | S |
| ¦-right-arrow | Find next token name RSN | D | D |

- RSN = Real Soon Now…

Use the + and - keys to select the block to edit (also PgUp and PgDn), or press the space bar to exit the editor and type

256 edit to edit block 256

Use the l u d r arrow keys to move the cursor to the required location. The arrow keys and the Home and End keys can also be used to move the cursor.

Pressing the keyboard Enter key executes the red word or locates the green word at the cursor.

Choose a colour, for example red to create a new word – the keypad now looks like this :

Press the required letters until you have finished entering the word, then press the space bar (‘9’).

The display then turns green to enter a word to be compiled :

Press the required letters until you have finished entering the word, then press the space bar (‘9’).

Repeat for the next word.

If you press the N key (‘.’) you will return to the main Editor keypad,so you can choose a different colour, move the cursor or select a new block to edit.

The AltGr key, labled * in the keypad toggles between the main alpha and Alternate alpha text entry mode :

Press the required letters until you have finished entering the word, then press the AltGr key to return to alpha mode, then the space bar (‘9’) as described above.

Pressing the space bar in alpha mode will change to numeric mode, pressing the AltGr key toggles between decimal and hexadecimal display mode :

The following diagram show the different keypad mnemonics and the keys to press to change them :

The other colours, and Editor / Interpret mode all have similar functionality.

Editor mode is shown by the two horizontal lines above and below the keypad.

When you have found and/or edited the block that you would like to run, leave the editor by pressing the space bar. You are now in Interpret mode (no more horizontal lines).

Type the command you would like followed by the space bar.

# colorForth and ASCII

The colorForth keypad has only 48 characters – this edition allows three ASCII (or UTF-8) characters to be encoded into a standard colorForth 32 bit token. This increases compatilbility with other Forth systems.

; ASCII / UTF8 support. If the first Shannon-Fano encoded letter is a 4 bit NULL,

; display the next 24 bits as three ASCII characters.

; $03e3c009 is displayed as ‘><’

cf2023.nasm ASCII Support code

This is the code to support decoding and display of ASCII characters.

lowercase:   ; display a white text word in normal lower-case letters

call white

showSF_EDI_: ; ( -- ) \ display a Shanon-Fano encoded token pointed to by  edi  in the current colour

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]   ; fetch the next token – drops through to showShannonFano

showShannonFano:    ; ( token -- ) \ display the Shannon-Fano encoded token on TOS

; ASCII / UTF8 support. If the first Shannon-Fano encoded letter is a 4 bit NULL,

; display the next 24 bits as three ASCII characters.

mov _SCRATCH_, _TOS_            ; save the token value

and _SCRATCH_, 0xF0000000

cmp _SCRATCH_, 0x00000000

jnz .forward

; display as three ASCII characters

mov _SCRATCH_, _TOS_

mov _TOS_, _SCRATCH_

shr _TOS_, 20

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

mov _TOS_, _SCRATCH_

shr _TOS_, 12

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

mov _TOS_, _SCRATCH_

shr _TOS_, 4

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

; arrive here if an ASCII character is an ASCII NULL, or if all three have been emitted

.null_terminator:

call space_                 ; display a space character at the end of the word

_DROP_

ret

.forward:

; display as Shannon-Fano encoded token name

and _TOS_, byte -0x10   ; and _TOS_, 0xFFFFFFF0 ignore token colour when displaying the letters

lowercasePrimitive: ; ( token -- ) \ display the given Shanon-Fano encoded word in the current colour

call unpack

jz lowercasePrimitiveEnd

call emitSF_

jmp lowercasePrimitive

lowercasePrimitiveEnd:

call space_

_DROP_

_DROP_

ret

# Useful Commands

| Command | Action |
| --- | --- |
| e | Run the Editor displaying the last block edited |
| 64 edit | Run the colorForth Editor displaying block 64 |
| xx | Run the colorForth Explorer |
| ll | Load the current block displayed by the Editor |
| vv | View the last block loaded by the command ld |
| uu | Undo all changes to the current block |
| ss | Save the current block to disk |
| save | Save the entire system to disk. You can change the USB stick to save a backup. |
| sa | Save and return to the Editor |
| logo | Show the colorForth logo screen |
| empty | Remove all compiled definitions since mark was called |
| mark | Mark the current system state for empty |
| $1000 dump | Dump the 16 32 bit cells starting at address $1000 |
| bye | Exit the system, discarding all edits since the last save, sa or ss |
| hlp | Update the hardware system info and display the start block. This is currently block 64, and displays a list of Apps. |
| life | Run the Conway’s Game of Life demo. Press the space bar to exit (marked by a ‘.’ In the keypad mnemonic), then type xx and press the space bar. Scroll to “Conways Game of Life” and press the e key to view the source code, then press the ‘*’ key in the Editor to view the documentation shadow block. |
| staks | Display the four task’s stacks. ‘U’ means Unused, ‘.’ means used. Checks that the stacks are not growing or shrinking… |

You can also use the cursor control keys in the editor or the arrow, Home and End keys to move the cursor immediately after a red word (in either Editor or Interpret mode), then press the Enter Key (on the QWERTY or QWERTZ keyboard, not the keypad) to execute that word.

# History

## Micro Forth

I discovered Forth, in the form of Micro Forth for the RCA CDP1802 processor chip, around 1979.

I was working for a small startup company in the UK, developing a Grain Moisture Meter, and was planning to use the RCA CDP1802 assembler. When I unpacked the newly arrived RCA COSMAC development system (an 8 bit CDP1802 processor clocked at 2 MHz, with 4K RAM) I noticed a single sheet of paper advertising MicroForth, and promising fast development times, small code size and fast run-time speed - all of these claims I later found out to be true.

Having convinced my boss that Micro Forth would be a good investment, I waited for some weeks for the 8-inch floppy disk to arrive by post from California, and followed the Quick Start Guide.

I typed 1 1 + . and saw the result : 2 . I could talk to the computer, and it could reply, and it could even do maths. I was impressed, and at that moment my career as a computer programmer changed direction to the Forth side – I was hooked.

## polyForth and chipForth

Having completed the Moisture Meter project , in the mid 1980’s I looked for more Forth work, and got contracts working for COMSOL in the UK (a Forth software development house and supplier of Forth, Inc. products such as Micro Forth, polyForth and chipForth). It was through COMSOL that I got a job working on the Riyadh Airport HVAC system, for AVCO in Huntsville Alabama.

## Slowing down for C

Towards the end of the 1990’s the demand for Forth had dimished, so I learned C. My first C contract was a six month project – when I discovered the details of the contract I was horrified – in Forth, I would normally have finished this sort of project in 6 weeks.

As I polished my C/C++ and later C# skills I found that it was not good etiquette to mention Forth at work, especially at interviews. In every big company, maybe 1 in 50 programmers would say something like “Oh yes, I used Forth back in the 80’s – loved it!”. And an equal number of people would look at me like I had escaped from a lunatic asylum, retro-computing museum or Area 51 UFO containment area.  So I went into stealth mode – I kept “...one of the best-kept secrets in the computing world” to myself. It seems that software development departments encourage programmers to use their own tools, so I rarely had problems using Forth to develop “tools”, even when the ultimate goal was to develop C or C++ programs.

This gave me an overall speed advantage of maybe 2 or 3 times compared to my colleagues – this resulted in long, relaxed contracts in C, interspersed with much shorter contracts in Forth.

## ANS Forth and Windows

With Windows replacing MSDOS, I started using SwiftForth (Forth, Inc.’s product for Windows), MPE’s VFX Forth and Win32Forth. Forth is a chameleon language – it adapts to its environment, in this case Windows.

I do not hate Windows, in fact I think Microsoft have produced products of a consistently high standard, at least since NT4. But Windows programming has become steadily more and more difficult. Using Forth I still have an advantage over my colleagues – my “secret weapon” is still loaded and ready for action.

## colorForth

Around 2001 I downloaded Chuck Moore’s public domain colorForth from his website and copied on to a 3.5 inch floppy disk. It was not easy to get working – I had to add a new, compatible floppy disk ISA board to make it work.

I was impressed, again, wrote the article : colorForth and the Art of the Impossible and presented it at EuroForth 2001. I also had the great good fortune to spend about 45 minutes with Chuck, looking at his colorForth CAD system, OKAD II.

I love working in colorForth – I think it must be something genetic, certainly it appears not to be curable.

I presented another paper at EuroForth 2003 “The colorForth Magenta Variable”, and handed out floppy disks with the first distribution of my version of colorForth.

Time marches on, and one of my two PCs still with a floppy disk drive, died. I still have the other one, in the cellar, “just in case”. But it became obvious that colorForth needed to be updated to run from a USB stick.

A decade or so later, I presented a paper “Crypto colorForth” at EuroForth 2017 (the video is here), and demonstrated colorForth running from a USB stick. I believe that security and complexity are incompatible in computer software, and that colorForth can be the basis of a very secure operating system (without using files).

Today I am launching colorForth  cf2023 – there are a lot of changes, most of them for the better :

The font is now in ASCII order, even though Shannon-Fano encoded names are still used internally.

Byte addresses are used throughout, all magic numbers have been replaced by equ’s and the code is now better documented. It is far from perfect.

The user experience is more comfortable, most of the original apps now work again.

# The Future

- Make colorForth  load and run from a FAT32 file system on a USB stick. I already have a FAT32 single sector bootloader. This will allow data transfer between the colorForth system and the rest of the world.

- Add more drivers for Ethernet and WiFi hardware and mouse support.

- Add an assembler/disassembler and the ability of colorForth to rebuild itself without NASM.

- Add a secure data/metadata sharing and backup system (the You-thMe Drive)

# Philosophy

## Keep it simple

From Chuck Moore’s book Programming a Problem-oriented Language :

“The Basic Principle

Keep it Simple

As the number of capabilities you add to a program increases, the complexity of the program increases exponentially. The problem of maintaining compatibility among these capabililties, to say nothing of some sort of internal consistency in the program, can easily get out of hand.

You can avoid this if you apply the Basic Principle.

You may be acquainted with an operating system that ignored the Basic Principle. It is very hard to apply. All the pressures, internal and external, conspire to add features to your program.

After all, it only takes a half-dozen instructions; so why not? The only opposing pressure is the Basic Principle, and if you ignore it, there is no opposing pressure.”

## Chuck Moore from Fireside chat Nov 2020

https://www.youtube.com/watch?v=81bkIqPpe0g 34:33

Sean: Why do you think Forth has failed to go mainstream?

Chuck: I used to think about that a great deal.

I think that what happens in the world is largely a matter of luck, unpredictable, unrelated to quality or cost - it's just a fad, and some fads stick, and some fads don't.

I don't think that there is any need for Forth to become a mainstream language.

I think it is a very excellent niche language, and a personal language.

It can be used to advantage by people like us, without requiring that the whole world give us permission.

## The Maze Effect

My own experience of writing computer software is that it is like traversing a maze, rather than travelling a well mapped journey. It is often the case that you get to within one hedge-width of the goal, only to find that there is no way through – you must back-track, re-think and repeat . This equates to discarding already written software, which is often interpreted in a commercial software development environment as an expensive mistake, and so must be avoided.

Figure 2 The Hedge Maze at Longleat, UK

The picture above shows a hedge maze – a good metaphor for real-world software development. You can also see here bridges that can be used to climb over hedges – the one in the foreground could represent the change from block-based to file-based source code. Clearly it is an enormous advantage if you want to go in that direction, but it can also prevent or hinder access to other goals. It is not clear in the above picture precisely what the goal is, and this often goes for software development.

It is very difficult to write a requirements specification for a system that has not yet been discovered.

## Operating Systems and the Free Market Economy

In the developed world, commercial companies exist to make a profit – it is illegal to run a company that makes a loss – therefore there are two areas where “due diligence” must be applied :

- Maximizing profit

- Assessing risk

In the field of software development, maximising profit can be approximated to maximizing sales, if the development cost is (or can be made to be) a fixed amount.

Microsoft, for example achieved success initially by selling the MSDOS operating system. The “killer idea” was to sell a program to hardware manufacturers that provided a smooth interface to programs written by software developers. An entire eco-system evolved in which everybody gained :

- hardware manufacturers sold more because their product could run more programs

- software developers could sell more software because it could then run on more hardware

- Microsoft made a huge profit

When competitive Operating Systems ( OS’s) came along, it became necessary to “Lock in” users.

A number of techniques have been used :

- Operating Systems must be complex, otherwise everyone can write their own

- The interface must be incompatible with other OS’s

- Metadata must be held outside of the users’ control

It is the last item that I wish to explain in more detail, and then confront :

## Metadata and Files

What is a file, where does it exist and who owns it?

Take as an example a file in the MSDOS or Windows Command Shell environment. Typing CD shows you the current folder, typing DIR lists the files in that folder. This gives the illusion that your file exists in that folder – you can see it, send it, open it, edit it etc. The file content, and the directory structure where it can be found may well exist on your computer – you can delete it after all, and its gone (or at least it has been moved somewhere else).

But the metadata (information about the file) can only be accessed through the Operating System – that is what the Operating System is for, after all.

Take another example :

I create a Word document using Windows and the Word app. I send it by TCP/IP to a colleague who opens it on an Apple computer using the MAC OS and Pages. The most important metadata of the file is its size. The TCP/IP programming interface requires an address of the file data and its length. The Windows file system provides these parameters to the TCP/IP program, the required amount of data is sent to the Apple computer, and its TCP/IP program passes the data to the MAC OS.

Note that the metadata (file size) is not attached to the file at any point. Certainly the TCP/IP program requires and supplies a length parameter, but it is not attached to the file.

Of course both Operating Systems provide a convenient user interface, but the users are locked-in.

Another protocol layer is required – for example SMB or SAMBA -  and these layers can be made as complex and incompatible as is required to prevent another Operating System from competing easily.

There are many data exchange formats, for example XML and JSON – but these describe the data content of files and are not an alternative to files.

An important part of lock-in is never to attach important metadata such as the filesize to a standardised data structure. This is to prevent simple applications from using the “file” without being tied to a complex Operating System.

## The Library Trap

Forth and colorForth provide an interactive environment, in which maze-traversal becomes an order of magnitude faster (and more enjoyable) than non-interactive, batch processed languages such as C/C++/C# - IMHO, YMMV.

Python also provides an interactive environment, with libraries - for certain classes of problems it is an extremely effective tool – I have used it for example to create AES128-GMAC signed packets for testing software. Protocols such as AES and GMAC are complex, maybe necessarily so for signing packets securely, and this certainly saved many weeks of work.

There are at least two problems with libraries :

- As implemented in Python they come with a lot of baggage - an Operating System, installer programs - and of course Python interprets files - not exactly KISS.

- One of the fundamental principles of both Forth and colorForth is that the user and computer develop a relationship – the user teaches the computer how to do new things by defining new words, and the computer tells the user what it thinks of these new words, by returning values and error messages. Libraries do not allow this interaction. Somebody else may have experienced this when the library was written, but now it is just code.

## The Polarizing Effect

IMO Forth is a polarizing language because it can dramatically increase productivity in software development. This has several effects :

- Non-Forth programmers feel uncomfortable when a Forth programmer produces a program in a half or a quarter of the time that they would take. Nobody likes to be made to feel like an idiot.

- Forth programmers have a significant advantage, so they naturally love “...one of the best-kept secrets in the computing world.”

- Programming in Forth is fun – it is creative rather than mechanical work

- Forth is different and can have a steep un-learning curve

So people either love Forth or hate Forth – there is wide distance between the two extremes.

## An anecdote :

In the late 1990’s I was working on a three month contract, programming an LCD driver in C.

There were four special constraints  :

- The interface was I2C and the power consumption was critical, so only the minimum information must be sent, that is required to update the LCD

- There were two LCD driver chips, one for the left half of the display, the other for the right

- The whole display is mounted upside down – the graphics mode x,y = (0,0) coordinate is in the bottom right hand corner (instead of top left).

- The 8x8 character glyphs have to be rotated by 180 degrees.

The interface is similar to ANS Forth’s TYPE and AT-XY, but in C.

To solve this problem I first added a simple talker program, written in C that connected the devices serial port to its I2C bus and memory - something like PEEK and POKE, I2Cread and I2Cwrite.

Then I used SwiftForth to create the other side of the talker interface and words to display characters and the memory map in the device that was used to store the current LCD data. I also a word to rotate the font data and save it as a C file.

Having developed the program in Forth on a PC, testing all the while on the actual ARM target device, I translated the Forth program to C. Since I knew that I would be doing this I did not use any fancy Forth features such as CREATE …. DOES> or EVALUATE , and I kept the Forth word names compatible with C – no “%&*+-. ><” etc. characters.

The timescale for this was roughly three weeks of fun Forth development, about a week to convert to C. Then I cleaned up the code a bit, improved the documentation and so on, and after about 6 weeks I presented the fnished code to my colleague – an experienced and competent C programmer. He was surprised that I had completed the work so quickly, but was embarrassed that I had shown that his original time estimate was so wrong. No one likes to feel that they are inferior to anybody else, or that their career based on C is maybe not the most productive. My contract was not extended.

# colorForth Under the Hood

## BIOS disk access

Originally using direct hardware access to Floppy disk controller hardware, now converted to use 16 bit BIOS calls from 32 bit protected mode. This means you can use a USB stick, or USB Floppy drive.

## Video Display

The display setup uses VESA calls, 1024x768 16 bit colour mode, with some support for 800x600 16 bit colour. I did this was so that I could run colorForth on my Samsung NC10 netbook.

## Keyboard

The keyboard keys are scanned directly from I/O ports, using a PAUSE in the wait loop. Luckily the BIOS handles a USB keyboard and emulates legacy hardware ports.

## Keypad

colorForth does not use the keyboard in the usual way, but instead uses any type of 102 key keyboard to emulate a 27 key keypad, along the lines of a Dvorak keyboard.

# Appendix A Chuck Moore’s colorForth Primer

colorForth Primer
Chuck Moore

colorForth  is a uniquely simple way of programming computers. It is particularly suited to the multi-computer chips of GreenArrays. How simple it is:

## Words

colorForth uses words much as English does. (A word can be a subroutine, if that helps.) A word is a string of lower-case characters (from a set of 48) ending with space. The character @ is pronounced fetchand fetches a number from some address. Likewise, ! (store) stores a number. Some words:

- and or drop dup over push pop

- for next unext -if if then

- ; @ ! @+ !+ @b !b @p !p

- + . +* 2* 2/ - b! a!

- 12345 -1 144 0

If you type a word, the computer will perform some action. For example

- on

might turn on a light.

## Numbers

Words that look like numbers are placed on a push-down stack (like a stack of dishes). @ also puts numbers on the stack. There they serve as arguments for later words:

- 1000 ms

- 3 !

## Definitions

New words are defined in terms of old:

- toggle on 1000 ms off ;

The red word is defined by the following green words. When you type toggle, the light is turned on, the computer waits 1000 ms (milliseconds) then turns it off. Semicolon marks the end of this word (return from subroutine).

Other words:

- on 3 ! ;

- off 2 ! ;

Here a number is stored into a register to change an output.

## Loops

Computers are good at repetition. Here's one way to define a loop:

- ms for 1ms next ;

The word for expects an argument and puts it into a counter. The word next returns to for that many times. The word 1ms waits 1 millisecond.

## Conditions

Computers sometimes need to make decisions:

- abs -if - 1 + then ;

abs will return the absolute value of its argument. If it is negative, -if does a ones-complement and adds 1. If it is not negative (positive or 0) -if jumps to then and does nothing.

## Compiler

colorForth compiles source code into machine instructions, which can then be executed. It uses color to indicate the function of a word:

- Yellow - a word to be executed

- Red - a word being defined

- Green - a word to be compiled as part of a definition

- White (or black) - a comment to be ignored

Color aids understanding, avoids syntax and simplifies the compiler.

The compiler reads words from text stored in memory. A special editor manages this text.  colorForth code is exceptionally compact.

## Program

A program in colorForth is a collection of simple words that describe a task. Although definitions can be long and complicated, that is not wise. A larger number of simpler words is easier to read, write, debug and document.

The computer begs fallible programmers: Keep It Simple, Stupid (KISS). colorForth helps.

# Appendix B NASM Source Code

; cf2023.nasm 2023 Apr 04  MD5 "roomed-zebra"

; Fixing Magenta Variables

; Added "locate" - pressing the Enter key executes the red word or locates the green word at the cursor

; "chm" ( check MD5 ) in colorForth shows "ff" "la:rrr" "nos" "td"

; colorForth for 80x86 PC for NASM , with 1024x768 and 800x600 graphics options

; Adapted by Howerd Oakford from code by :

; Chuck Moore : inventor, MASM

; Mark Slicker : ported to GNU Assembler

; Peter Appelman : ported to NASM with qwerty keyboard

; Josh Grams : multitasker

; John Comeau : BIOS boot from ClusterFix

; Marco Nicola : 2drop and 2dup bug fix

; Miroslav Popov : keyboard instead of keypad typos

; and others... Thanks to all!!!

; Feedback welcome : howerd@inventio.co.uk www.inventio.co.uk

; %define NOT_BOCHS ; Bochs cannot handle resetting of the PIT chips, so we disable this to allow operation in Bochs

; CPU 386 ; Assemble instructions for the 386 instruction set

%define FORCE_800x600_VESA  0       ; true to force 800 x 600 x 16 bits for testing in  bochs

%define START_BLOCK_NUMBER      64  ; must be an even number. Note: if you change this you must shift the blocks in cf2022Ref.img accordingly!

%define SIZE_OF_FONT_IN_BLOCKS  12

%define OFFSET_OF_FONT          ( ( START_BLOCK_NUMBER - SIZE_OF_FONT_IN_BLOCKS ) * 0x400 )

%define LAST_BLOCK_NUMBER       511 ; must be an odd number

%define SECTORS_TO_LOAD ( ( LAST_BLOCK_NUMBER + 1 ) * 2 )   ; number of 512 byte sectors

%define BITS_PER_PIXEL  16  ; MUST BE 16 !!! display pixel sizes, colour depth = 16 bit ( 2 bytes )

; for the maximum supported screen : 1024 x 76go8 pixels :

%define MAX_SCREEN_WIDTH ( 1024 )   ; maximum screen width in pixels

%define MAX_SCREEN_HEIGHT ( 768 )   ; maximum screen height in pixels

%define BYTES_PER_PIXEL ( BITS_PER_PIXEL / 8 )

PIXEL_SHIFT equ 1           ; how many bits to shift to scale by BYTES_PER_PIXEL

; Memory Map

; start   length

; 0x100000 ....   RAM

; 0xC0000 0xFFFFF BIOS video ROM - its not RAM!

; 0xB8000 0x08000 BIOS video RAM

; 0x10000 0xA8000 cf2022.img file is copied here

; 0x0F000 0x01000 BIOS shadow RAM - its OK to use this if we do not call the video BIOS

; 0x0A000 0x05000 BIOS video RAM - do not use until we have changed video mode

; 0x07c00 0x00200 BPB Boot sector after loading by BIOS

; 0x07c0b <----- di points here, the BPB ( + offset )  and variables ( - offset ) are accessed via [di]

; 0x07b8c 0x00080 variables referenced via [di], followed by BPB variables referenced via [di]

; 0x07800         Stacks, size = 0x0200 each , growing downwards

; 0x02000 0x06800 SECTOR_BUFFER

; 0x00000 0x02000 BIOS RAM

%define SECTOR_BUFFER        0x00002000                     ; buffer for disk reads and writes

%define SECTOR_BUFFER_SIZE   0x4800                         ; 18 K bytes, 36 x 512 byte sectors

%define INTERRUPT_VECTORS    ( SECTOR_BUFFER - 0x0400 )      ; the IDT register points to these interrupt vectors

%define VESA_BUFFER          ( INTERRUPT_VECTORS - 0x0400 ) ; for the VESA mode information

%define DAP_BUFFER           ( VESA_BUFFER   - 0x0020 )     ; 0x1BE0 for the Int 0x13 Disk Address Packet (DAP)

%define DISK_INFO            ( DAP_BUFFER    - 0x0020 )     ; for the Int 0x13 AH=08h get info

%define IDT_AND_PIC_SETTINGS ( DISK_INFO     - 0x0040 )     ; bytes 0x00 - 0x05 SIDT value, 0x06 PIC1 IMR , 0x07 PIC2 IMR values saved at startup

%define V_REGS            ( IDT_AND_PIC_SETTINGS - 0x0020 ) ; test only - registers before and after thunk call

%define MD5_OUTPUT_BUFFER    ( V_REGS - 0x0020 )            ; the MD5 hash result

%define TRASH_BUFFER         ( (508 * 0x0400) + 0x10000 )   ; Block 508, saves words deleted while editing

%define PIC_BIOS_IDT_SETTINGS   ( IDT_AND_PIC_SETTINGS )   ; bytes 0x00 - 0x05 SIDT value, 0x06 PIC1 IMR , 0x07 PIC2 IMR values saved at startup

%define PIC_BIOS_IMR_SETTINGS   ( IDT_AND_PIC_SETTINGS + 6 ) ; bytes 0x00 - 0x05 SIDT value, 0x06 PIC1 IMR , 0x07 PIC2 IMR

%define PIC_NEW_IDT_SETTINGS    ( IDT_AND_PIC_SETTINGS + 0x10 )   ; bytes 0x00 - 0x05 SIDT value, 0x08 new PIC1 IMR , 0x09 new PIC2 IMR

%define PIC_NEW_IMR_SETTINGS    ( IDT_AND_PIC_SETTINGS + 0x16 )   ; bytes 0x00 - 0x05 SIDT value, 0x08 new PIC1 IMR , 0x09 new PIC2 IMR

%define IDT_AND_PIC_SETTINGS_PAD ( IDT_AND_PIC_SETTINGS + 0x20 )

%define vesa_BytesPerScanLine   ( VESA_BUFFER + 0x0E )  ; screen width ( number of horizontal pixels )

%define vesa_XResolution        ( VESA_BUFFER + 0x12 )  ; screen width ( number of horizontal pixels )

%define vesa_YResolution        ( VESA_BUFFER + 0x14 )  ; screen height ( number of vertical pixels )

%define vesa_BitsPerPixel       ( VESA_BUFFER + 0x19 )  ; bits per pixel

%define vesa_SavedMode          ( VESA_BUFFER + 0x1E )  ; "Reserved" - we save the VESA mode here

%define vesa_PhysBasePtr        ( VESA_BUFFER + 0x28 )  ; address of linear frame buffer

%define BOOTOFFSET      0x7C00

%assign RELOC_BIT 16                            ; the relocation address must be a power of 2

%assign RELOCATED 1 << RELOC_BIT                ; 0x10000

; *****************************************************************************

; Data and Return stack allocation, four pairs of data and return stacks

; Note : the return stack must be in the lowest 64K byte segment, for the BIOS calls to work

; *****************************************************************************

%define DATA_STACK_SIZE_0   $0400   ;

%define DATA_STACK_SIZE_1   $0500   ; must be > $400 for colorForth "Life" program to work

%define DATA_STACK_SIZE_2   $0100   ;

%define DATA_STACK_SIZE_3   $0100   ;

%define DATA_STACK_SIZE_GAP $0100   ; leave space under the last data stack to check for underflow

%define RETURN_STACK_SIZE   $0100   ;

; return stacks

%define RETURN_STACK_0      ( $7800 )         ; top of stack memory area

%define RETURN_STACK_1      ( RETURN_STACK_0 - RETURN_STACK_SIZE )

%define RETURN_STACK_2      ( RETURN_STACK_1 - RETURN_STACK_SIZE )

%define RETURN_STACK_3      ( RETURN_STACK_2 - RETURN_STACK_SIZE )

; data stacks

%define DATA_STACK_0        ( RETURN_STACK_3 - RETURN_STACK_SIZE )

%define DATA_STACK_1        ( DATA_STACK_0   - DATA_STACK_SIZE_0 ) ; BIG data stack for the show task

%define DATA_STACK_2        ( DATA_STACK_1   - DATA_STACK_SIZE_1 )

%define DATA_STACK_3        ( DATA_STACK_2   - DATA_STACK_SIZE_2 )

%define STACK_MEMORY_START  ( DATA_STACK_3   - DATA_STACK_SIZE_3 - DATA_STACK_SIZE_GAP )

; four pairs of stacks, one for each task

%define TOTAL_STACK_SIZE    ( RETURN_STACK_0 - STACK_MEMORY_START )

%define STACK_ANALYSIS_BUFFER    ( STACK_MEMORY_START - 0x200 )

; *****************************************************************************

; *****************************************************************************

%define _TOS_  eax

%define _TOS_x_  ax

%define _TOS_l_  al

%define _SCRATCH_  ebx

%define _SCRATCH_x_  bx

%define _SCRATCH_l_  bl

%define _MOV_TOS_LIT_  (0xB8)   ; the opcode for mov eax, 32_bit_literal  (in next 32 bit cell)

%macro _DUP_  0                 ; Top Of Stack is in the  _TOS_  register

lea esi, [ esi - 4 ]        ; pre-decrement the stack pointer, does not set the flags

; sub esi, byte 0x04        ; pre-decrement the stack pointer, also sets the flags

mov [ esi ], _TOS_          ; copy the Top Of Stack ( TOS ) register to Second On Stack ( on the real stack )

%endmacro

%macro _SWAP_  0

xchg _TOS_, [ esi ]

%endmacro

%macro _OVER_  0

sub esi, byte 0x04          ; pre-decrement the stack pointer

mov [ esi ], _TOS_          ; copy the Top Of Stack ( TOS ) register to Second On Stack ( on the real stack )

mov _TOS_, [ esi + 4 ]

%endmacro

%macro _DROP_ 0

lodsd  ; loads a 32 bit dword from [ds:esi] into  eax  then increments  esi  by 4

%endmacro

; Note : stosd ; stores  eax  into the location pointed to by  edi  then increments  edi  by 4

; Note also : eax is used as  _TOS_  ( Top Of Stack )

; Define the location of the wordlists in RAM

%define START_OF_RAM    0x00468000

%define WORDLIST_SIZE      0x2000

%define ForthNames      START_OF_RAM                ; copied to RAM here from ROM ( i.e. boot program ) version

%define ForthLocates    ( ForthNames     + WORDLIST_SIZE ) ; copied to RAM here from ROM ( i.e. boot program ) version

%define ForthJumpTable  ( ForthLocates   + WORDLIST_SIZE ) ; copied to RAM here from ROM ( i.e. boot program ) version

%define MacroNames      ( ForthJumpTable + WORDLIST_SIZE ) ; copied to RAM here from ROM ( i.e. boot program ) version

%define MacroJumpTable  ( MacroNames     + WORDLIST_SIZE ) ; copied to RAM here from ROM ( i.e. boot program ) version

%define MacroLocates    ( MacroJumpTable + WORDLIST_SIZE ) ; copied to RAM here from ROM ( i.e. boot program ) version

; some more room...

%define H0              0x00480000 ;  ( MacroLocates + WORDLIST_SIZE )   ; initial value of the dictionary pointer

%define SECTOR  512     ; bytes per floppy sector

%define HEADS   2       ; heads on 1.44M floppy drive

%define SECTORS 18      ; floppy sectors per track

%define CYLINDER (SECTOR * SECTORS * HEADS)

%define CELL 4          ; bytes per cell

%define DEBUGGER 0xe1   ; port to hardware debugger?

; int 0x13 Disk Address Packet (DAP) pointed to by  si  :

%define o_Int13_DAP_size           ( 0x00 ) ; 2  0x0010

%define o_Int13_DAP_num_sectors    ( 0x02 ) ; 2  0x0001

%define o_Int13_DAP_address        ( 0x04 ) ; 2  0x2000

%define o_Int13_DAP_segment        ( 0x06 ) ; 2  0x0000

%define o_Int13_DAP_LBA_64_lo      ( 0x08 ) ; 4  0x00000028

%define o_Int13_DAP_LBA_64_hi      ( 0x0C ) ; 4  0x00000000

; extended DAP values

%define o_Int13_DAP_readwrite      ( 0x10 ) ; 2  0x0000

%define o_Int13_DAP_saved_DX       ( 0x12 ) ; 2  0x0000

%define o_Int13_DAP_returned_AX    ( 0x14 ) ; 2  0xHH00 see AH Return Code below

%define o_Int13_DAP_returned_carry_flag ( 0x16 ) ; 2  0x0000

%define o_Int13_DAP_saved_CHS_CX   ( 0x18 ) ; 2  0x0000

%define o_Int13_DAP_saved_CHS_DX   ( 0x1A ) ; 2  0x0000

%macro LOAD_RELATIVE_ADDRESS  1

mov _TOS_, ( ( ( %1 - $$ ) + RELOCATED ) )

%endmacro

; emit the given following character

%macro EMIT_IMM 1

;    push esi

_DUP_

mov _TOS_, %1

call emit_

;    pop esi

%endmacro

; *****************************************************************************

; Registers used

; *****************************************************************************

; _TOS_ is the top stack item ( eax --> ebx )

; esp the call ... ret  return stack pointer

; edi  dictionary pointer ( H --> : HERE ( -- a )   H @ ; )

; esi is the stack pointer, also needed by lods and movs

; e.g. lodsd  loads a 32 bit dword from [ds:esi] into _TOS_, increments  esi  by 4

; ebx  scratch register

; ecx  counter and scratch register

; edx  run-time pointer (?), "a register" used by  a!  , otherwise scratch register

; ebp  variable pointer register

; "ds" = selector 0x10 ==> 0x0000:0000

; "es" = selector 0x10 ==> 0x0000:0000

; "ss" = selector 0x10 ==> 0x0000:0000

; colours RGB in 16 bits

colour_background   equ 0x0000

colour_yellow       equ 0xFFE0

colour_black        equ 0x0000

colour_red          equ 0xF800

colour_green        equ 0x0600

colour_cyan         equ 0x07FF

colour_white        equ 0xFFFF

colour_light_blue   equ 0x841F

colour_silver       equ 0xC618

colour_magenta      equ 0xF81F

colour_magentaData  equ 0xD010

colour_blue         equ 0x001F

colour_orange       equ 0xE200

colour_dark_yellow  equ 0xFFE0

colour_dark_green   equ 0x07C0

colour_PacMan       equ 0xE200

colour_blockNumber  equ 0xE200

[BITS 16]                           ; Real Mode code (16 bit)

org RELOCATED

start:

codeStart:

jmp  main_16bit    ; 0x03 bytes |  EB 58 90  00 Jump to boot code

times 3 - ($ - $$) nop        ; fill with 1 or 0 no-ops to address 3

; BIOS boot parameter table = 0x25 bytes

db 'cf2022 0'      ; 03 Eight byte OEM name

dw 0x0200          ; 11 Number of Bytes Per Sector

db 0x08            ; 13 Number of Sectors Per Cluster

dw 0x05E0          ; 14 Number of Reserved Sectors until the FAT

db 0x02            ; 16 Number of Copies of FAT : always = 2

dw 0x0000          ; 17 Maximum number of Root Directory Entries

dw 0x0000          ; 19 Not used for FAT32

db 0xF8            ; 21 Media type F0 = 1.44M 3.5 inch floppy disk, F8 = hard disk changes 2022 Mar14

dw 0x0000          ; 22 Sectors Per FAT for FAT12 and FAT16 - not used for FAT32

dw 0x003F          ; 24 Sectors per Track

dw 0x00FF          ; 26 Number of heads

dd 0x00000038      ; 28 Hidden sectors preceding the partition that contains this FAT volume

dd 0x007477C8      ; 32

dd 0x00001D10      ; 36 Sectors Per FAT for FAT32

dw 0x0000          ; 40

dw 0x0000          ; 42

dd 0x00000002      ; 44 Start of all directories, including root.

dw 0x0001          ; 48

dw 0x0006          ; 50 Offset in sectors from this sector to the backup BPB sector

;    times 12 db 0      ; 0x0C bytes |  00 00 00 00 00 00 00 00 00 00 00 00  52

;    db 0x00            ; 64

;    db 0x00            ; 65

;    db 0x29            ; 66 Extended Boot Signature

;    dd 0x44444444      ; 67 serial number

;    db 'colorForth '   ; 71 Eleven byte Volume Label

;    db 'cFblocks'      ; 82 Eight byte File System name

; ******************************************************************************

; ******************************************************************************

align 8, nop    ; has to be aligned to 8 for GDT

; Note : we are NOT using null descriptor as GDT descriptor, see: http://wiki.osdev.org/GDT_Tutorial

; "The null descriptor which is never referenced by the processor. Certain emulators, like Bochs, will complain about limit exceptions if you do not have one present.

; Some use this descriptor to store a pointer to the GDT itself (to use with the LGDT instruction).

; The null descriptor is 8 bytes wide and the pointer is 6 bytes wide so it might just be the perfect place for this."

gdt:                                ; the GDT descriptor

dw gdt_end - gdt - 1            ; GDT limit

dw gdt0 + BOOTOFFSET            ; pointer to start of table, low 16 bits

dw 0 , 0                        ; the high bits of the longword pointer to gdt

gdt0:                               ; null descriptor

dw 0    ; 0,1 limit 15:0

dw 0    ; 2,3 base  15:0

db 0    ; 4   base  23:16

db 0    ; 5   type

db 0    ; 6   limit 19:16, flags

db 0    ; 7   base  31:24

code32p_SELECTOR_0x08 equ $ - gdt0

; bytes   1 0     3 2     5 4     7 6

dw 0xFFFF, 0x0000, 0x9A00, 0x00CF   ; 32-bit protected-mode code, limit 0xFFFFF

data32p_SELECTOR_0x10 equ $ - gdt0

dw 0xFFFF, 0x0000, 0x9200, 0x00CF   ; 32-bit protected-mode data, limit 0xFFFFF

code16r_SELECTOR_0x18 equ $ - gdt0

dw 0xFFFF, 0x0000, 0x9A00, 0x0000   ; 16-bit real-mode code, limit 0xFFFFF

data16r_SELECTOR_0x20 equ $ - gdt0

dw 0xFFFF, 0x0000, 0x9200, 0x0000   ; 16-bit real-mode data, limit 0xFFFFF

gdt_end:

; ******************************************************************************

; ******************************************************************************

; align to 4 so we can access variables from high-level Forth

align 4, nop

data_area:   ; data area begins here

bootsector:                         ; LBA of boot sector

dd 0

; save disk information, cylinder, sector, head and drive from BIOS call

driveinfo_Drive_DX:                 ; use low byte to store boot Drive into from BIOS DL

dw 0

driveinfo_CX:         ; [7:6] [15:8][7] logical last index of cylinders = number_of - 1 (because index starts with 0)

; [5:0][7] logical last index of sectors per track = number_of (because index starts with 1)

dw 0

; cylinders, sectors, heads of boot drive

; low word: high byte is head

; high word: cylinder and sector: C76543210 C98S543210

driveinfo_Cylinder:

db 0

driveinfo_Head:

db 0

driveinfo_SectorsPertrack:

dw 0

align 4, nop

destination:

dd RELOCATED

dispPtr:

dd 0x00000140

v_bytesPerLine:

dd 0x00

v_scanCode:

dd 0x00

align 4

; ******************************************************************************

; the main program called from initial 16 bit mode

; ******************************************************************************

main_16bit:

cli                         ; clear interrupts

; turns out we don't need interrupts at all, even when using BIOS routines

; but we need to turn them off after disk calls because BIOS leaves them on

push si                     ; need to transfer SI to unused register BX later

; note: cannot touch DX or BP registers until we've checked for partition boot

; (SI could be used as well as BP but we use SI for relocation)

;see mbrboot.nasm

; Note : relocate the bootblock before we do anything else

pop bx                      ; we cannot use the current stack after changing SS or SP

; ... because mbrboot.nasm places stack at 0x7c00, in SECTOR_BUFFER

; and we cannot use BP because its default segment is SS

xor ax, ax

mov ds, ax

mov es, ax

mov si, BOOTOFFSET

mov di, SECTOR_BUFFER

mov sp, di

mov cx, 0x100

rep movsw                   ; note that this instruction doesn't change AX , it moves DS:SI to ES:DI and increments SI and DI

mov ss, ax                  ; stack segment also zero

mov ah, 0xb8                ; video RAM

mov gs, ax                  ; store in unused segment register

lgdt [gdt - $$ + BOOTOFFSET]

call SetupUnrealMode     ; gs and ss must be initialized before going to Unreal Mode

; *****************************************************************************

; Enable the A20 address line, otherwise all odd 1 MByte pages are disabled

; Using the "PS/2 Controller" or 8042 "Keyboard controller"

; *****************************************************************************

; from  http://wiki.osdev.org/%228042%22_PS/2_Controller#Step_1:_Initialise_USB_Controllers

; Write a command to the on-board 8042 "Keyboard controller" port 0x64 :

; 0x20     Read "byte 0" from internal RAM     Controller Configuration Byte

; 0x21 to 0x3F    Read "byte N" from internal RAM (where 'N' is the command byte & 0x1F)

; 0x60    Write next byte to "byte 0" of internal RAM (Controller Configuration Byte)

; 0x61 to 0x7F    Write next byte to "byte N" of internal RAM (where 'N' is the command byte & 0x1F)

; 0xA7    Disable second PS/2 port

; 0xA8    Enable second PS/2 port

; 0xA9    Test second PS/2 port

;     0x00 test passed

;     0x01 clock line stuck low

;     0x02 clock line stuck high

;     0x03 data line stuck low

;     0x04 data line stuck high

; 0xAA    Test PS/2 Controller

;     0x55 test passed

;     0xFC test failed

; 0xAB    Test first PS/2 port

;     0x00 test passed

;     0x01 clock line stuck low

;     0x02 clock line stuck high

;     0x03 data line stuck low

;     0x04 data line stuck high

; 0xAC    Diagnostic dump (real all bytes of internal RAM)    Unknown

; 0xAD    Disable first PS/2 port     None

; 0xAE    Enable first PS/2 port  None

; 0xC0    Read controller input port  Unknown (none of these bits have a standard/defined purpose)

; 0xC1    Copy bits 0 to 3 of input port to status bits 4 to 7    None

; 0xC2    Copy bits 4 to 7 of input port to status bits 4 to 7    None

; 0xD0    Read Controller Output Port     Controller Output Port (see below)

; 0xD1    Write next byte to Keyboard Controller Output Port Note: Check if output buffer is empty first

; 0xD2    Write next byte to first PS/2 port output buffer

; 0xD3    Write next byte to second PS/2 port output buffer

; 0xD4    Write next byte to second PS/2 port input buffer

; 0xF0 to 0xFF  Pulse output line low for 6 ms.

;     Bits 0 to 3 are used as a mask (0 = pulse line, 1 = do not pulse line) and correspond to 4 different output lines.

;     Bit 0 is the "reset" line, active low.

mov al, 0xD1    ; 0xD1 = Write next byte to Keyboard Controller Output Port

out 0x64, al    ; On-board controller Command Write

.back:

in al, 0x64

and al, 0x02

jnz .back

mov al, 0x4B

out 0x60, al

; *****************************************************************************

; Get disk drive parameters from the BIOS

; *****************************************************************************

mov di, (data_area - $$ + BOOTOFFSET)   ; setup the data index pointer

xor eax, eax

bts eax, 16                     ; in case NOT booted from partition: sector 1, head 0, cylinder 0

or dh, dh                       ; booted from partition?

jz .forward3

mov eax, [ bx + 8 ]             ; SI (now BX) contains pointer to partition record

mov [ byte di + (bootsector - data_area) ], eax     ; offset 8 was LBA of first absolute sector

mov eax, [bx]                   ; CHS of first sector in partition

.forward3:

mov al, dl                      ; bootdrive into AL

mov [ word di + ( driveinfo_Drive_DX - data_area) ], eax    ; save the Drive info from BIOS

mov ah, 8                       ; get drive parameters

push es                         ; this operation messes with ES

push di                         ; and DI

mov di, DISK_INFO               ; point di at the table returned by this software interrupt

int 0x13

jc $                            ; stop here on error

call ReSetupUnrealMode

pop di

pop es

; ******************************************************************************

; load the bootdisk into both low and high RAM

; ******************************************************************************

mov [ byte di + ( driveinfo_Cylinder - data_area) ], dx             ; heads in high byte

and cl, 0x3F                    ; we don't care about two high bits of cylinder count

mov [ byte di + ( driveinfo_SectorsPertrack - data_area) ], cx     ; cylinders and sectors/track

mov dx, [ byte di + ( driveinfo_Drive_DX - data_area) ]         ; restore dl Drive value from BIOS, dh = 0

;    mov dl, 0x00    ; try this 2022 Mar 14

mov cx, [ di + ( driveinfo_CX - data_area) ]               ; restore cl value, ch = 0

mov si, SECTORS_TO_LOAD

mov bx, SECTOR_BUFFER            ; relocate the sector we are running from

call relocate

mov bx, BOOTOFFSET              ; we will fix this below by adding 0x200

; remember the sector is 1-based, head and cylinder both 0-based

.nextsector:

inc cl

dec si

jz setVideoMode    ; success, so setup the video now...

.bootload:

mov ax, 0x201                   ; read 1 sector

add bh, 0x02                    ; into next available slot in RAM

jnz .forward

sub bh, 0x02                    ; at 0x10000 we go back to 0xfe00

.forward:

int 0x13

call ReSetupUnrealMode

jc $                            ; stop here on error

call relocate

mov al, cl

and al, 0x3F                    ; low 6 bits

cmp al, [ byte di + ( driveinfo_SectorsPertrack - data_area) ]

jnz .nextsector

inc dh                          ; next head

cmp dh, [ byte di + ( driveinfo_Head - data_area) ]

jna .forward2                   ; not JNZ, the head index is 1 less than head count

xor dh, dh

inc ch                          ; next cylinder

jnz .forward2

add cl, 0x40                    ; bit 8 of cylinder count

.forward2:

and cl, 0xC0                    ; clear sector count, low 6 bits of cl

jmp short .nextsector

; ******************************************************************************

; ******************************************************************************

; Start here after loading the program

; ******************************************************************************

; ******************************************************************************

; From : VESA BIOS EXTENSION (VBE) Core Functions Standard Version: 3.0 Date: September 16, 1998

; Mandatory information for all VBE revisions

; dw ModeAttributes         ; 0x00 mode attributes

; db WinAAttributes         ; 0x02 window A attributes

; db WinBAttributes         ; 0x03 window B attributes

; dw WinGranularity         ; 0x04 window granularity

; dw WinSize                ; 0x06 window size

; dw WinASegment            ; 0x08 window A start segment

; dw WinBSegment            ; 0x0A window B start segment

; dd WinFuncPtr             ; 0x0C real mode pointer to window function

; dw BytesPerScanLine       ; 0x10 bytes per scan line               <--------------

; Mandatory information for VBE 1.2 and above

; dw XResolution            ; 0x12 horizontal resolution in pixels   <-------------- scrnw

; dw YResolution            ; 0x14 vertical resolution in pixels     <-------------- scrnh

; db XCharSize              ; 0x16 character cell width in pixels

; db YCharSize              ; 0x17 character cell height in pixels

; db NumberOfPlanes         ; 0x18 number of memory planes

; db BitsPerPixel           ; 0x19 bits per pixel                    <-------------- bpp

; db NumberOfBanks          ; 0x1A number of banks

; db MemoryModel            ; 0x1B memory model type

; db BankSize               ; 0x1C bank size in KB

; db NumberOfImagePages     ; 0x1D number of images

; db Reserved               ; 0x1E reserved for page function        <-------------- mode (we copy it here)

; Direct Color fields (required for direct/6 and YUV/7 memory models)

; db RedMaskSize            ; 0x1F size of direct color red mask in bits

; db RedFieldPosition       ; 0x20 bit position of lsb of red mask

; db GreenMaskSize          ; 0x21 size of direct color green mask in bits

; db GreenFieldPosition     ; 0x22 bit position of lsb of green mask

; db BlueMaskSize           ; 0x23 size of direct color blue mask in bits

; db BlueFieldPosition      ; 0x24 bit position of lsb of blue mask

; db RsvdMaskSize           ; 0x25 size of direct color reserved mask in bits

; db RsvdFieldPosition      ; 0x26 bit position of lsb of reserved mask

; db DirectColorModeInfo    ; 0x27 direct color mode attributes

; Mandatory information for VBE 2.0 and above

; dd PhysBasePtr            ; 0x28 physical address for flat memory frame buffer  <-------------- vframe

; dd Reserved               ; 0x2C Reserved - always set to 0

; dw Reserved               ; 0x30 Reserved - always set to 0

; Mandatory information for VBE 3.0 and above

; dw LinBytesPerScanLine    ; 0x32 bytes per scan line for linear modes

; db BnkNumberOfImagePages  ; 0x34 number of images for banked modes

; db LinNumberOfImagePages  ; 0x35 number of images for linear modes

; db LinRedMaskSize         ; 0x36 size of direct color red mask (linear modes)

; db LinRedFieldPosition    ; 0x37 bit position of lsb of red mask (linear modes)

; db LinGreenMaskSize       ; 0x38 size of direct color green mask  (linear modes)

; db LinGreenFieldPosition  ; 0x39 bit position of lsb of green mask (linear modes)

; db LinBlueMaskSize        ; 0x3A size of direct color blue mask  (linear modes)

; db LinBlueFieldPosition   ; 0x3B bit position of lsb of blue mask (linear modes)

; db LinRsvdMaskSize        ; 0x3C size of direct color reserved mask (linear modes)

; db LinRsvdFieldPosition   ; 0x3D bit position of lsb of reserved mask (linear modes)

; dd MaxPixelClock          ; 0x3E maximum pixel clock (in Hz) for graphics mode

; times 189 db 0            ; 0x42 remainder of ModeInfoBlock

; End                       ; 0xFF

scanVESA:   ; ( w+h+b -- )  in ax

mov bx, ax

push di                         ; save di

mov cx, ( 0x4117 - 1 )          ; start scanning from the expected VESA mode 0x4117 ( the -1 is because of the  inc cx  below )

.back:

inc cl                          ; increment just the bottom byte, we test 0x41xx

cmp cl, 0x16                    ; scanned from 0x4117 to 0x4116, not found, so show error

jz .failure

mov di, VESA_BUFFER             ; buffer for the VESA mode information block

mov ax, 0x4F01                  ; INT 0x10, AX=0x4F01, CX=mode Get Mode Info

int 0x10

cmp al, 0x4F                    ; success code = 0x4F

jne .back                       ; try the next VESA mode

mov ax, [di + 0x12]             ; width

add ax, [di + 0x14]             ; height

add al, [di + 0x19]             ; bits per pixel

;    adc ah, 0                       ; should not be necessary for the expected result, 0x400+0x300+0x10

cmp ax, bx                      ; width + height + bits per pixel

je .success

jne .back                       ; try the next VESA mode

.failure:                           ; VESA mode not found, so continue

pop di                          ; restore di

mov ax, 0                       ; return flag false

add ax, 0                       ; set the zero flag

ret

.success:

mov [ di + ( vesa_SavedMode - VESA_BUFFER ) ], cx ; save the VESA mode in the VESA_BUFFER at offset 0x1E "Reserved"

mov ax, 1                       ; return flag true

add ax, 0                       ; set the zero flag

pop di                          ; restore di

ret

setVESA:    ; we found a valid VESA mode

push ds                         ; clear all flags including Interrupt using DS, known to be zero

popf                            ; this is necessary to clear T flag also, end register display

call greet      ; show greeting message

mov bx, cx

mov ax, 0x4F02  ; INT 0x10, AX=0x4F02, BX=mode, ES:DI=CRTCInfoBlock Set Video Mode

int 0x10

jmp main_32bit

setVideoMode:

%if ( FORCE_800x600_VESA == 0 )     ; test the 800x600 mode in bochs, which supports 1024x768

mov ax, ( 1024 + 768 + BITS_PER_PIXEL ) ; try the highest resolution first

call scanVESA                   ; if VESA mode is found, jump to setVESA

jnz setVESA                     ; success - we found the requested VESA mode

%endif

mov ax, ( 800 + 600 + BITS_PER_PIXEL ) ; then try a lower resolution

call scanVESA                   ; if VESA mode is found, jump to setVESA

jnz setVESA                     ; success - we found the requested VESA mode

;    mov ax, 640 + 480 + BITS_PER_PIXEL  ; then try an even lower resolution

;    call scanVESA                   ; if VESA mode is found, jump to setVESA

;    jnz setVESA                     ; success - we found the requested VESA mode

jmp showVESAerror               ; we have tried all VESA modes without success, so report an error

; ******************************************************************************

; ******************************************************************************

relocate:                   ; copy 512 bytes from  [bx]  to FS:[destination]

pusha

mov cx, 0x200 / 2

mov si, bx

mov ebx, [ byte di + ( destination - data_area) ]

.back:

lodsw               ; load the 16 bit value pointed to by SI into  ax

mov [fs:ebx], ax    ; Note : the  fs:  uses the 32 bit FS value setup in Unreal Mode to move the data outside of the 1 Mbyte Real Mode address range

add ebx, byte +2

loop .back

mov [ byte di + ( destination - data_area) ], ebx

popa

ret

; not used because it is very slow :

; now set up for trap displaying registers on screen during bootup

;    push cs

;    push showstate - $$ + BOOTOFFSET

;    pop dword [word +4]

; ******************************************************************************

; ******************************************************************************

;1. MasterBoot Record - MBR   at Sector     0 (decimal 0)        MBR

; Partition at offset 1BE

;   BootSignature                0

;   Start Head|Sector|Cylinder   1   1     0

;   Partition Type               B  DOS 7.1+

;   End   Head|Sector|Cylinder  FE  3F   3E5

;   BPBsectorNumber                   00  \ was 3F

;   Size of partition (decimal)  16035777 sectors,  8210317824 bytes,  8017889 Ki bytes,  7830 Mi bytes,  8 Gi bytes

; Partition at offset 1CE

;   BootSignature                0

;   Start Head|Sector|Cylinder   0   0     0

;   Partition Type               0  Empty partition

;   End   Head|Sector|Cylinder   0   0     0

;   BPBsectorNumber                    0

;   Size of partition (decimal)         0 sectors,           0 bytes,  0 Ki bytes,  0 Mi bytes,

; pretend to be a Master Boot Record so that the BIOS will load us

times ( 0x000001BE - ( $ - $$ ) ) db 0x77

db 0x80, 0x01, 0x01, 0x00, 0x0B, 0xFE, 0xFF, 0xE5, 0x00, 0x00, 0x00, 0x00, 0xC1, 0xAF, 0xF4, 0x00 ; 0x1BE DOS partition 0  working on PC

db 00, 00, 00, 00, 00, 00, 00, 00   ; 0x1CE first 8 bytes of empty partition 1

SetupUnrealMode:

; set the FS segment in "unreal" mode, must be done before the Trap Flag is set in EFLAGS register

mov eax, cr0

or al, 1    ; set the "protected mode enable" bit => "unreal mode"

mov cr0, eax

push word data32p_SELECTOR_0x10 ; set the FS segment

pop fs

dec al      ; clear the "protected mode enable" bit

mov cr0, eax

push ds                         ; now set FS to 0

pop fs

ReSetupUnrealMode:

push cs                         ; for iret

pushf                           ; for iret

pusha

mov bp, sp

mov ax, [bp + 16]               ; get flags

;    or ah, 0x01                    ; set Trap Flag, bit 8 in the EFLAGS register ; debug only - very slow!

and ah, ~0x02                   ; reset interrupt flag

xchg ax, [ bp + 20 ]            ; swap flags with return address

mov [ bp + 16 ], ax             ; return address at top of stack after popa

popa

iret

; ******************************************************************************

; ******************************************************************************

times 512 - 2 - ($ - $$) nop        ; fill with no-ops to 55AA at end of boot sector

db 0x55 , 0xAA  ; boot sector terminating bytes

; ******************************************************************************

; End of Boot Sector

; ******************************************************************************

; ******************************************************************************

; Show the user a null terminated string - writes directly into video RAM

; ******************************************************************************

displayString:

; restore the pointer to screen memory into di

mov di, (data_area - $$ + BOOTOFFSET)

mov ax, [ di + ( dispPtr - data_area) ]

mov di, ax

push es             ; save es

mov ax, 0xb800      ; video RAM segment

mov es, ax

backhere2:

lodsb               ; loads a byte from [ds:si] into al, then increments  si

cmp al, 0

jz forward1         ; If al = 0 then leave the loop

mov ah, 0x0D        ; text colour, magenta on black background

stosw               ; stores  ax  into  [es:di]  then increments  di

jmp backhere2

forward1:

; save the pointer to screen memory from di

mov ax, di

mov di, (data_area - $$ + BOOTOFFSET)

mov [ di + ( dispPtr - data_area) ], ax

pop es          ; restore es

ret

; display a string then Wait for a key press

displayStringW:

pusha

call displayString

xor  ax, ax     ; wait for and get a key press ( AX = 0 )

int  0x16       ; BIOS interrupt Read a Key From the Keyboard

popa

ret

; msg_greeting2:

;     db ' Press any key : ' , 0x00

msg_VESAerror:

db 'No valid VESA mode found! ' , 0x02, 0x00

;    db ' No VESA mode ' , 0x02, 0x00

[BITS 16]                           ; Real Mode code (16 bit)

showVESAerror:

call greet

push si

mov word [ di + ( dispPtr - data_area) ] , 0x000001E0    ; line 3 0x50 x 2 x 3 = 0x1E0

mov si, ( msg_VESAerror - $$ + BOOTOFFSET )  ; string to display

call displayStringW

pop si

ret

greet:     ; jump here to show 16 bit version text

push si

mov word [ di + ( dispPtr - data_area) ] , 0x00000140    ; line 2 0x50 x 2 x 2 = 0x140

mov si, ( version - $$ + BOOTOFFSET )  ; string to display

call displayString

;    mov si, ( msg_greeting2 - $$ + BOOTOFFSET )  ; string to display

;    call displayStringW

pop si

ret

; ******************************************************************************

; the main program in 32 bit ( protected ) mode

; ******************************************************************************

main_32bit:

call setProtectedModeAPI        ; called from 16 bit code, returns in 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit) - assemble for 32 bit mode from now on

mov esp, RETURN_STACK_0         ; setup the return stack pointer

mov esi, ( DATA_STACK_0 + 4 )   ; setup our data stack pointer

call save_BIOS_idt_and_pic      ; to be restored later, when making BIOS calls

call init_default_PIC_IMRs      ; set the default values and copy the BIOS Interrupt Vectors to our new table

_DUP_

mov _TOS_, INTERRUPT_VECTORS

call lidt_                      ; Load the new Interrupt Descriptor Table

jmp dword warm

; *****************************************************************************

; calculate Cylinder, Head and Sector from zero-based sector number

; see http://teaching.idallen.com/dat2343/00f/calculating_cylinder.htm

; Note : uses pushad to copy registers onto the ESP stack, stores the

; calculated values onto the stack at the correct offsets, then restores the

; stack back to the registers.

; *****************************************************************************

sector_chs:  ; ( sector -- eax ) calculate CHS from a sector number in eax,

; returns with DX = HHDD, CX = CCSS where HH=head, DD=drive, CC=cylinder, SS=sector

; Note that the input sector number is zero based, and that the high 16 bits of EAX must be 0

pushad  ; Pushes all general purpose registers onto the stack in the following order:

; EAX, ECX, EDX, EBX, ESP, EBP, ESI, EDI. The value of ESP is the value before the actual push of ESP

;  7    6    5    4    3    2    1    0   offset in cells from ESP

mov ebp, esp    ; copy the original ESP stack pointer to EBP so we can access items on the stack easily

; save the register values in the DAP buffer for use later, via ESI

mov esi, DAP_BUFFER

add eax, [ bootsector - $$ + BOOTOFFSET]

push eax                            ; save it while we calculate heads*sectors-per-track

mov al, [ driveinfo_Head - $$ + BOOTOFFSET]      ; index of highest-numbered head

inc al                              ; 1-base the number to make count of heads

mul byte [ driveinfo_SectorsPertrack - $$ + BOOTOFFSET]     ; sectors per track

mov ebx, eax

pop eax

xor edx, edx                        ; clear high 32 bits

div ebx                             ; leaves cylinder number in eax, remainder in edx

mov ecx, eax                        ; store cylinder number in another register

mov eax, edx                        ; get remainder into AX

mov bl, [ driveinfo_SectorsPertrack - $$ + BOOTOFFSET]      ; number of sectors per track

div bl                              ; head number into AX, remainder into DX

mov bl, al                          ; result must be one byte, so store it in BL

rol ecx, 8                          ; high 2 bits of cylinder number into high 2 bits of CL

shl cl, 6                           ; makes room for sector number

or cl, ah                           ; merge cylinder number with sector number

inc cl                              ; one-base sector number

mov [ ebp + ( 6 * 4 ) ], ecx        ; store the result in ECX position on esp stack

mov word [ esi + o_Int13_DAP_saved_CHS_CX ], cx  ; also save the calculated CX value

mov cx, [ driveinfo_Drive_DX - $$ + BOOTOFFSET]    ; drive number in low 8 bits

mov ch, bl                          ; place head number in high bits

;    mov cl, 0x80

mov [ ebp + ( 5 * 4 ) ], ecx    ; store the result in EDX position on esp stack

mov word [ esi + o_Int13_DAP_saved_CHS_DX ], cx  ; also save the calculated DX value

popad                           ; restore registers from esp stack

ret

; *****************************************************************************

; enter Protected Mode (32 bit) and Real Mode (16 bit)

; from http://ringzero.free.fr/os/protected%20mode/Pm/PM1.ASM

; *****************************************************************************

[BITS 16]   ; Real Mode code (16 bit)

enterProtectedMode:                 ; must come from a 'call' , can not be inlined

pop ax

push code32p_SELECTOR_0x08

push ax

retf

setProtectedModeAPI:                ; set protected mode from 'Real' mode. Called from 16 bit code, returns to 32 bit code

pushad                          ; save all registers as doublewords

mov eax, cr0

or al, 1

mov cr0, eax                    ; set the Protected Mode bit in the Control Register

xor eax, eax                    ; clear high bits of eax

call enterProtectedMode

[BITS 32]                           ; Protected Mode code (32 bit)

mov eax, data32p_SELECTOR_0x10  ; Protected Mode data segment

mov es, ax

mov ds, ax

mov ss, ax                      ; this makes stack segment 32 bits

popad

o16 ret

enter16bitProtectedMode:            ; 32 bit code. Must come from a 'call' , can not be inlined

pop eax                         ; return address

push dword code16r_SELECTOR_0x18    ; select 16-bit Protected Mode AKA 'Real' Mode

push eax

retf

setRealModeAPI:                     ; set 'Real' mode from protected mode.

; Called from 32 bit code, returns to 16 bit code

; assumed that protected-mode stack is based at 0

; and that bits 16 through 19 will not change during time in realmode

pushad                          ; save 32-bit values of registers

mov ecx, esp                    ; do all possible 32-bit ops before going to 16 bits

mov edx, cr0

call enter16bitProtectedMode

[BITS 16]                           ; Real Mode code (16 bit)

mov ax, data16r_SELECTOR_0x20

mov ds, ax

mov es, ax

mov ss, ax                      ; here the stack becomes 16 bits based at 0, and SP used not ESP

; *** consider stack to be invalid from here until we reach real mode ***

xor cx, cx                      ; clear low 16 bits

shr ecx, 4                      ; move high 4 bits into cl

dec dl                          ; leave protected mode, only works if we KNOW bit 0 is set

mov cr0, edx

call enterRealMode

xor ax, ax

mov ds, ax

mov es, ax

mov ss, cx

; note we don't need to set SP to 8xxx if ESP is b8xxx, since

; the b000 is now in SS, and the b of b8xxx is ignored in real mode

popad

o32 ret

enterRealMode:                      ; 16 bit code. Must come from a 'call' , can not be inlined

pop ax

push fs                         ; real-mode code segment

push ax

retf

[BITS 32]                           ; Protected Mode code (32 bit)

; *****************************************************************************

; *****************************************************************************

;%include "JCreadwrite.nasm"

; JCreadwrite.nasm 2012 Oct 23   read and write the disk using 16 bit BIOS calls

; BIOS read and write routines for colorForth

[BITS 32]                           ; Protected Mode code (32 bit)

bios_read:  ; ( a c -- a' c' )   \  read cylinder c into address a , leave next address and cylinder

; c is cylinder, we will use 1.44Mb floppy's idea of cylinder regardless

; a is byte address

; leave updated c and a on stack as c' and a'

; a cylinder is 36 tracks of 512 bytes each, 0x4800 bytes, 0x1200 cells (words)

cli                             ; disable interrupts

pushad                          ; push all registers ( except esp ) and flags onto the stack

mov ebp, esp                    ; copy of stack pointer for use below ( * ), points to registers copied by pushad , above

mov ecx, HEADS * SECTORS        ; sectors per track (both heads)

mul cl                          ; sector number goes into AX

; note that resultant sector number is zero-based going into sector_chs!

; set up loop to read one floppy cylinder's worth

push eax                        ; absolute sector number to start

.back:

push ecx

call sector_chs                 ; convert to Cylinder-Head-Sector in CX-DX

call .readsector

mov ebx, [ ebp + ( 1 * 4 ) ]    ; ( * ) get ESI stored on stack, via stack pointer saved in ebp

mov edi, [ebx]                  ; destination index address for movsd

mov ecx, ( 512 >> 2 )           ; number of 32-bit words to move, 512 bytes

mov esi, SECTOR_BUFFER          ; source index for movsd

rep  movsd                      ; copy ecx 32 bit words from ds:esi to es:edi

mov [ebx], edi

pop ecx

pop eax

inc eax

push eax

loop .back

pop eax

inc dword [ebp + 7 * 4]         ; for updated cylinder number after return

popad

ret

.readsector:                        ; no need to save registers because we take care of them in calling routine

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

mov bx, SECTOR_BUFFER

mov ax, 0x0201                  ; read 1 sector

int 0x13

cli                             ; BIOS might have left interrupts enabled

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

ret

bios_write:     ; ( a c -- a' c' )   \  write cylinder c from address a , leave next address and cylinder

cli                             ; disable interrupts

pushad

mov ebp, esp

; eax contains cylinder to start, the 'c' parameter

mov ecx, HEADS * SECTORS        ; sectors per track (both heads)

mul cl                          ; absolute sector number goes into AX

mov ebx, [ebp + ( 1 * 4 ) ]     ; stored ESI on stack

mov esi, [ebx]                  ; word address, 'a' parameter

;        shl esi, 2                 ; change word address into byte address

; set up loop to write one floppy cylinder's worth

push eax                        ; absolute sector number to start

.back:

push ecx

; load sector data into buffer

; DO NOT take advantage of knowing ECX only has byte value

mov ecx, 128 ; ( 512 >> 2 )     ; number of 32-bit words to move

mov edi, SECTOR_BUFFER

rep  movsd                      ; copy ecx 32 bit words from ds:esi to es:edi

call sector_chs                 ; convert to Cylinder-Head-Sector in CX-DX

call .writesector

pop ecx

pop eax

inc eax

push eax

loop .back

pop eax

inc dword [ ebp + ( 7 * 4 ) ]   ; for updated cylinder after return (EAX)

mov ebx, [ ebp + ( 1 * 4 ) ]    ; stored ESI on stack

mov [ebx], esi                  ; updated address

popad

ret

.writesector:                       ; no need to save registers because we take care of them in calling routine

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

mov bx, SECTOR_BUFFER

mov ax, 0x0301                  ; write 1 sector

int 0x13

cli                             ; BIOS might have left interrupts enabled

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

ret

times (0x400 - ($ - $$)) nop

; *****************************************************************************

; *****************************************************************************

; After Two Sectors

; *****************************************************************************

; *****************************************************************************

version:

db 'cf2023 1v0 2023Apr04 Chuck Moore' , 0x00    ; 0x20 + 1 bytes

db ' Howerd Oakford inventio.co.uk' ,  0x00     ; 0x1E + 1 bytes, total 0x40

nul:                ; do nothing...

ret

; *****************************************************************************

; Co-operative multi-tasker with comments from code by Josh Grams

; *****************************************************************************

; This version of colorforth has three tasks; main (the quit loop),

; draw (user defined), and serve (also user defined).  Each has two

; grows-down stacks.  A suffix of 's' indicates the return stack, 'd'

; indicates the data stack.  Thus 'draws' and 'drawd' are the tops of

; the return and data stacks, respectively, for the draw task.

; When we switch tasks, we need to switch stacks as well.  We do this

; by pushing eax (cached top-of-stack) onto the data stack, pushing

; the data stack pointer onto the return stack, and then saving the

; return stack pointer into the save slot for the task.

; 'me' points to the save slot for the current task

me:

dd main

x_screenTask:

dd nul

x_serverTask:

dd nul

x_serverTask2:

dd nul

pause_:

_DUP_

push esi

mov _TOS_, [ me ]               ; points to main at startup

mov [_TOS_], esp

add _TOS_, byte 0x04

jmp _TOS_

resume:

pop _TOS_

mov esp, [_TOS_]

mov [ me ], _TOS_

pop esi

_DROP_

ret

; these are the save slots - each is followed by code to resume the

; next task - the last one jumps 'round to the first.

round:

call resume

main:                               ; main task

dd 0                            ; new stack location

call resume

draw:                               ; screen draw task

dd 0                            ; new stack location

call resume

serv1:                               ; server task

dd 0                            ; new stack location

call resume

serv2:                              ; server task 2

dd 0                            ; new stack location

jmp short round                 ; loop forever between 3 stacks

activate:   ; ( a -- )    \ activate the draw task to execute colorForth code at the given address

mov edx, DATA_STACK_1 - 4

mov [edx], ecx

mov ecx, RETURN_STACK_1 - 4

pop dword [ecx]

lea ecx, [ ecx - 0x04 ]

mov [ecx], edx

mov dword [ draw ], ecx

_DROP_

ret

show:   ; ( -- )    \ set the screen task to execute the code following  show

pop dword [ x_screenTask ]      ; copy the return address of the calling word into the screenTask variable

_DUP_

xor _TOS_, _TOS_

call activate

.back:

call graphAction                ; perform a graphical update

call [ x_screenTask ]           ; execute the code that called show, saved on entry

call switch                     ; copy the screen image to the VESA buffer

xor _TOS_, _TOS_

call pause_

inc _TOS_

jmp short .back

initshow:                           ; called by warm

call show

; <--- this address ( on the return stack from the preceding call ) goes into  x_screenTask

ret                             ; makes this a no-op "show"

freeze:

pop dword [ x_screenTask ]

_DUP_

xor _TOS_, _TOS_

call activate

.back:

; call graphAction                ; perform a graphical update

call [ x_screenTask ]           ; execute the code that called show, saved on entry

; call switch                     ; copy the screen image to the VESA buffer

xor _TOS_, _TOS_

call pause_

inc _TOS_

jmp short .back

; *****************************************************************************

; ; Server task 1

; *****************************************************************************

activate1:  ; ( a -- )    \ activate the server task to execute colorForth code at the given address

mov edx, DATA_STACK_2 - 4

mov [edx], ecx

mov ecx, RETURN_STACK_2 - 4

pop dword [ecx]

lea ecx, [ ecx - 0x04 ]

mov [ecx], edx

mov [ serv1 ], ecx

_DROP_

ret

serv1_:

pop dword [ x_serverTask ]

call activate1

.back:

; call graphAction                ; perform a graphical update

call [ x_serverTask ]           ; execute the code that called show, saved on entry

; call switch                     ; copy the screen image to the VESA buffer

xor _TOS_, _TOS_

call pause_

inc _TOS_

jmp short .back

initserv1_:

call serv1_

ret

; *****************************************************************************

; Server task 2

; *****************************************************************************

activate2:  ; ( a -- )    \ activate the server task to execute colorForth code at the given address

mov edx, DATA_STACK_3 - 4

mov [edx], ecx

mov ecx, RETURN_STACK_3 - 4

pop dword [ecx]

lea ecx, [ ecx - 0x04 ]

mov [ecx], edx

mov [ serv2 ], ecx

_DROP_

ret

serv2_:

pop dword [ x_serverTask2 ]

call activate2

.back:

; call graphAction              ; perform a graphical update

call [ x_serverTask2 ]          ; execute the code that called show, saved on entry

; call switch                   ; copy the screen image to the VESA buffer

xor _TOS_, _TOS_

call pause_

inc _TOS_

jmp short .back

initserv2_:

call serv2_

ret

; *****************************************************************************

; *****************************************************************************

c_:     ; ( -- )   \ clear the data stack for keyboard task

mov esi, ( DATA_STACK_0 + 4 )

ret

; *****************************************************************************

; *****************************************************************************

mark:

mov ecx, [ v_MacroWordCount]

mov [ mark_MacroWordCount], ecx

mov ecx, [ v_ForthWordCount ]

mov [ mark_v_ForthWordCount], ecx

mov ecx, [ v_BlueWordCount ]

mov [ mark_v_BlueWordCount], ecx

mov ecx, [ v_H ]

mov [ mark_H ], ecx

ret

empty_:

cli                             ; disable interrupts

call initserv1_                 ; we must set the server tasks to their Nop loop

call initserv2_                 ; because the code that they might be running will soon be gone...

mov ecx, [ mark_H ]

mov [ v_H ], ecx

mov ecx, [ mark_v_BlueWordCount]

mov [ v_BlueWordCount ], ecx

mov ecx, [ mark_v_ForthWordCount]

mov [ v_ForthWordCount ], ecx

mov ecx, [ mark_MacroWordCount]

mov [ v_MacroWordCount ], ecx

ret

; *****************************************************************************

; *****************************************************************************

mfind:  ; ( sf -- )   \ ecx = index ; find the Shannon-Fano word sf in the Macro wordlist, return its index in ecx

mov ecx, [ v_MacroWordCount ]   ; count of Macro wordlist words

push edi

lea edi, [ ( ecx * 4 ) + MacroNames - 4 ]

jmp short ffind

find_:   ; ( sf -- )   \ ecx = index ; find the Shannon-Fano word sf in the Forth wordlist, return its index in ecx

mov ecx, [ v_ForthWordCount ]   ; count of Forth wordlist words

push edi

lea edi, [ ( ecx * 4 ) + ForthNames - 4 ]   ; set edi to the top of the Forth name table

ffind:

std                    ; scan backwards

repne scasd            ; find the 32 bit Shanon-Fano encoded name, compare eax with doubleword at es:edi and set status flags.

cld                    ; reset the direction flag

pop edi

ret

; *****************************************************************************

; *****************************************************************************

abort_:

jmp dword [ x_abort ]

; *****************************************************************************

; compile drop - inline compilation of a single byte 'lodsd' :

;   loads a 32 bit dword from [ds:esi] into  eax  then increments  esi  by 4

; *****************************************************************************

cdrop:

mov edx, [ v_H ]                ; HERE into edx

mov [ v_lastAddress ], edx      ; save HERE into v_lastAddress

mov byte [edx], 0xAD            ; 0xAD is the opcode for 'lodsd' = _DROP_

inc dword [ v_H ]               ; increment HERE

ret

; *****************************************************************************

; adup , dup and qdup

; *****************************************************************************

;  1147                                      _DUP_

;  1148 000004E7 8D76FC              <1>  lea esi, [ esi - 4 ]

;  1150 000004EA 8906                <1>  mov [ esi ], _TOS_

adup:

_DUP_   ; runtime action of the  _DUP_  macro_

ret

cdup:   ; compile the action of the  _DUP_  macro_

mov edx, [ v_H ]

; this code sets the flags

;    mov dword [edx], 0x8904EE83     ; assemble the instruction sequence for  DUP  "sub esi, byte 0x04" , "mov [esi], eax"

; this code does not set the flags

mov dword [edx], 0x89FC768D     ; assemble the instruction sequence for  DUP  "lea esi, [ esi - 4 ]" , "mov [esi], eax"

mov byte [ edx + 4 ], 0x06      ;  "8d 76 fc" , "89 06" ( the first 4 are expressed in little endian format above )

add dword [ v_H ], byte 0x05    ; update HERE by the 5 bytes we have just compiled

ret

; qdup is a Macro word that optimises what would be a compiled sequence of  DROP  DUP  into doing nothing

qdup:   ; compile a DUP unless we have just compiled a  DROP  , in which case just delete the  DROP.

mov edx, [ v_H ]                ; look at HERE

dec edx                         ; step back one byte

cmp dword [ v_lastAddress ], edx ; test if we have just compiled one byte in the current word

jnz cdup                        ; if we have just compiled one byte, compile in the runtime code for  DUP  after this one byte

cmp byte [edx], 0xAD            ; test if we have just compiled a DROP, 0xAD is the opcode for 'lodsd' = _DROP_

jnz cdup                        ; if we have not just compiled a DROP, compile in the runtime code for  DUP  after this one byte

mov [ v_H ], edx                ; update HERE to remove the DROP that was just compiled

ret

; *****************************************************************************

; *****************************************************************************

select_define:      ; select how to define the word

pop dword [ adefine ]

ret

macro_:     ; select the Macro wordlist

call select_define

macrod:

push _TOS_

mov ecx, [ v_MacroWordCount]                ; save the word count into ecx

inc dword [ v_MacroWordCount]               ; increment the word count for the next time around

; save the source address in the locate table

shl edi, 2                                  ; convert to an address in bytes

mov dword [ ( MacroLocates ) + ( ecx * 4 ) ], edi

shr edi, 2

lea ecx, [ ( ecx * 4 ) + MacroNames ]       ; ecx contains the address to store the new token name in the wordlist

mov _TOS_, ( MacroJumpTable - MacroNames )  ; _TOS_ contains the relative address of the jump table

jmp short add_word_to_wordlist

forth_:      ; select the Forth wordlist

call select_define

forthd:

push _TOS_

mov ecx, [ v_ForthWordCount ]               ; save the word count into ecx

inc dword [ v_ForthWordCount ]              ; increment the word count for the next time around

; save the source address in the locate table

shl edi, 2                                  ; convert to an address in bytes

mov dword [ ( ForthLocates ) + ( ecx * 4 ) ], edi

shr edi, 2                                  ; convert back to an address in 32 bit cells

lea ecx, [ ( ecx * 4 ) + ForthNames ]       ; ecx contains the address to store the new token name in the wordlist

mov _TOS_, ( ForthJumpTable - ForthNames )  ; _TOS_ contains the relative address of the jump table

; Note : falls through to add_word_to_wordlist

add_word_to_wordlist:

mov edx, [ ( edi * 4 ) - 0x04 ]     ; edi points to the source token in the block, edx contains the token Shannon-Fano token name to compile

and edx, dword 0xFFFFFFF0           ; 'and' out the token colour. Tokens in a wordlist of token names have a 'colour' of 0

mov [ecx], edx                      ; ecx contains the address to store the new token name in the wordlist

mov edx, [ v_H ]

mov [ecx+_TOS_], edx

lea edx, [ecx+_TOS_]

shr edx, 0x02

mov [ v_lastToken ], edx

pop _TOS_

mov [ v_lastAddress ], esp

mov dword [ lit ], adup

ret

; *****************************************************************************

; *****************************************************************************

alit:

mov dword [ lit ], adup

literal:

call qdup                       ; compile the runtime action for  DUP , unless we have just compiled a  DROP , in which case do nothing

mov edx, [ v_lastAddress ]      ; select the wordlist to add the literal to

mov [ v_lastAddress_copy ], edx

mov edx, [ v_H ]

mov [ v_lastAddress ], edx

mov byte [edx], _MOV_TOS_LIT_   ; the opcode for mov eax, 32_bit_literal  (in next 32 bit cell)

mov [ edx + 0x01 ], _TOS_       ; the literal value follows in the next 4 bytes in the dictionary

add dword [ v_H ], byte 0x05    ; move the dictionary pointer forward 5 bytes

ret

; *****************************************************************************

; *****************************************************************************

cnum:

call [ lit ]

mov _TOS_, [ ( edi * 4 ) + 0x00 ]

inc edi

jmp short cshrt

cshort:

call [ lit]

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

sar _TOS_, 0x05

cshrt:

call literal

_DROP_

ret

%if 1

; new improved code

; *****************************************************************************

; Magenta Variables run time code

; ecx contains the token number of the new Magenta Variable in the wordlist

;    This is found by searching the rodlist for the variable's name

; *****************************************************************************

m_var_forth_action:             ; code field for a magenta variable in the Forth wordist - return the variable's address

_DUP_                       ; runtime code to duplicate the TOS

mov [ v_test1 ], ecx

mov _TOS_, ecx              ; the offset in 32 bit cells of the Name of the current word being executed

shl _TOS_, 2                ; the offset in bytes of the Name of the current word being defined

add _TOS_, ForthLocates     ; the address in bytes of the Name of the current word being defined

mov _TOS_, [ _TOS_ ]        ; fetch the address in 32 bit cells of the data

;    shl _TOS_, 2                ; convert to an address in bytes

ret

m_var_macro_action:             ; code field for a magenta variable in the Macro wordist - compile a literal that returns the variable's address

call [ lit ]

mov [ v_test2 ], ecx

mov _TOS_, ecx              ; the offset in 32 bit cells of the Name of the current word being defined

shl _TOS_, 2                ; the offset in bytes of the Name of the current word being defined

add _TOS_, MacroLocates     ; the address in bytes of the Name of the current word being defined

mov _TOS_, [ _TOS_ ]        ; fetch the address in 32 bit cells of the data

; shl _TOS_, 2                ; convert to an address in bytes

jmp short cshrt

; *****************************************************************************

; Create a Magenta Variable

; ecx contains the token number of the new Magenta Variable in the wordlist

;    i.e. where the variable name is stored, set by forthd or macrod

; *****************************************************************************

m_variable:                     ; create a magenta variable

call forthd                 ; select the Forth wordlist

mov dword [ ( ForthJumpTable - ForthNames ) + ecx ], m_var_forth_action

shl edi, 2                  ; convert to an address in bytes

mov dword [ ( ForthLocates   - ForthNames ) + ecx ], edi

shr edi, 2                  ; convert back to an address in 32 bit cells

call macrod                 ; select the Macro wordlist

mov dword [ ( MacroJumpTable - MacroNames ) + ecx ], m_var_macro_action

shl edi, 2                  ; convert to an address in bytes

mov dword [ ( MacroLocates   - MacroNames ) + ecx ], edi

shr edi, 2                  ; convert back to an address in 32 bit cells

inc edi

ret

%else

; original code

; *****************************************************************************

; magenta variables

; ecx contains the address to store the new token name in the wordlist

;   set by forthd or macrod

; *****************************************************************************

m_var_forth_action:           ; code field for a magenta variable in the Forth wordist

_DUP_               ; runtime code to duplicate the TOS

mov _TOS_, [ 4 + ForthNames + ( ecx * 4 ) ]

shl _TOS_, 2

ret

m_variable:     ; define a magenta variable

call forthd         ; select the Forth wordlist

mov dword [ ForthJumpTable - ForthNames + ecx ], m_var_forth_action

inc dword [ v_ForthWordCount ]

mov [ ecx + 4 ], edi

call macrod         ; select the Macro wordlist

mov dword [ MacroJumpTable - MacroNames + ecx ], m_var_macro_action

inc dword [ v_MacroWordCount ]

mov [ ecx + 4 ], edi

inc edi

ret

m_var_macro_action:           ; code field for a magenta variable in the Macro wordist

call [ lit ]

mov _TOS_, [ 4 + MacroNames + ( ecx * 4 ) ]

shl _TOS_, 2

jmp short cshrt

%endif

; *****************************************************************************

; *****************************************************************************

ex1:

xor edi, edi

.back:

dec dword [ v_words ]

jz ex2

_DROP_

jmp short .back

execute_lit:    ; ( -- )

mov dword [ lit ], alit

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

execute_:    ; ( name -- )

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

ex2:

call find_

jnz abort_

_DROP_

jmp dword [ ( ecx * 4 ) + ForthJumpTable ]

; *****************************************************************************

; *****************************************************************************

qcompile:

call [ lit ]

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call mfind

jnz .forward

_DROP_

jmp dword [ ( ecx * 4 ) + MacroJumpTable ]

.forward:

call find_

mov _TOS_, [ ( ecx * 4 ) + ForthJumpTable ]

qcom1:

jnz abort_

call_:

mov edx, [ v_H ]

mov [ v_lastAddress ], edx

mov byte [edx], 0xE8        ; 0xE8 is the opcode for 'call immediate'

add edx, byte 0x05

sub _TOS_, edx

mov [ edx - 0x04 ], _TOS_

mov [ v_H ], edx

_DROP_

ret

; *****************************************************************************

; *****************************************************************************

compile:

call [ lit]

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call mfind

mov _TOS_, [ ( ecx * 4 ) + MacroJumpTable ]

jmp short qcom1

; *****************************************************************************

; *****************************************************************************

short_:

mov dword [ lit], alit

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

sar _TOS_, 0x05

ret

; *****************************************************************************

; *****************************************************************************

num:

mov dword [ lit], alit

_DUP_

mov _TOS_, [ ( edi * 4 ) + 0x00 ]

inc edi

ret

; *****************************************************************************

; *****************************************************************************

comma_:          ; 4 byte  ,

mov ecx, 0x04

dcomma:     ; c,  performed n times ( n in ecx )

mov edx, [ v_H ]

mov [edx], _TOS_

mov _TOS_, [ esi ]

lea edx, [ ecx + edx ]

lea esi, [ esi + 0x04 ]

mov [ v_H ], edx

ret

comma1_:     ; 1 byte  c,

mov ecx, 0x01

jmp short dcomma

comma2_:     ; 2 byte  w,

mov ecx, 0x02

jmp short dcomma

comma3_:     ; 3 byte  c, c, c,

mov ecx, 0x03

jmp short dcomma

; *****************************************************************************

; *****************************************************************************

semicolon:

mov edx, [ v_H ]

sub edx, byte 0x05

cmp [ v_lastAddress ], edx

jnz .forward

cmp byte [edx], 0xE8            ; 0xE8 is the opcode for 'call immediate'

jnz .forward

inc byte [edx]

ret

.forward:

mov byte [ edx + 0x05 ], 0xC3   ; 0xC3 is the opcode for 'ret'

inc dword [ v_H ]

ret

; *****************************************************************************

; *****************************************************************************

then:

mov [ v_lastAddress ], esp

mov edx, [ v_H ]

sub edx, _TOS_

mov [ _TOS_ - 0x01 ], dl

_DROP_

ret

begin_:

mov [ v_lastAddress ], esp

here_:

_DUP_

mov _TOS_, [ v_H ]

ret

; *****************************************************************************

; *****************************************************************************

qlit:  ; ?lit

mov edx, [ v_H ]

lea edx, [ edx - 0x05 ]

cmp [ v_lastAddress ], edx

jnz .forward

cmp byte [edx], _MOV_TOS_LIT_   ; the opcode for mov eax, 32_bit_literal  (in next 32 bit cell)

jnz .forward

_DUP_

mov _TOS_, [ v_lastAddress_copy ]

mov [ v_lastAddress ], _TOS_

mov _TOS_, [ edx + 0x01 ]

cmp dword [ edx - 5 ], 0x89FC768D   ; assemble code 8D 76 FC 89 rr => lea esi, [ esi - 0x04 ] ;  mov [ esi ], register

; like dup but with the register value still to follow in the next byte

jz .forward2

mov [ v_H ], edx

jmp dword cdrop

.forward2:

add dword [ v_H ], byte -0x0A

ret

.forward:

xor edx, edx

ret

less:

cmp [ esi ], _TOS_

js .forward

xor ecx, ecx

.forward:

ret

qignore:

test dword [ ( edi * 4 ) - 0x04 ], 0xFFFFFFF0

jnz .forward

pop edi

pop edi

.forward:

ret

jump:

pop edx

add edx, _TOS_

lea edx, [ edx + ( _TOS_ * 4 ) + 0x05 ]

add edx, [ edx - 0x04 ]

_DROP_

jmp edx

; convert block start address to cell address, add the RELOCATED colorForth system base

blockToCellAddress:  ; ( blk -- a' )   \ add the RELOCATED offset and convert to cell address

add _TOS_, [ v_offset ]   ; add the RELOCATED block number offset

shl _TOS_, 0x08           ; convert to cell address

ret

cellAddressToBlock:   ; ( a -- blk )  \ convert cell address to block number and subtract the RELOCATED block number offset

shr _TOS_, 0x08           ;  convert cell address to block number

sub _TOS_, [ v_offset ]   ; subtract the block number of block 0

ret

_load_:   ; ( blk -- )    \ load the given block number

call blockToCellAddress   ; add the RELOCATED block number offset and convert to cell address

push edi

mov edi, _TOS_

_DROP_

interpret:

mov edx, [ ( edi * 4 ) + 0x00 ]

inc edi

and edx, byte 0x0F

call [ ( edx * 4 ) + tokenActions ]

jmp short interpret

align 4, db 0   ; fill the gap with 0's

; : r@ qdup $8B 1, $C7 1, ;    \ mov _TOS_, edi  also db 0x89, 0xF8

; : nload r@ $0100 / #2 + load ;

; : +load ( n -- ) r@ $0100 / + load ;

nload:  ; ( -- )    \ load the next source block following the one currently being loaded

call cblk_

add _TOS_, 0x02

jmp _load_

plusLoad:   ; ( n -- )    \ load the n'th source block following the one currently being loaded

mov _SCRATCH_, _TOS_    ; save the required offset

_DROP_

call cblk_

add _TOS_, _SCRATCH_

jmp _load_

; : THRU ( f l -- )   1+ SWAP DO  I LOAD  LOOP ;

thru_:  ; ( first last -- )   \ load from the first to the last block

add _TOS_, 0x02

mov _SCRATCH_, _TOS_

_DROP_                          ; TOS = first, SCRATCH = last

mov ecx, _SCRATCH_

sub ecx, _TOS_                  ; ecx = count

jz .end                         ; exit if count is zero

jc .end                         ; exit if count is negative

shr ecx, 1                      ; divide by 2, as we skip 2 blocks each time round the loop

.back:

_DUP_

_DUP_   ; just to be safe...

push ecx

push _SCRATCH_

call _load_

pop _SCRATCH_

pop ecx

_DROP_  ; just to be safe...

add _TOS_, 0x02

loop .back

.end:

_DROP_

ret

v_temp:

dd 0

plusThru_:  ; ( first+ last+ -- )   \ load from the first to the last block relative to the current block being loaded

call cblk_

mov [ v_temp ], _TOS_

_DROP_

mov _SCRATCH_, [ v_temp ]

add [ esi ], _SCRATCH_          ; add current block to second on stack

add _TOS_, _SCRATCH_            ; add current block to top of stack

call thru_

ret

cblk_:   ; ( -- n )  \ return the currently compiling block number - only valid while compiling

_DUP_

mov _TOS_, edi            ; edi  contains the cell address in the block currently being compiled

call cellAddressToBlock   ; convert to block number relative to block 0

ret

rblk_:   ; ( -- n )  \ return the block number offset of the RELOCATED address

_DUP_

mov _TOS_, ( RELOCATED >> ( 2 + 8 ) )

ret

ablk_:   ; ( a -- n )  \ convert byte address to block number

shr _TOS_, 0x02

call cellAddressToBlock

ret

erase_:   ; ( a n -- )  \ erase n bytes starting at address a

mov ecx, eax

_DROP_

push edi

mov edi, eax

xor eax, eax

rep stosb

pop edi

_DROP_

ret

v_curs_to_source:   ; ( n -- a32 )   \ return the cell address of the current cursor position in the current block being edited

mov _SCRATCH_, _TOS_

mov _TOS_, [ v_blk ]            ; get the currently edited block number

call blockToCellAddress

add _TOS_, _SCRATCH_            ; add the cursor position (cell address) in the block

ret

nth_to_token:   ; ( n -- tok )   \ return the token at the n'th cursor position in the current block being edited

call v_curs_to_source

shl _TOS_, 0x02                 ; convert cell address to byte address

mov _TOS_, [ _TOS_ ]            ; fetch the token

ret

v_curs_to_token:   ; ( -- tok )   \ return the token at the current cursor position in the current block being edited

_DUP_

mov _TOS_, [ v_blk ]            ; get the currently edited block number

call nth_to_token

ret

; : ?f $C021 2, ;

;qf:

;    db 0x21, 0xC0   ; and _TOS_, _TOS_

;    ret

; *****************************************************************************

; *****************************************************************************

top_:   ; ( -- )   \ set the cursor to the left margin horizontally and 3 pixels down from the top vertically

mov ecx, [ v_leftMargin ]

shl ecx, 0x10

add ecx, byte 0x03

mov [ v_gr_xy ], ecx

; mov [ xycr], ecx

ret

qcr: ; ( -- )   \ ?cr  do a  CR  if the cursor has gone past the right margin

mov cx, [ v_gr_x ]

cmp cx, [ v_rightMargin ]

js cr_forward

cr_:  ; ( -- )

mov ecx, [ v_leftMargin ]

shl ecx, 0x10

mov cx, [ v_gr_xy ]

add cx, [ v_iconh ]

mov [ v_gr_xy ], ecx

cr_forward:

ret

green:  ; ( -- )

_DUP_

mov _TOS_, colour_green

jmp set_color_

yellow:  ; ( -- )

_DUP_

mov _TOS_, colour_yellow

jmp set_color_

; red:  ; ( -- )    ; see redWord:

;    _DUP_

;    mov _TOS_, colour_red

;    jmp set_color_

white:  ; ( -- )

_DUP_

mov _TOS_, colour_white

set_color_:  ; ( rgb16 -- )

mov [ v_foregroundColour ], _TOS_

_DROP_

ret

rgb:    ; ( rgb32 -- rgb16 )    ; convert from 32 bit ( 8:8:8:8 _RGB ) colour to 16 bit ( 5:6:5 RGB ) colour value

ror _TOS_, 8

shr ax, 2

ror _TOS_, 6

shr al, 3

rol _TOS_, ( 6 + 5 )

and _TOS_, 0x0000FFFF

ret

bye_:  ; ( -- )   \ exit colorForth

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

int 0x19    ; reboot the computer

; should never get past this point.... but in case we do...

cli                             ; BIOS might have left interrupts enabled

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

ret

%if 0

pci:

mov edx, 0x0CF8

out dx, _TOS_

lea edx, [ edx + 0x04 ]

in _TOS_, dx

ret

device:

times ( 0x93a - ( $ - $$ ) ) nop  ; fill with nops to find_display ???

find_display:                       ; called by warm

mov _TOS_, 0x3000000            ; PCI class code 3 = display controller

call device                     ; returns header address

lea _TOS_, [ _TOS_ + 0x10 ]     ; point to Base Address #0 (BAR0)

mov cl, 0x06

.next:

_DUP_

call pci

and al, 0xFB

xor al, 0x08

jz .forward

_DROP_

lea _TOS_, [ _TOS_ + 0x04 ]

loop .next

lea _TOS_, [ _TOS_ - 0x18 ]

_DUP_

call pci

and al, 0xF0

.forward:

mov [ v_frameBuffer ], _TOS_     ; set framebuffer address

_DROP_

ret

fifo:

_DROP_

ret

%endif

graphAction:

ret

; *****************************************************************************

; *****************************************************************************

; grapics mode dependent code

; *****************************************************************************

; *****************************************************************************

; *****************************************************************************

; 1024x768 display

; *****************************************************************************

scrnw1 equ 1024              ; screen width in pixels

scrnh1 equ 768               ; screen height in pixels

iconw1 equ ( 16 + 4 )        ; icon width

iconh1 equ ( 24 + 4 )        ; icon height for 768 pixel high screen

keypadY1 equ 4               ; location of keyboard display vertically in lines from the bottom

initIconSize1:

mov dword [ v_iconw ], iconw1

mov dword [ v_nine_iconw ], ( iconw1 * 9 )

mov dword [ v_twentytwo_iconw ], ( iconw1 * ( 13 + 9 ) )

mov dword [ v_10000_iconw ], ( iconw1 * 0x10000 )

mov dword [ v_iconh ], iconh1

mov dword [ v_keypadY_iconh ], keypadY1 * iconh1

ret

switch1:     ; copy our created image to the real display buffer

push esi

push edi

mov  esi, dword [ vframe ]      ; vframe  points to where we create our image

mov  edi, [ vesa_PhysBasePtr ]  ; VESA frame buffer, saved by VESA BIOS call, the address in RAM that is displayed by the hardware

mov  ecx, ( ( scrnw1 * scrnh1 ) / 4 ) * BYTES_PER_PIXEL   ; the / 4 is because we are moving doubles = 4 bytes each

rep  movsd                      ; copy ecx 32 bit words from ds:esi to es:edi

pop  edi

pop  esi

ret

clip1:

mov  edi, [ v_gr_xy ]

mov  ecx, edi

test cx, cx

jns  .forward

xor  ecx, ecx

.forward:

and  ecx, 0x0000FFFF

mov  [ v_yc ], ecx

imul ecx, ( scrnw1 * BYTES_PER_PIXEL )

sar  edi, 16

jns  .forward2

xor  edi, edi

.forward2:

mov  [ v_xc ], edi

lea  edi, [ edi * BYTES_PER_PIXEL + ecx ]

add  edi, [ vframe ]

ret

bit16:                              ; write a 16 x 24 glyph to the graphic screen

lodsw                           ; load the 16 bit value pointed to by SI into  ax

xchg al, ah                     ; eax_TOS_

.back:

shl ax, 0x01                    ; eax_TOS_

jnc  .forward

mov  [ edi ], dx                ;

jmp .forward2

.forward:

ror edx, 0x10                   ; use the background colour, in the high 16 bits

;    mov [ edi ], dx                ;

ror edx, 0x10                   ; return to the foreground colour, in the low 16 bits

.forward2:

add edi, byte BYTES_PER_PIXEL

loop .back

ret

; write the background after the glyph

bit16Background:          ; number of pixels to write in  ecx ,  screen address in  edi , colours in edx

ror edx, 0x10           ; use the background colour, in the high 16 bits

.back:

;    mov [ edi ], dx        ;

add edi, byte BYTES_PER_PIXEL

loop .back

ror edx, 0x10           ; return to the foreground colour, in the low 16 bits

ret

bit32:                          ; write a 32 x 48 double size glyph to the graphic screen

lodsw                   ; load the 16 bit value pointed to by SI into  ax

xchg al, ah             ; eax_TOS_

mov ecx, 0x10

.back:

shl  _TOS_, 1           ; eax_TOS_

jnc  .forward

mov  [ edi ], dx

mov  [ edi + BYTES_PER_PIXEL ], dx

cmp byte [ displayMode ], 0

jnz .width2

mov  [ edi + ( scrnw1 * BYTES_PER_PIXEL ) ], dx

mov  [ edi + ( scrnw1 * BYTES_PER_PIXEL ) + BYTES_PER_PIXEL ], dx

jmp .widthEnd

.width2:

mov  [ edi + ( scrnw2 * BYTES_PER_PIXEL ) ], dx

mov  [ edi + ( scrnw2 * BYTES_PER_PIXEL ) + BYTES_PER_PIXEL ], dx

.widthEnd:

.forward:

add edi, byte ( BYTES_PER_PIXEL * 2 )

loop .back

ret

; Table that maps the three levels of Shannon-Fano codes to ASCII, followed by a copy in Capitalised or larger form

; The original colorForth font has "capital numbers" - larger bold versions, all offset by 0x30 from the normal ones.

; The new font maps the "larger forms" to offset 0x80, i.e. $30 --> $B0, two forms of '0'.

ShannonFano:

;       000000001111111122222222222222222222222222222222                         <- levels

;       0123456789ABCDEF0123456789ABCDEF0123456789ABCDEF0123456789ABCDEF01234567 <- index

db 0x00     ; a space in the cf font

;  0123456789ABCDEF0123456789ABCDEF0123456789ABCDEF0123456789ABCDEF01234567 <- index

db "rtoeanismcylgfwdvpbhxuq0123456789j-k.z/;'!+@*,? RTOEANISMCYLGFWDVPBHXUQ" ; ASCII equivalents

; 89ABCDEF01

db 0xB0,0xB1,0xB2,0xB3,0xB4,0xB5,0xB6,0xB7,0xB8,0xB9; larger "capital"forms of "0123456789"

;   23456

db "J_K.Z"  ; normal capitals

; 789ABCDEF

db 0xAF,0xBB,0xA7,0xA1,0xAB,0xC0,0xAA,0xAC,0xBF; larger "capital" forms of "/;'!+@*,?"

; 0123456789ABCDEF0123456789ABCDEF

db 0x10,0x11,0x12,0x13,0x23,0x14,0x15,0x16,0x5B,0x5D,0x17,0x18,0x19,0x28,0x29,0x24 ; $60 ....#...[]...()$

emit1:      ; ( c -- )   \ display a single width and height character

call qcr

push esi

push edi

push edx

imul _TOS_, byte 16*24/8

mov esi, [ v_font ]

add esi, _TOS_

call clip1

mov edx, [ v_foregroundColour ]

mov ecx, 0x18   ; 24 lines

.back:

push ecx

mov ecx, 0x10

call bit16

mov ecx, 0x04

push edi

call bit16Background

pop edi

pop ecx

add edi, ( scrnw1 - 16 ) * BYTES_PER_PIXEL  ; address of the next line of the glyph

loop .back      ; next horizontal line

mov ecx, 0x04   ; 4 background lines

.back2:

push ecx

mov ecx, 0x10

call bit16Background

mov ecx, 0x04

push edi

call bit16Background

pop edi

pop ecx

add edi, ( scrnw1 - 16 ) * BYTES_PER_PIXEL  ; address of the next line of the glyph

loop .back2      ; next horizontal line

pop edx

pop edi

pop esi

_DROP_

space1:

add dword [ v_gr_xy ], iconw1 * 0x10000 ; 22 horizontal pixels

ret

two_emit1:  ; double width and height character

push esi

push edi

push edx

imul _TOS_, byte 16*24/8

mov esi, [ v_font ]

add esi, _TOS_

call clip1

mov edx, [ v_foregroundColour ]

mov ecx, 0x18   ; 24 lines

.back:

push ecx

call bit32

add edi, (2*scrnw1-16*2)*BYTES_PER_PIXEL

pop ecx

loop .back

pop edx

pop edi

pop esi

add dword [ v_gr_xy ], iconw1 * 2 * 0x10000 ; 44 horizontal pixels

_DROP_

ret

setupText__1:   ; setup for full screen text window display

call white

mov dword [ v_leftMargin ], 0x03

mov dword [ v_rightMargin ], ( scrnw1 - iconw1 )

jmp dword top_

box1: ; ( width height -- )

call clip1

cmp _TOS_, scrnh1+1

js .forward

mov _TOS_, scrnh1

.forward:

mov ecx, _TOS_

sub ecx, [ v_yc ]

jng .forward3

cmp dword [esi], scrnw1+1

js .forward2

mov dword [esi], scrnw1

.forward2:

mov _TOS_, [ v_xc ]

sub [esi], _TOS_

jng .forward3

mov edx, scrnw1

sub edx, [esi]

shl edx, PIXEL_SHIFT

mov _TOS_, [ v_foregroundColour ]

.back:

push ecx

mov ecx, [esi]

rep stosw   ; stosw depends on BYTES_PER_PIXEL, either stosw or stosd

add edi, edx

pop ecx

loop .back

.forward3:

_DROP_

_DROP_

ret

wash1:   ; ( colour -- )   \ fill the full screeen with the given colour

call set_color_

_DUP_

xor _TOS_, _TOS_     ; x,y = 0,0 top left corner

mov [ v_gr_xy ], _TOS_

mov _TOS_, scrnw1

_DUP_

mov _TOS_, scrnh1

jmp dword box_

; *****************************************************************************

; 800x600 screen

; *****************************************************************************

scrnw2 equ 800               ; screen width in pixels

scrnh2 equ 600               ; screen height in pixels

iconw2 equ ( 16 + 1 )        ; icon width

iconh2 equ ( 24 - 1 )        ; icon height for NC10 600 pixel high screen

keypadY2 equ 4               ; location of keyboard display vertically in lines from the bottom

initIconSize2:

mov dword [ v_iconw ], iconw2

mov dword [ v_nine_iconw ], ( iconw2 * 9 )

mov dword [ v_twentytwo_iconw ], ( iconw2 * ( 13 + 9 ) )

mov dword [ v_10000_iconw ], ( iconw2 * 0x10000 )

mov dword [ v_iconh ], iconh2

mov dword [ v_keypadY_iconh ], keypadY2 * iconh2

ret

switch2:     ; copy our created image to the real display buffer

push esi

push edi

mov  esi, dword [ vframe ]      ; vframe  points to where we create our image

mov  edi, [ vesa_PhysBasePtr ]  ; VESA frame buffer, saved by VESA BIOS call, the address in RAM that is displayed by the hardware

mov  ecx, ( ( scrnw2 * scrnh2 ) / 4 ) * BYTES_PER_PIXEL   ; the / 4 is because we are moving doubles = 4 bytes each

rep  movsd                      ; copy ecx 32 bit words from ds:esi to es:edi

pop  edi

pop  esi

ret

clip2:

mov  edi, [ v_gr_xy ]

mov  ecx, edi

test cx, cx

jns  .forward

xor  ecx, ecx

.forward:

and  ecx, 0x0000FFFF

mov  [ v_yc ], ecx

imul ecx, ( scrnw2 * BYTES_PER_PIXEL )

sar  edi, 16

jns  .forward2

xor  edi, edi

.forward2:

mov  [ v_xc ], edi

lea  edi, [ edi * BYTES_PER_PIXEL + ecx ]

add  edi, [ vframe ]

ret

emit2:      ; ( c -- )   \ display a single width and height character

call qcr

push esi

push edi

push edx

imul _TOS_, byte 16*24/8

mov esi, [ v_font ]

add esi, _TOS_

call clip2

mov edx, [ v_foregroundColour ]

mov ecx, 0x18   ; 24 lines

.back:

push ecx

mov ecx, 0x10

call bit16

mov ecx, 0x04

push edi

call bit16Background

pop edi

pop ecx

add edi, ( scrnw2 - 16 ) * BYTES_PER_PIXEL  ; address of the next line of the glyph

loop .back      ; next horizontal line

mov ecx, 0x04   ; 4 background lines

.back2:

push ecx

mov ecx, 0x10

call bit16Background

mov ecx, 0x04

push edi

call bit16Background

pop edi

pop ecx

add edi, ( scrnw2 - 16 ) * BYTES_PER_PIXEL  ; address of the next line of the glyph

loop .back2      ; next horizontal line

pop edx

pop edi

pop esi

_DROP_

space2:

add dword [ v_gr_xy ], iconw2 * 0x10000 ; 22 horizontal pixels

ret

two_emit2:  ; double width and height character

push esi

push edi

push edx

imul _TOS_, byte 16*24/8

mov esi, [ v_font ]

add esi, _TOS_

call clip2

mov edx, [ v_foregroundColour ]

mov ecx, 0x18   ; 24 lines

.back:

push ecx

call bit32

add edi, (2*scrnw2-16*2)*BYTES_PER_PIXEL

pop ecx

loop .back

pop edx

pop edi

pop esi

add dword [ v_gr_xy ], iconw2 * 2 * 0x10000 ; 44 horizontal pixels

_DROP_

ret

setupText__2:   ; setup for full screen text window display

call white

mov dword [ v_leftMargin ], 0x03

mov dword [ v_rightMargin ], ( scrnw2 - iconw2 )

jmp dword top_

box2: ; ( width height -- )

call clip2

cmp _TOS_, scrnh2+1

js .forward

mov _TOS_, scrnh2

.forward:

mov ecx, _TOS_

sub ecx, [ v_yc ]

jng .forward3

cmp dword [esi], scrnw2+1

js .forward2

mov dword [esi], scrnw2

.forward2:

mov _TOS_, [ v_xc ]

sub [esi], _TOS_

jng .forward3

mov edx, scrnw2

sub edx, [esi]

shl edx, PIXEL_SHIFT

mov _TOS_, [ v_foregroundColour ]

.back:

push ecx

mov ecx, [esi]

rep stosw   ; stosw depends on BYTES_PER_PIXEL, either stosw or stosd

add edi, edx

pop ecx

loop .back

.forward3:

_DROP_

_DROP_

ret

wash2:    ; ( colour -- )   \ fill the full screeen with the given colour

call set_color_

_DUP_

xor _TOS_, _TOS_     ; x,y = 0,0 top left corner

mov [ v_gr_xy ], _TOS_

mov _TOS_, scrnw2

_DUP_

mov _TOS_, scrnh2

jmp dword box_

; *****************************************************************************

; select which display mode code to use

; *****************************************************************************

displayMode:

dd 1    ; 0 = 1024x768x16, 1 = 800x600x16

initIconSize: ; sets up the size of an icon (glyph) according to the 800x600 or 1024x768 display size

cmp byte [ displayMode ], 0

jz initIconSize1

jmp initIconSize2

switch:

cmp byte [ displayMode ], 0

jz switch1

jmp switch2

clip:

cmp byte [ displayMode ], 0

jz clip1

jmp clip2

emitSF_:

mov al, [ ShannonFano + _TOS_ ]

emit_:      ; ( c -- )   display byte c on the screen

cmp byte [ displayMode ], 0

jz emit1

jmp emit2

space_:

cmp byte [ displayMode ], 0

jz space1

jmp space2

type_:      ; ( a n -- )   display n bytes at address a on the screen

mov ecx, _TOS_

_DROP_

mov _SCRATCH_, _TOS_

.back:

pusha

_DUP_

mov al, [ _SCRATCH_ ]

and _TOS_, 0x000000FF

call emit_

popa

inc _SCRATCH_

loop .back

_DROP_

ret

; double size versions of emit, 32 x 48 pixels per glyph

two_emit_SF:

mov al, [ ShannonFano + _TOS_ ]

two_emit:

cmp byte [ displayMode ], 0

jz two_emit1

jmp two_emit2

setupText_:     ; setup for full screen text window display

cmp byte [ displayMode ], 0

jz setupText__1

jmp setupText__2

line_:  ; ( startX length -- )   \ draw a horizontal line in the current colour, from startX relative to current clip window, of given length in pixels

cmp byte [ displayMode ], 0

jnz .forward

call clip1

jmp .common

.forward:

call clip2

.common:

mov ecx, [esi]

shl ecx, PIXEL_SHIFT

sub edi, ecx

mov ecx, _TOS_

mov _TOS_, [ v_foregroundColour ]

rep stosw   ;

inc dword [ v_gr_xy ]

_DROP_

_DROP_

ret

box_:

cmp byte [ displayMode ], 0

jz box1

jmp box2

page_:    ; ( -- )    \ fill the full screen with the current background colour

_DUP_

mov _TOS_, colour_background   ;

jmp wash_

screen_:    ; ( -- )   \ fill the full screen with the current foreground colour

_DUP_

mov _TOS_, [ v_foregroundColour ]  ;     ; select the foreground colour in the low 16 bits

;    jmp wash_                      ; fall through to wash1

wash_:  ; ( colour -- )   \ fill the full screeen with the given colour

mov [ v_washColour ], _TOS_

cmp byte [ displayMode ], 0

jz wash1

jmp wash2

; *****************************************************************************

; *****************************************************************************

; *****************************************************************************

setCyan:    ; ( -- )

_DUP_

mov _TOS_, colour_cyan

jmp dword set_color_

setMagenta:    ; ( -- )

_DUP_

mov _TOS_, colour_magenta

jmp dword set_color_

setMagentaData:    ; ( -- )

_DUP_

mov _TOS_, colour_magentaData

jmp dword set_color_

setBlue:    ; ( -- )

_DUP_

mov _TOS_, colour_blue

jmp dword set_color_

setRed:    ; ( -- )

_DUP_

mov _TOS_, colour_red

jmp dword set_color_

setGreen:    ; ( -- )

_DUP_

mov _TOS_, colour_green

jmp dword set_color_

setSilver:    ; ( -- )         \ AKA grAy

_DUP_

mov _TOS_, colour_silver

jmp dword set_color_

history:

times 11 db 0

echo_:

push esi

mov ecx, 11-1

lea edi, [ history ]

lea esi, [ edi + 1 ]

rep movsb

pop esi

mov byte [ history+11-1 ], al

_DROP_

ret

right:

_DUP_

mov ecx, 11

lea edi, [history]

xor _TOS_, _TOS_

rep stosb

_DROP_

ret

down:

_DUP_

xor edx, edx

mov ecx, [ v_iconh ]

div ecx

mov _TOS_, edx

sub edx, [ v_iconh ]

add edx, ( 3 * 0x10000 )+ 0x8000 + 3

mov [ v_gr_xy ], edx

; zero:

test _TOS_, _TOS_

mov _TOS_, 0

jnz .dw

inc _TOS_

.dw:

ret

lm:     ; ( leftMargin -- )

mov [ v_leftMargin ], _TOS_

_DROP_

ret

rm:     ; ( rightMargin -- )

mov [ v_rightMargin ], _TOS_

_DROP_

ret

_at:    ; ( y x -- )

mov word [ v_gr_y ], ax

_DROP_

mov word [ v_gr_x ], ax

_DROP_

ret

plus_at:    ; ( y x -- )

add word [ v_gr_y ], ax

_DROP_

add word [ v_gr_x ], ax

_DROP_

ret

storew_:    ; ( w a -- ) \  ; : !w a! $00028966 3, drop ;

db 0x8B, 0xD0           ; mov edx,eax       a! $D08B 2,   ( ?lit not true )

db 0x66, 0x89, 0x02     ; mov [edx],ax      $00028966 3,

_DROP_                  ; lodsd

ret                     ; ret

storeu_:    ; ( u a -- ) \  ; : !l a! $0289 2, drop ; forth

db 0x8B, 0xD0           ; mov edx,eax       a! $D08B 2,   ( ?lit not true )

db 0x89, 0x02           ; mov [edx],eax     $0289 2,

_DROP_                  ; lodsd

ret                     ; ret

uplus_: ; ( u u -- u )   \ : u+ ?lit if $0681 2, , ; then $00044601 3, drop ;

db 0x01, 0x46, 0x04     ; add [esi+0x4],eax   $00044601 3,   ( ?lit not true )

_DROP_                  ; lodsd

ret                     ; ret

%if 1

; the various pieces of code used by a! and +! in colorForth blocks 22 and 24

plusStore:  ; ( n a -- )

;  : a! ?lit if $BA 1, , ; then $D08B 2, drop ;

mov dword edx, 0x12345678                  ; db 0xBA, 0x78, 0x56, 0x34, 0x12

mov edx, _TOS_                               ; db 0x8B, 0xD0 == db 0x89, 0xC2

;  : +! ?lit if ?lit if $0581 2, swap a, , ; then $0501 2, a, drop ; then a! $0201 2, drop ;

add [ dword 0x12345678 ], _TOS_              ; db 0x01, 0x05, 0x78, 0x56, 0x34, 0x12

add dword [ dword 0x12345678 ], 0x98765432 ; db 0x81, 0x05, 0x78, 0x56, 0x34, 0x12, 0x32, 0x54, 0x76, 0x98

add [ edx ], _TOS_                           ; db 0x01, 0x02

ret

%endif

octant:

_DUP_

mov _TOS_, 0x43

mov edx, [ esi + 0x04 ]

test edx, edx

jns .forward

neg edx

mov [ esi + 0x04 ], edx

xor al, 0x01

.forward:

cmp edx, [ esi ]

jns .forward2

xor al, 0x04

.forward2:

ret

hicon:

db 0x30, 0x31, 0x32, 0x33

db 0x34, 0x35, 0x36, 0x37

db 0x38, 0x39, 0x61, 0x62

db 0x63, 0x64, 0x65, 0x66

;  db 0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F

;  db 0x20, 0x21, 0x05, 0x13, 0x0A, 0x10, 0x04, 0x0E

edig1:

_DUP_

digit:

push ecx

mov al, [ _TOS_ + hicon ]

call emit_

pop ecx

ret

odig:

rol _TOS_, 0x04

_DUP_

and _TOS_, byte 0x0F

ret

h_dot_n:

mov edx, _TOS_

neg _TOS_

lea ecx, [ ( _TOS_ * 4 ) + 0x20 ]

_DROP_

rol _TOS_, cl

mov ecx, edx

jmp short h_dot_one

dotHex8_: ; ( u -- )   \ display a hexadecimal number with leading zeros, 8 .hex

mov ecx, 0x08

h_dot_one:

call odig

call digit

loop h_dot_one

_DROP_

ret

dotHex2_: ; ( c -- )   \ display a hexadecimal number with leading zeros, 2 .hex

shl _TOS_, 24

mov ecx, 0x02

call h_dot_one

ret

dotHex4_: ; ( w -- )   \ display a hexadecimal number with leading zeros, 4 .hex

shl _TOS_, 16

mov ecx, 0x04

call h_dot_one

ret

dotHex:     ; ( u -- )   \ display a hexadecimal number

EMIT_IMM('$')

mov ecx, 0x07

.back:

call odig

jnz .forward

_DROP_

loop .back

inc ecx

.back2:

call odig

.back3:

call digit

loop .back2

call space_

_DROP_

ret

.forward:

inc ecx

jmp short .back3

qdot:   ; ( u -- )   \ display a decimal or hexadecimal number, depending on base

cmp dword [ base ], byte 10

jnz dotHex

dotDecimal:                         ; display a decimal number

;    EMIT_IMM('#')

mov edx, _TOS_

test edx, edx

jns .forward

neg edx                         ; negate the value and display a minus sign if required

EMIT_IMM('-')

.forward:

mov ecx, 0x08

.back:

mov _TOS_, edx

xor edx, edx

div dword [ ecx * 4 + tens ]

test _TOS_, _TOS_

jnz .forward2

dec ecx

jns .back

jmp short .forward3

.back2:

mov _TOS_, edx

xor edx, edx

div dword [ ecx * 4 + tens ]

.forward2:

call edig1

dec ecx

jns .back2

.forward3:

mov _TOS_, edx

call edig1

call space_

_DROP_

ret

eight:  ; display eight characters for one long line in a keypad mnemonic, with a space between the groups of four

add edi, byte 0x0C

call four

call space_

sub edi, byte 0x10

four:       ; display four characters for one line in a keypad mnemonic

mov ecx, 0x04

four1:      ; set ecx to the required number of characters to display

push ecx

_DUP_

xor _TOS_, _TOS_

mov al, [edi+0x04]

inc edi

call emit_

;    call emitSF_    ; Note : The characters returned by a keypad are Shannon-Fano encoded

pop ecx

loop four1

ret

displayTheStack:  ; display the stack

mov edi, ( DATA_STACK_0 - 4 )   ; save empty stack pointer, plus one ( stack grows downwards )

.back:

mov edx, [ main ]        ; copy the current stack pointer

cmp [edx], edi

jnc .forward            ; test for empty stack, meaning done

_DUP_

mov _TOS_, [edi]          ; fetch the value of the current stack item

sub edi, byte 0x04      ;

call qdot               ; display one stack item

jmp short .back         ; next stack item

.forward:

ret

yShift equ 3

displayBlockNumber:    ;  ( -- )     ; in the top right corner of the screen

_DUP_

mov _TOS_, [ v_foregroundColour ]

_DUP_

mov _TOS_, [ vesa_XResolution ]   ; was this  : mov _TOS_, ( scrnw )

and _TOS_, 0xFFFF

sub _TOS_, [ v_nine_iconw ]

mov _SCRATCH_, _TOS_                ; save for later

mov [ v_leftMargin ], _TOS_

mov [ word v_gr_y ], ax

add _TOS_, [ v_nine_iconw ]

mov [ v_rightMargin ], _TOS_

mov _TOS_, _SCRATCH_

shl _TOS_, 16

add _TOS_, yShift

mov [ v_gr_xy ], _TOS_

_DUP_

mov _TOS_, [ v_washColour ]  ; so we do not see the number yet, just measure its width

;    mov _TOS_, colour_blockNumber

;    shr _TOS_, 16                 ; select the background colour in the high 16 bits

call set_color_

_DUP_

mov _TOS_, [ v_blk ]

call qdot

mov _SCRATCH_, [ v_gr_xy ]            ; current x,y coordinate, x in high 16 bits

shr _SCRATCH_, 16

sub _SCRATCH_, [ v_leftMargin ]   ; _SCRATCH_ is now the width of number string, in pixels

sub _SCRATCH_, [ v_iconw ]        ; correction...

shl _SCRATCH_, 16

mov _TOS_, [ vesa_XResolution ]   ; screen width in pixels

; and _TOS_, 0xFFFF   ; not needed because of the  shl  below

shl _TOS_, 16

add _TOS_, yShift

sub _TOS_, _SCRATCH_

mov [ v_gr_xy ], _TOS_

_DUP_

mov _TOS_, colour_blockNumber

ror _TOS_, 16

call set_color_

_DUP_

mov _TOS_, [ v_iconw ]

add _TOS_, _TOS_

_DUP_

mov _TOS_, [ v_iconh ]

call box_

mov [ v_gr_xy ], _TOS_

mov _TOS_, colour_blockNumber

_DUP_

call set_color_

_DUP_

mov _TOS_, [ v_blk ]

;    mov _TOS_, [ v_numberOfMagentas ]

call qdot

_DROP_

mov [ v_foregroundColour ], _TOS_

_DROP_

ret

; *****************************************************************************

; keyboard displays

; *****************************************************************************

showEditBox:    ; v_at set up for start coordinate of box, width and height on stack

sub dword [ v_gr_xy ], 0x000C0004   ; move the start position left  and up   by 0xXXXXYYYY

mov dword _SCRATCH_, [ v_foregroundColour ]

mov dword [ v_foregroundColour ], colour_orange

mov ecx, 2

.loop:

push ecx

_DUP_

mov _TOS_, 0            ; SOS = x start position in pixels, relative to current clip "window"

_DUP_

mov _TOS_, [ v_iconw ]

shl _TOS_, 3            ; multiply by 8

add _TOS_, [ v_iconw ]  ; multiply by 9

add _TOS_, [ v_iconw ]  ; multiply by 10

; TOS = length of horizontal line in pixels

call line_

mov ecx, [ v_iconh ]

shl ecx, 2              ; multiply by 4

add ecx, 4              ; draw the lower line below the text

add dword [ v_gr_xy ], ecx ; move the start position down by 4 character heights

pop ecx

loop .loop

mov dword [ v_foregroundColour ], _SCRATCH_

ret

; keypa (d)

displayTheKeypad_:   ; the Keypad is the mnemonic at the bottom right of the display, showing the actions of each of the 27 keys used

call setupText_

mov edi, [ dword currentKeypadIcons ]

_DUP_

mov _TOS_, [ keypad_colour ]

call set_color_

mov _TOS_, [ vesa_XResolution ] ; was this  : mov _TOS_, ( scrnw )

and _TOS_, 0xFFFF

sub _TOS_, [ v_nine_iconw ]

sub _TOS_, 16

mov [ v_leftMargin ], _TOS_     ; x coordinate of left margin of keypad display

mov edx, _TOS_                  ;

add edx, [ v_nine_iconw ]       ; x coordinate of right margin of keypad display

mov [ v_rightMargin ], edx

shl _TOS_, 0x10

mov edx, [ vesa_YResolution ]   ; was this  : mov _TOS_, ( scrnw )

and edx, 0x0000FFFF

push _SCRATCH_

mov _SCRATCH_, [ v_keypadY_iconh ]

add _SCRATCH_, 10

sub edx, _SCRATCH_              ; ( ( keypadY * iconh ) + 10 )

add _TOS_, edx

mov [ v_gr_xy ], _TOS_

test byte [ v_quitMode ], 0xFF

jz .forward

pusha

call showEditBox

popa

mov [ v_gr_xy ], _TOS_

.forward:

pop _SCRATCH_

call eight

call eight

call eight

call cr_

;    add dword [ v_gr_xy ],  ( 4 * iconw * 0x10000 )        ; shift horizontal pixels to the right

mov _SCRATCH_, [ v_iconw ]

shl _SCRATCH_, ( 2 + 16 ) ; ( 4 * iconw * 0x10000 )  ; shift horizontal pixels to the right

add dword [ v_gr_xy ], _SCRATCH_

mov edi, [ shiftAction ]

add edi, byte 0x0C

mov ecx, 0x03

call four1

call space_

_DUP_

mov _TOS_, [ v_hintChar ]

call emit_

mov dword [ v_leftMargin ], 0x03

mov word [ v_gr_x ], 0x03

call displayTheStack

mov _TOS_, [ vesa_XResolution ]   ; was this  : mov _TOS_, ( scrnw )

and _TOS_, 0xFFFF

sub _TOS_, [ v_twentytwo_iconw ]

add _TOS_, 3

mov word [ v_gr_x ], ax

lea edi, [ ( history - 4 )]     ; the text entered so far

mov ecx, 0x0B

jmp dword four1

; *****************************************************************************

; Tables of keys to return when each of the 24 main keypad positions are pressed

; Note : The keypad key lists below use Shannon-Fano encoded characters

; *****************************************************************************

alphaKeypad:        ; the 'alpha' character keypad keys, the start screen for key entry

db 'gcrl' ;

db 'htns' ;

db 'bmwv' ;

db 'pyfi' ;

db 'aoeu' ;

db 'qkxd' ;

graphicsKeypad:     ; the 'graphics' character keypad icons (Note: not numbers, just characters)

db '123 ' ;     Note : these are Capital (larger) numbers

db '4560' ;

db '789?' ;

db ':;!@' ;

db 'zj.,' ;

db '*/+-' ;

decimalKeypad:      ; the decimal number entry keypad icons

db '123 ' ;

db '4560' ;

db '789 ' ;

db '    ' ;

db '    ' ;

db '    ' ;

hexadecimalKeypad:  ; the hexadecimal number entry keypad icons

db '123 ' ;

db '4560' ;

db '789 ' ;

db ' abc' ;

db ' def' ;

db '    ' ;

; *****************************************************************************

; get keyboard keys

; *****************************************************************************

letter:

cmp al, 0x04  ; ignore 0 to 3, NOP, N, spacebar, AltGr

js .forward

mov edx, [ currentKeypadIcons ]

mov al, [ _TOS_ + edx ]

.forward:

ret

key_map_table:   ; map 8042 scan type 1 keycode to colorForth character values

db 16, 17, 18, 19,  0,  0,  4,  5 ; 0x10 - 0x17

db  6,  7,  0,  0,  0,  0, 20, 21 ; 0x18 - 0x1F

db 22, 23,  0,  0,  8,  9, 10, 11 ; 0x20 - 0x27

db  0,  0,  0,  0, 24, 25, 26, 27 ; 0x28 - 0x2F

db  0,  1, 12, 13, 14, 15,  0,  0 ; 0x30 - 0x37 N

db  3,  2                         ; 0x38 - 0x39 alt space

; ToDo: add a timeout to the loop

WaitToReceiveKey:      ; Wait until there is byte to receive from the keyboard controller

.back:

in al, 0x64     ; On-board controller status read

test al, 1      ; OBF (Output Buffer Full)

jnz .forward    ; exit when bit 0 = 1 the On-board controller has a new character for us

xor _TOS_, _TOS_

call pause_    ; not ready yet, so let the other task(s) have a turn

jmp  .back      ; jump back and try again

.forward:

;    call pause_    ; not ready yet, so let the other task(s) have a turn

ret

v_lineOffsetTablePtr:

dd 0 ; times 16 dd 0

lineOffsetZero:

mov dword [ v_lineOffset ], 0x00

ret

lineOffsetPlus:

add dword [ v_lineOffset ], 0x0C

ret

lineOffsetMinus:

sub dword [ v_lineOffset ], 0x0C

jns .forward

call lineOffsetZero

.forward:

ret

; *****************************************************************************

; F1 Help screens

; *****************************************************************************

help0:  ; save  v_blk , display the first help screen

_DUP_

cmp dword [ v_blk ], LAST_BLOCK_NUMBER     ; we are displaying the first Help screen

je .forward

mov _TOS_, [ v_blk ]

mov [ v_saved_v_blk ], _TOS_

.forward:

mov dword [ v_blk ], LAST_BLOCK_NUMBER

_DROP_

ret

help1:  ; display the second help screen

mov dword [ v_blk ], ( START_BLOCK_NUMBER + 1 )

ret

help2:  ; display the second third screen

mov dword [ v_blk ], ( START_BLOCK_NUMBER )

ret

help3:  ; restore the original screen being edited

_DUP_

mov _TOS_, [ v_saved_v_blk ]

mov [ v_blk ], _TOS_

_DROP_

ret

HelpTable:

dd help0

dd help1

dd help2

dd help3

help:

_DUP_

mov _TOS_, [ v_help_counter ]

and _TOS_, 0x03

call dword [ ( _TOS_ * 4 ) + HelpTable ]

_DROP_

inc byte [ v_help_counter ]

ret

; *****************************************************************************

; Editor

; *****************************************************************************

e_plus:

call colourBlindModeToggle

jmp abort_e

abort_e:

; call abort

call c_

abort_e2:

mov esp, RETURN_STACK_0

call e_

ret

EnterKeyAction:   ; ( -- )    \ action when the keyboard enter key is pressed

; mov byte [ v_quitMode ], 0x00     ; turn off the edit mode orange lines around the keypad

; _DUP_

mov _TOS_, [ v_cad ]

sub _TOS_, 1                ; step to before the token before the cursor

shl _TOS_, 2                ; convert cell address to byte address

mov _TOS_, [ _TOS_ ]

mov _SCRATCH_, _TOS_

and _SCRATCH_, 0x0F         ; check the token type = 3 == red

cmp _SCRATCH_, 0x03

je  .redEnterAction

cmp _SCRATCH_, 0x0C         ; check the token type = 12 == magenta.  NOT WORKING YET ToDo: fix this

je  .magentaEnterAction

cmp _SCRATCH_, 0x04

je  .greenEnterAction       ; check the token type = 4 == green

jmp .skip

.greenEnterAction:

; and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call fnd_

call loc_

jmp .skip

.redEnterAction:

; and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call execute_

jmp .skip

.magentaEnterAction:    ; ToDo: make this work!!!

; and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

_DUP_

mov _TOS_, [ v_cad ]

_DUP_

jmp .skip

mov _TOS_, [ v_cad ]

sub _TOS_, 2                ; step to before two tokens before the cursor

shl _TOS_, 2                ; convert cell address to byte address

; mov _TOS_, [ _TOS_ ]

; call execute_

_DUP_

jmp .skip

.skip:

_DROP_

ret

%define FirstFkey  (59)     ; F1 = 59

FkeyTable:  ; ( c -- a )   \ function key action table

;    dd nul              ; 57

;    dd nul              ; 58

dd help             ; 59 F1

dd toggleBase0      ; 60 F2 decimal/hex number display

dd seeb             ; 61 F3 show/hide blue words

dd e_plus           ; 62 F4 editor

dd tog_show_ASCII   ; 63 F5 show/hide the ASCII keyboard entry field at the cursor

dd otherBlock       ; 64 F6 display the previously edited block

dd locate           ; 65 F7 locate the definition of the word under the cursor

dd nul              ; 66 F8

dd toggleBase       ; 67 F9

dd c_               ; 68 F10

dd nul              ; 69 Num Lock

dd nul              ; 70

dd cursorHome       ; 71 Home

dd cursorUp         ; 72 Up arrow

dd nextBlock        ; 73 PgUp

dd nul              ; 74 -

dd cursorLeft       ; 75 Left arrow

dd otherBlock       ; 76    display the previously edited block

dd cursorRight      ; 77 Right arrow

dd nul              ; 78 +

dd cursorEnd        ; 79 End

dd cursorDown       ; 80 Down arrow

dd previousBlock    ; 81 PgDn

dd destack          ; 82 Insert

dd deleteAction     ; 83 Delete

dd nul              ; 84

dd nul              ; 85

dd nul              ; 86

dd toggleBase0      ; 87 F11

dd nul              ; 88 F12

dd EnterKeyAction   ; 89 really 121 Enter

dd abort_e          ; 90 really 123 Escape

processFkey:    ; ( n -- )   \ process the given function key code

;    cmp _TOS_, 121

;    jne .forward1

;    sub _TOS_, ( 121 - 89 )

;.forward1:

sub _TOS_, FirstFkey   ; convert Fn key value to index from 0

and _TOS_, 0x1F

call dword [ ( _TOS_ * 4 ) + FkeyTable ]

;    _DROP_

; call e_

ret

get_key_:    ; ( -- c )   \ waits for and returns a character from the keyboard, assumes Scan Code Set 1, set up by the BIOS

_DUP_

xor _TOS_, _TOS_

.back:

; check if the key is a function key

cmp _TOS_, FirstFkey  ; F1 key

js .forward4

cmp _TOS_, FirstFkey + 32  ; Fxx key + 1

jns .forward4

call processFkey

.forward4:

_DROP_

call get_qwerty_key_

;    call WaitToReceiveKey  ; Wait until there is a byte to receive from the keyboard controller

;    in   al, 0x60       ; read the key value from the Keyboard data port

mov al, [ v_scanCode ]

;    test al, 0xF0       ; we are only interested in certain key codes (?)

;    jz   .back

cmp  al, 0x3A       ; exclude keycodes greater than 0x39,  cmp  is like  sub  but only affects the flags

jnc  .back

mov  al, [ key_map_table - 0x10 + EAX ] ; convert to the colorForth value using the 'key_map_table' table

ret

; *****************************************************************************

; get qwerty keys

; *****************************************************************************

align 4, db 0   ; fill the gap with 0's

; times 0x40 db 0x00,

qwerty_key_map_table:

;         0     1     2     3     4     5     6     7     8     9     A     B     C     D     E     F

db 0x0B, 0x18, 0x02, 0x19, 0x03, 0x1A, 0x04, 0x1B, 0x05, 0x1C, 0x06, 0x1D, 0x07, 0x1E, 0x08, 0x1F ; 0x00

db 0x09, 0x20, 0x0A, 0x21, 0x1e, 0x05, 0x30, 0x13, 0x2E, 0x0A, 0x20, 0x10, 0x12, 0x04, 0x21, 0x0E ; 0x10

db 0x22, 0x0D, 0x23, 0x14, 0x17, 0x07, 0x24, 0x22, 0x25, 0x24, 0x26, 0x0C, 0x32, 0x09, 0x31, 0x06 ; 0x20

db 0x18, 0x03, 0x19, 0x12, 0x10, 0x17, 0x13, 0x01, 0x1F, 0x08, 0x14, 0x02, 0x16, 0x16, 0x2F, 0x11 ; 0x30

db 0x11, 0x0F, 0x2D, 0x15, 0x15, 0x0B, 0x2C, 0x26, 0x0C, 0x23, 0x34, 0x25, 0x35, 0x27, 0x27, 0x28 ; 0x40

db 0x28, 0x29, 0x82, 0x2A, 0x8D, 0x2B, 0x83, 0x2C, 0x89, 0x2D, 0x33, 0x2E, 0xB5, 0x2F, 0x39, 0x80 ; 0x50

db 0x1C, 0x81, 0x0E, 0x82, 0x01, 0x83, 0x3B, 0x84, 0x29, 0x30

; test only

; times 0x40 db 0x00,

get_qwerty_key_:           ; get a qwerty key character

_DUP_

.back:

call WaitToReceiveKey

in al, 0x60

cmp _TOS_, 0x1C     ; the Enter key scan code

jne .forward1

; add _TOS_, ( 89 - 0x1C ) ; convert the code for the Enter key to 89

mov _TOS_, 89

.forward1:

cmp _TOS_, 0x81     ; the Escape key scan code

jne .forward2

add _TOS_, ( 90 - 0x81 ) ; convert the code for the Escape key to 90

.forward2:

;    cmp _TOS_, 0x03     ; the Left Alt key scan code

;    jne .forward3

;    add _TOS_, 0x02 ; convert the code for the Left Alt key to a space key

; .forward3:

mov [ v_scanCode ], al

mov ecx, _TOS_              ; copy keycode into cl

and cl, 0x7F                ; filter out key-up bit 7

cmp cl, 0x2A                ; g?

jz .got_c_or_g

cmp cl, 0x36                ; c?

jnz .not_c_or_g

.got_c_or_g:

and al, 0x80                ; extract key-up bit

xor al, 0x80                ; complement it

mov [ v_qwerty_key ], _TOS_

jmp short .back

.not_c_or_g:

or al, al                   ; check if key-up

js .back                    ; if so, try again to get keydown event

and al, 0x7F                ; filter out key-up bit

or _TOS_, [ v_qwerty_key ]

mov edx, qwerty_key_map_table

mov ecx, 0x35

.back2:

cmp [edx], al

jz .forward

add edx, byte 0x02

loop .back2

xor _TOS_, _TOS_

ret

.forward:

mov al, [edx+0x01]

sub edx, qwerty_key_map_table

shr edx, 1

mov [ v_digin ], edx

cmp _TOS_, 59  ; F1 key

;    jnz .forward4

;    ; jmp dword [ _TOS_ * 4 + qwertyActionTable - 0x200 ]

;    xor dword [ current], ((setBase_decimal - $$) ^ (setBase_hex - $$))

;    call toggleBase

;.forward4:

ret

; *****************************************************************************

; keypad jump tables

; actions for the three editor state change keys : N spacebar AltGr

; *****************************************************************************

graph0:

dd nul0, nul0, nul0, alph0

db '  a ' ; _ _ a _ '  a ' ;

graph1:

dd word0, exit_, lj, alph

db 'x.a ' ;

alpha0:

dd nul0, nul0, number, star0

db ' 9* ' ;

alpha1:

dd word0, exit_, lj, graph

db 'x.* ' ;

numb0:  ; the number keypad before the '-' key has been pressed ???

dd nul0, minusSign, alphn, toggleBase

db '-af ' ; 0x23, 0x05, 0x0E, 0x00       ; - a f _ '-af ' ;

numb1:  ; the number keypad after the '-' key has been pressed ???

dd number0, minusSign, endn, toggleBase

db '-af ' ; 0x15, 0x25, 0x00, 0x00       ; x . _ _ 'x.  ' ;

; *****************************************************************************

; Shannon-Fano compression

; *****************************************************************************

bits_:

db 0x1C

lj0:

mov cl, [ bits_ ]

add cl, 0x04

shl dword [ esi ],cl

ret

lj:

call lj0

_DROP_

ret

full:

call lj0

inc dword [ v_words ]

mov byte [ bits_ ], 0x1C

sub [ bits_ ], ch

mov _TOS_, edx

_DUP_

ret

pack0:

add _TOS_, byte  0x50

mov cl, 0x07

jmp short pack1

pack_:

cmp al, 0x10

jnc pack0

mov cl, 0x04

test al, 0x08

jz pack1

inc ecx

xor al, 0x18

pack1:

mov edx, _TOS_

mov ch,cl

.back:

cmp [ bits_ ], cl

jnc .forward

shr al,1

jc full

dec cl

jmp short .back

.forward:

shl dword [ esi ],cl

xor [ esi ], _TOS_

sub [ bits_ ], cl

ret

exit_:      ; exit to the quit loop

call right

mov _TOS_, [ v_words ]

lea esi, [ esi + (_TOS_ * 4 ) ]

_DROP_

jmp quit_

word_:

call right

mov dword [ v_words ], 0x01

mov dword [ chars ], 0x01

_DUP_

mov dword [ esi ], 0x00

mov byte [ bits_ ], 0x1C

word1:

call letter

jns .forward

mov edx, [ shiftAction ]

jmp dword [edx+_TOS_*4]

.forward:

test al,al

jz word0

_DUP_

call echo_

mov al, [ _TOS_ + ASCII_to_SF_table ]

call pack_

inc dword [ chars ]

word0:

_DROP_

call get_key_

jmp short word1

; *****************************************************************************

; number display

; *****************************************************************************

digitTable:                         ; convert a keypad key value to a number

times 0x30 db 0x00                                       ;

db 0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07, 0x08, 0x09   ; '0123456789' ; 0x30 to 0x39

times 0x27 db 0x00

db 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F                           ; 'abcdef'

times 0x34 db 0x00

ASCII_to_SF_table:                  ; to convert ASCII value to ShannonFano number

;  0x00, 0x01, 0x02, 0x03, 0x04, 0x05, 0x06, 0x07,  0x08, 0x09, 0x0A, 0x0B, 0x0C, 0x0D, 0x0E, 0x0F

db 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,  0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 ; 0x00

db 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00,  0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 ; 0x10

db 0x00, 0x2A, 0x00, 0x00, 0x00, 0x00, 0x00, 0x59,  0x00, 0x00, 0x2D, 0x2B, 0x2E, 0x23, 0x25, 0x27 ; 0x20

db 0x18, 0x19, 0x1A, 0x1B, 0x1C, 0x1D, 0x1E, 0x1F,  0x20, 0x21, 0x29, 0x28, 0x00, 0x00, 0x00, 0x2F ; 0x30

db 0x2C, 0x35, 0x43, 0x3A, 0x40, 0x34, 0x3E, 0x3D,  0x44, 0x37, 0x52, 0x54, 0x3C, 0x39, 0x36, 0x33 ; 0x40

db 0x42, 0x47, 0x31, 0x38, 0x32, 0x46, 0x41, 0x3F,  0x45, 0x3B, 0x56, 0x00, 0x00, 0x00, 0x00, 0x53 ; 0x50

db 0x00, 0x05, 0x13, 0x0A, 0x10, 0x04, 0x0E, 0x0D,  0x14, 0x07, 0x22, 0x24, 0x0C, 0x09, 0x06, 0x03 ; 0x60

db 0x12, 0x17, 0x01, 0x08, 0x02, 0x16, 0x11, 0x0F,  0x15, 0x0B, 0x26, 0x00, 0x00, 0x00, 0x00, 0x00 ; 0x70

v_sign:                             ; set to 0xXX when the '-'key is pressed on the keypad

db 0x00

minusSign:

; not byte [ v_sign ]

mov byte [ v_sign ], '-'

jmp short number2

number0:

_DROP_

jmp short number3

number:

call [ setCurrentBase ]

mov byte [ v_sign ] , 0x00

xor _TOS_, _TOS_

number3:

call get_key_

call letter

jns .forward

mov edx, [ shiftAction ]

jmp dword [edx+_TOS_*4]

.forward:

test al,al

jz number0

mov al, [ _TOS_ + digitTable ]

test byte [ v_sign ], '-'

jz .forward2

neg _TOS_

.forward2:

mov edx, [ esi ]

imul edx, [ base ]

add edx, _TOS_

mov [ esi ], edx

number2:

_DROP_

mov dword [ shiftAction ], numb1

jmp short number3

endn:

_DROP_

call [ anumber]

jmp quit_

setBase_decimal:                     ; set the system base to decimal

mov dword [ base ], 0x0A

mov dword [ shiftAction ], numb0

mov dword [ currentKeypadIcons ], ( decimalKeypad - 4 )

ret

setBase_hex:                         ; set the system base to hexadecimal

mov dword [ base ], 0x10

mov dword [ shiftAction ], numb0

mov dword [ currentKeypadIcons ], ( hexadecimalKeypad - 4 )

ret

toggleBase0:

; the 'xor's below change the content of 'setCurrentBase_base' and the keypad icon

xor dword [ setCurrentBase], ((setBase_decimal - $$) ^ (setBase_hex - $$))

xor byte [ numb0 + 18 ], ( 0x39 ^ 0x66 ) ;  0x39 = '9' , 0x66 = 'f' toggle '9' and 'f' on keypad display line

call [ setCurrentBase ]

ret

toggleBase:

call toggleBase0

jmp dword number0

; *****************************************************************************

; text entry

; *****************************************************************************

exitn_:

_DROP_

_DROP_

jmp quit_

nul0:

_DROP_

jmp short quit2

clearHintChar:

push _TOS_

xor _TOS_, _TOS_

mov byte [ v_hintChar ], 0x00   ; clear the hint character

pop _TOS_

ret

quit_:                               ; get a word from keypad and interpret it

mov dword [ shiftAction ], alpha0

lea edi, [ alphaKeypad - 4]

quit1:

mov [ dword currentKeypadIcons ], edi

quit2:

test dword [ x_qwerty ], 0xFFFFFFFF

jz .forward

jmp dword [ x_qwerty ]          ; jump to the address in x_qwerty if it is non-zero

.forward:

call get_key_                   ; calls pause_ while waiting for a character

cmp al, 0x04                    ;

jns .forward2

mov edx, [ shiftAction ]

jmp dword [ edx + _TOS_ * 4 ]   ; alpha0 jump table element

.forward2:

add dword [ shiftAction ], byte +0x14

call word_

call [ aword ]

jmp short quit_                ; endless loop

alphn:

_DROP_

alph0:

mov dword [ shiftAction ], alpha0

lea edi, [ alphaKeypad - 4 ]

jmp short Xstar0

star0:

mov dword [ shiftAction ], graph0

lea edi, [ ( graphicsKeypad - 4 ) ]

Xstar0:

_DROP_

jmp short quit1

alph:

mov dword [ shiftAction ], alpha1

lea edi, [ alphaKeypad - 4]

jmp short Xgraph

graph:

mov dword [ shiftAction ], graph1

lea edi, [ ( graphicsKeypad - 4 ) ]

Xgraph:

mov [ currentKeypadIcons ], edi

jmp dword word0

; Note: defining drawTheCursor as a sub-routine and calling it produces a strange bug :

; moving left 24 times using the left arrow key, from the end of the block, crashes the editor.

; I suspect that the use of the stack to store (and later replace) deleted tokens gets confused

; if a call to drawTheCursor happens occasionally...

; This code should be re-worked. It is just too delicate...

; drawTheCursor:

;     mov [ v_cad ], edi

;     push _SCRATCH_

;     mov _SCRATCH_, [ v_10000_iconw ]

;     sub dword [ v_gr_xy ], _SCRATCH_   ; move one icon's worth of horizontal pixels to the left

;     _DUP_

;     mov _SCRATCH_, [ v_foregroundColour ]   ; save the current colour

;     mov _TOS_, colour_PacMan        ; for the "PacMan" cursor

;     call set_color_

;     mov _TOS_, 0x04                 ; display the "PacMan" cursor

;     mov cx, [ v_gr_x ]

;     cmp cx, [ v_rightMargin ]

;     js .forward5

;     ; the cursor is too far to the right on the screen

;     call emit_

;     mov [ v_10000_iconw ], _SCRATCH_

;     sub dword [ v_gr_xy ], _SCRATCH_  ; move one icon's worth of horizontal pixels to the left

;     jmp .forward6

; .forward5:

;     ; the cursor can be drawn

;     call emit_

; .forward6:

;     call doShowASCII                ; optionally show the ASCII entry field

;     mov dword [ v_foregroundColour ], _SCRATCH_     ; restore the current colour

;     pop _SCRATCH_

;     ret

; *****************************************************************************

; Shannon-Fano decompression and display

; *****************************************************************************

unpack:     ; ( token -- token' nextCharacter )

_DUP_   ; copy TOS to our data stack SOS

test _TOS_, _TOS_

js .forward

shl dword [ esi ], 0x04

rol _TOS_, 0x04

and _TOS_, byte 0x07

ret

.forward:

shl _TOS_,1

js .forward2

shl dword [ esi ], 0x05

rol _TOS_, 0x04

and _TOS_, byte 0x07

xor al, 0x08

ret

.forward2:

shl dword [ esi ], 0x07

rol _TOS_, 0x06

and _TOS_, byte 0x3F

sub al, 0x10

ret

; show the PacMan-like cursor

show_cursor:  ; ( a cursor -- a' )  edi  contains pointer to current address to display

_DUP_

inc dword [ esi ]

cmp [ v_curs ], edi

jnz .forward                    ; address to display = cursor address?

mov [ v_curs ], _TOS_           ; yes,

.forward:

cmp _TOS_, [ v_curs ]           ; time to draw the cursor?

jz .forward2

jns .forward4                   ; time to draw the cursor?

mov [ v_pcad ], edi             ; no, so exit

.forward4:

_DROP_

ret                             ; exit here

.forward2:

; call drawTheCursor ; Note: do not do this!!! See notes for drawTheCursor:

mov [ v_cad ], edi

push _SCRATCH_

mov _SCRATCH_, [ v_10000_iconw ]

sub dword [ v_gr_xy ], _SCRATCH_  ; move the graphic position one icon's worth of horizontal pixels to the left

_DUP_

mov _SCRATCH_, [ v_foregroundColour ]   ; save the current colour

mov _TOS_, colour_PacMan

call set_color_

mov _TOS_, 0x04   ; display the "PacMan" cursor

mov cx, [ v_gr_x ]

cmp cx, [ v_rightMargin ]

js .forward5

call emit_

mov _SCRATCH_, [ v_10000_iconw ]

sub dword [ v_gr_xy ], _SCRATCH_  ; move one icon's worth of horizontal pixels to the left

jmp .forward6

.forward5:

call emit_

.forward6:

mov dword [ v_foregroundColour ], _SCRATCH_     ; restore the current colour

pop _SCRATCH_

ret

ret

; *****************************************************************************

; Conventional Forth display (does not require colours) - colour-blind mode

; *****************************************************************************

currentState:   ; the current token colour

dd 0

lastState:      ; the last token colour

dd 0

txt0:

call white

EMIT_IMM('(')

call space_

ret

txt1:

call white

EMIT_IMM(')')

call space_

ret

imm0:

call yellow

EMIT_IMM('[')

call space_

ret

imm1:

call yellow

EMIT_IMM(']')

call space_

ret

mvar0:

call yellow

EMIT_IMM('[')

call space_

EMIT_IMM('m')

EMIT_IMM('v')

EMIT_IMM('a')

EMIT_IMM('r')

call space_

ret

mvar1:

call yellow

EMIT_IMM(']')

call space_

ret

; unfortunately we need to display the ':' after the CR, so must do this in  redWord , not here

; colon0:

;     call red

;     EMIT_IMM(':')

;     call space_

;     ret

;

;     dd nul, imm0, nul, colon0, nul, nul, nul, nul, nul, txt0, nul, nul, mvar0, nul, nul, nul

txts:

db 0, 1, 1, 3, 4, 5, 6, 7, 1, 9, 9, 9, 12, 13, 14, 15

tx:     ; ( c -- c )   \ return the value in the given offset in  txts

and _TOS_, 0xFF

mov _TOS_, [ _TOS_ + txts ]

and _TOS_, 0xFF

ret

newActions:

dd nul, imm0, nul, nul, nul, nul, nul, nul, nul, txt0, nul, nul, mvar0, nul, nul, nul

dotNew:     ; ( state -- )

call [ ( _TOS_ * 4 ) + newActions ]

ret

oldActions:

dd nul, imm1, nul, nul, nul, nul, nul, nul, nul, txt1, nul, nul, mvar1, nul, nul, nul

dotOld:     ; ( state -- )

call [ ( _TOS_ * 4 ) + oldActions ]

ret

colourBlindAction:  ; ( state -- state )    \ perform the required action on change of state

push _SCRATCH_

_DUP_

call tx

cmp _TOS_, 0x00

jz .end                             ; no action on extension tokens, value 0

mov _SCRATCH_, [ currentState ]

mov [ currentState ], _TOS_

cmp _SCRATCH_, [ currentState ]     ; compare the new state on TOS to the last one saved in currentState

jz .end                             ; exit if there has been no change of state

_DUP_

mov _TOS_, _SCRATCH_

call dotOld                         ;

mov _TOS_, [ currentState ]

call dotNew

_DROP_

cmp byte [ currentState ], 0x0000

jz .end

mov _SCRATCH_, [ currentState ]

mov [ lastState ], _SCRATCH_

.end:

_DROP_

pop _SCRATCH_

ret

; \ Block 70

; ( Colourblind Editor Display )

; #1 MagentaV currentState $01 MagentaV lastState

; : +txt white $6D emit space ;

; : -txt white $6E emit space ;

; : +imm yellow $58 emit space ;

; : -imm yellow $59 emit space ;

; : +mvar yellow $09 emit $11 emit $05 emit $01 emit space ;

; : txts string $03010100 , $07060504 , $09090901 , $0F0E0D0C , ( ; )

; : tx ( c-c ) $0F and txts + 1@ $0F and ;

; : .new currentState @ $0F and jump nul +imm nul nul nul nul nul nul nul +txt nul nul +mvar nul nul nul ;

; : .old lastState @ $0F and jump nul -imm nul nul nul nul nul nul nul -txt nul nul nul nul nul nul ;

; here

; : cb ( n-n ) #0 + 0if ; then tx

;    currentState @ swap dup currentState ! - drop if .old .new

;    currentState @ #0 + if dup lastState ! then then ;

; : cbs ( -- here ) #0 + $00 + cblind ! ;

; colourBlind:    ; ( state -- state )    \ vectored colorForth to display colourBlind extra characters ( e.g. ':' for red words )

;    call dword [ x_colourBlind ]

;    ret

; *****************************************************************************

; Show an ASCII editable entry field at the cursor

; *****************************************************************************

ShowASCIIAction:    ; ( -- )

;    call white

call space_

EMIT_IMM('U')

EMIT_IMM('U')

EMIT_IMM('U')

EMIT_IMM('U')

call space_

ret

ret

; *****************************************************************************

; *****************************************************************************

lowercase:   ; display a white text word in normal lower-case letters

call white

showSF_EDI_:  ; ( -- )   \ display a Shanon-Fano encoded token pointed to by  edi  in the current colour

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]   ; fetch the next token - drops through to showShannonFano

showShannonFano:    ; ( token -- ) \ display the Shannon-Fano encoded token on TOS

; ASCII / UTF8 support. If the first Shannon-Fano encoded letter is a 4 bit NULL,

; display the next 24 bits as three ASCII characters.

mov _SCRATCH_, _TOS_            ; save the token value

and _SCRATCH_, 0xF0000000

cmp _SCRATCH_, 0x00000000

jnz .forward

; display as three ASCII characters

mov _SCRATCH_, _TOS_

mov _TOS_, _SCRATCH_

shr _TOS_, 20

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

mov _TOS_, _SCRATCH_

shr _TOS_, 12

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

mov _TOS_, _SCRATCH_

shr _TOS_, 4

and _TOS_, 0x000000FF

jz .null_terminator

_DUP_

call emit_

; arrive here if an ASCII character is an ASCII NULL, or if all three have been emitted

.null_terminator:

call space_                 ; display a space character at the end of the word

_DROP_

ret

.forward:

; display as Shannon-Fano encoded token name

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )  ignore the token colour when displaying the letters

lowercasePrimitive:  ; ( token -- ) \ display the given Shanon-Fano encoded word in the current colour

call unpack

jz lowercasePrimitiveEnd

call emitSF_

jmp lowercasePrimitive

lowercasePrimitiveEnd:

call space_

_DROP_

_DROP_

ret

typeNumber32tok:     ; ( token -- ) \ display the given Shanon-Fano encoded word as a number in the current colour

_DROP_ ; call dotHex8_

mov dword [ lastTokenWasLiteral ], 0xFFFFFFF

ret

typeNumber32:     ; ( token -- ) \ display the given Shanon-Fano encoded word as a hex number in the current colour

call dotHex8_

mov dword [ lastTokenWasLiteral ], 0x00000000

ret

typeNumber27:     ; ( token -- ) \ display the given Shanon-Fano encoded word as a 27 bit hex number in the current colour

shr _TOS_, 5

call dotHex

ret

lastTokenWasLiteral:

dd 0x00

lastShannonFanoToken:

dd 0x00

magentaPrimitive:   ; ( token -- )

call showShannonFano

mov dword [ lastTokenWasLiteral ], 0xFFFFFFF

ret

displayOneShannonFanoActions:   ;    * = number

dd showShannonFano          ; 0     extension token, remove space from previous word, do not change the colour

dd showShannonFano          ; 1     yellow "immediate" word

dd typeNumber32tok          ; 2  *  yellow "immediate" 32 bit number in the following pre-parsed cell

dd showShannonFano          ; 3     red forth wordlist "colon" word

dd showShannonFano          ; 4     green compiled word

dd typeNumber32tok          ; 5  *  green compiled 32 bit number in the following pre-parsed cell

dd typeNumber27             ; 6  *  green compiled 27 bit number in the high bits of the token

dd showShannonFano          ; 7     cyan macro wordlist "colon" word

dd typeNumber27             ; 8  *  yellow "immediate" 27 bit number in the high bits of the token

dd showShannonFano          ; 9     white lower-case comment

dd camelcasePrimitive       ; A     first letter capital comment

dd uppercasePrimitive       ; B     white upper-case comment

dd magentaPrimitive         ; C     magenta variable

dd showShannonFano          ; D

dd showShannonFano          ; E     editor formatting commands

dd showShannonFano          ; F

times 0x20 db 0x55

testme:

dd 0x75240CFF ; 0xFF, 0x0C, 0x24, 0x75

dd 0x123456

ret

times 0x20 db 0x77

leave_:     ; terminate a  for ... next  loop

mov dword [ esp + 4 ], 0x01

ret

dotsf_:  ; ( token -- )   \ display the given Shannon-Fano encoded word in the token's colour

push edi

mov edx , _TOS_

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

_DUP_

mov edi, [ lastTokenWasLiteral ]

test edi, 0x00000000

jz .forward3

mov edx, 0

.forward3:

and edx, byte 0x0F

jnz .forward     ; do not change the colour if this is an extension token

; this is an extension token

mov edx, [ lastShannonFanoToken ]

; if the colour is Camelcase 0x0A, make it lowercase 0x09

; e.g. Interrupt would be shown as InterrUpt if the exension token is displayed with an initial Capital

mov _SCRATCH_, edx

and _SCRATCH_, 0x0F     ; just the colour

sub _SCRATCH_, 0x0A

jne .foward4

and edx, 0xFFFFFFF0     ; remove the colour

or edx, 0x00000009      ; make it lowercase

.foward4:

mov _SCRATCH_, [ v_10000_iconw ]

sub dword [ v_gr_xy ], _SCRATCH_   ; move iconw horizontal pixels back, to remove the space at the end of the last word

jmp .forward2

.forward:

; this is not an extension token

mov [ lastShannonFanoToken ], edx

.forward2:

push _TOS_

mov _TOS_, [ ( edx * 4 ) + actionColourTable ]

call set_color_

pop _TOS_

call [ ( edx * 4 ) + displayOneShannonFanoActions ]

pop edi

ret

redWord:     ; display a red word

mov cx, [ v_gr_x ]

cmp cx, [ v_leftMargin ]

jz .forward     ; do not do a  cr  if we are already at the left margin

mov cl, [ v_not_cr ]

cmp cl, 0

jnz .forward    ; do not do a  cr  if it has been disabled by a blue  -cr  token

call cr_

.forward:

mov byte [ v_not_cr ], 0

call setRed

cmp byte [ v_colourBlindMode ], 0x00

jz .forward2

test byte [ v_blk ], 0x01   ; do not display colour-blind characters in odd numbered shadow blocks

jnz .forward2

EMIT_IMM(':')   ; emit a ':' if in colour-blind mode

call space_

.forward2:

jmp showSF_EDI_

greenWord:     ; display a green word

call setGreen

jmp showSF_EDI_

cyanWord:     ; display a cyan word

call setCyan

jmp showSF_EDI_

yellowWord:     ; display a yellow word

call yellow

jmp showSF_EDI_

; Note : Camelcase tokens do not support ASCII output

camelcase:    ; display a white word with the first letter Capitalised

call white

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

camelcasePrimitive:

call unpack

add al, 0x30                    ; make the first character upper case

call emitSF_                    ; display it

camelcasePrimitive_2:               ; display the rest of the word

call unpack

jz lowercasePrimitiveEnd

call emitSF_

jmp camelcasePrimitive_2

; Note : UPPERCASE tokens do not support ASCII output

uppercase:   ; display a white word with all letters CAPITALISED

call white

_DUP_

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

uppercasePrimitive:

call unpack

jz lowercasePrimitiveEnd

add al, 0x30

call emitSF_

jmp uppercasePrimitive

extension:  ; display an extension token, do not change the colour

mov _SCRATCH_, [ v_10000_iconw ]

sub dword [ v_gr_xy ], _SCRATCH_   ; move iconw horizontal pixels back, to remove the space at the end of the last word

test dword [ ( edi * 4 ) - 0x04 ], 0xFFFFFFF0

jnz showSF_EDI_

dec edi

mov [ v_lcad ], edi

call space_

call show_cursor                ; show the PacMan-like cursor

pop edx                         ; EXIT from calling word

_DROP_                          ; the ret below will return to the word that called  extension

ret                             ; so it looks like it never happened

greenShortNumber:    ; display the green compiled 27 bit number in the high bits of the token

mov edx, [ ( edi * 4 ) - 0x04 ]

sar edx, 0x05

jmp short greenNumber1

magentaVariable:    ; display a magenta variable using the 32 bit number in the following pre-parsed cell

mov dword [ x_numberDisplay ], dotDecimal

cmp dword [ base ], byte 0x0A   ; check the current BASE value ( 10 or 16 for decimal or hex)

jz .forward

mov dword [ x_numberDisplay ], dotHex

.forward:

call setMagenta

call showSF_EDI_              ; display the name of the variable

mov edx, [ ( edi * 4 ) + 0x00 ]    ; load the value of the variable from the pre-parsed source

inc edi                 ; step over the variable value in the pre-parsed source

call setMagentaData

jmp short displayNumber

greenNumber:                ; display the value of a hexadecimal/decimal number in green

mov edx, [ ( edi * 4 ) + 0x00 ]    ; load the value of the variable from the pre-parsed source

inc edi                 ; step over the variable value in the pre-parsed source

greenNumber1:

call green

jmp short displayNumber

yellowShortNumber:

mov edx, [ ( edi * 4 ) - 0x04 ] ; load the value of the number from the current token in the pre-parsed source

sar edx, 0x05                   ; remove the token colour bits

jmp short yellowNumber1

yellowNumber:     ; ( -- ) display a number word, constant value following in the pre-parsed source

mov edx, [ ( edi * 4 ) + 0x00 ]    ; load the value of the number from the pre-parsed source

inc edi                 ; step over the number value in the pre-parsed source

yellowNumber1:    ; ( -- ) display a yellow number word

call yellow

displayNumber: ; ( rgb -- )   display the number in edx  with the given colour, using the base implied in  x_numberDisplay

_DUP_

mov _TOS_, edx

; jmp qdot

jmp dword [ x_numberDisplay ]

; *****************************************************************************

; Blue words - formatting the editor display

; *****************************************************************************

get_x:  ; ( -- c )  \ return the current x character position

push edx

_DUP_

xor _TOS_, _TOS_

mov ax, word [ v_gr_x ]

xor edx, edx                    ; clear high 32 bits of dividend

div dword [ v_iconw ]           ; EDX:EAX divided by the icon width , EAX now contains the current character position, EDX the remainder

pop edx

ret

set_x:  ; ( c -- )  \ set the current x character position

push edx

xor edx, edx

mul dword [ v_iconw ]

mov word [ v_gr_x ], ax

pop edx

_DROP_

ret

tab_n:    ; ( n -- )    \ align to the next n character column

mov ecx, _TOS_

pusha

call get_x

xor edx, edx                    ; clear high 32 bits of dividend

mov _SCRATCH_, ecx

div _SCRATCH_

mul _SCRATCH_

add _TOS_, ecx

call set_x

popa

_DROP_

ret

tab3:       ; ( -- )

_DUP_

mov _TOS_, 0x03

call tab_n

ret

tab4:       ; ( -- )

_DUP_

mov _TOS_, 0x04

call tab_n

ret

tab5:       ; ( -- )

_DUP_

mov _TOS_, 0x05

call tab_n

ret

tab6:       ; ( -- )

_DUP_

mov _TOS_, 0x06

call tab_n

ret

tab7:       ; ( -- )

_DUP_

mov _TOS_, 0x07

call tab_n

ret

tab8:       ; ( -- )

_DUP_

mov _TOS_, 0x08

call tab_n

ret

%define TAB_SIZE  24

tab:        ; ( -- )

_DUP_

mov _TOS_, TAB_SIZE

call tab_n

ret

; tab:    ; ( -- )    \ align to the next n character column

;     pusha

;     call get_x

;     xor edx, edx                    ; clear high 32 bits of dividend

;     mov _SCRATCH_, TAB_SIZE

;     div _SCRATCH_

;     mul _SCRATCH_

;     add _TOS_, TAB_SIZE

;     call set_x

;     popa

;     ret

; tab3:

;     pusha

;     call get_x

;     xor edx, edx                    ; clear high 32 bits of dividend

;     mov _SCRATCH_, 0x03

;     div _SCRATCH_

;     mul _SCRATCH_

;     add _TOS_, 0x03

;     call set_x

;     popa

;     ret

;

; tab6:

;     pusha

;     call get_x

;     xor edx, edx                    ; clear high 32 bits of dividend

;     mov _SCRATCH_, 0x06

;     div _SCRATCH_

;     mul _SCRATCH_

;     add _TOS_, 0x06

;     call set_x

;     popa

;     ret

;

; tab7:

;     pusha

;     call get_x

;     xor edx, edx                    ; clear high 32 bits of dividend

;     mov _SCRATCH_, 0x07

;     div _SCRATCH_

;     mul _SCRATCH_

;     add _TOS_, 0x07

;     call set_x

;     popa

;     ret

;

; tab8:

;     pusha

;     call get_x

;     xor edx, edx                    ; clear high 32 bits of dividend

;     mov _SCRATCH_, 0x08

;     div _SCRATCH_

;     mul _SCRATCH_

;     add _TOS_, 0x08

;     call set_x

;     popa

;     ret

br:         ; ( -- )

call cr_

call cr_

ret

not_cr:     ; ( -- )

not byte [ v_not_cr ]

ret

not_tab:    ; ( -- )

call not_cr

call tab

ret

cr_plus:    ; ( -- )

call cr_

call space_

call space_

call space_

ret

four_spaces:       ; ( -- )

call space_

three_spaces:      ; ( -- )

call space_

two_spaces:        ; ( -- )

call space_

call space_

ret

BlueNames:   ; name   routine      comment

dd 0x9080000E  ; cr     cr_          move to the next line

dd 0xE64B8C0E  ; -tab   not_tab      prevent the next CR

dd 0x25C6000E  ; tab    tab          align to next TAB_SIZE space column

dd 0xC620000E  ; br     br           cr cr

dd 0xE721000E  ; -cr    not_cr       prevent the next CR

dd 0x90FB000E  ; cr+    cr_plus      cr and 3 spaces

dd 0x25C7AC0E  ; tab3   tab3         align to next 3 space column

dd 0x25C7B00E  ; tab4   tab4         align to next 4 space column

dd 0x25C7B40E  ; tab5   tab5         align to next 5 space column

dd 0x25C7B80E  ; tab6   tab6         align to next 6 space column

dd 0x25C7BC0E  ; tab7   tab7         align to next 7 space column

dd 0x25C7C00E  ; tab8   tab8         align to next 8 space column

dd 0xEA00000E  ; .      space_

dd 0xEBD4000E  ; ..     two_spaces

dd 0xEBD7A80E  ; ...    three_spaces

dd 0xEBD7AF5E  ; ....   four_spaces

BlueJumpTableROM:  ; name_SF     name    comment

dd cr_          ; 0x9080000E  cr      move to the next line

dd not_tab      ; 0xE64B8C0E  -tab    prevent the next CR

dd tab          ; 0x25C6000E  tab     align to next TAB_SIZE space column

dd br           ; 0xC620000E  br      cr cr

dd not_cr       ; 0xE721000E  -cr     prevent the next CR

dd cr_plus      ; 0x90FB000E  cr+     cr and 3 spaces

dd tab3         ; 0x25C7AC0E  tab3    align to next 3 space column

dd tab4         ; 0x25C7B00E  tab4    align to next 4 space column

dd tab5         ; 0x25C7B40E  tab5    align to next 5 space column

dd tab6         ; 0x25C7B80E  tab6    align to next 6 space column

dd tab7         ; 0x25C7BC0E  tab7    align to next 7 space column

dd tab8         ; 0x25C7C00E  tab8    align to next 8 space column

dd space_       ; 0xEA00000E  .       space_

dd two_spaces   ; 0xEBD4000E  ..      two_spaces

dd three_spaces ; 0xEBD7A80E  ...     three_spaces

dd four_spaces  ; 0xEBD7AF5E  ....    four_spaces

BlueJumpTableROM_end:

find_Blue_word_:   ; ( sf -- index )   \ ecx = index ; find the Shannon-Fano word sf in the Blue wordlist, return its index in ecx

push edi

mov ecx, 16   ; count of Blue wordlist words

lea edi, [ ( ecx * 4 ) + BlueNames - 4 ]   ; set edi to the top of the Blue name table

std                    ; scan backwards

repne scasd            ; find the 32 bit Shanon-Fano encoded name, compare eax with doubleword at es:edi and set status flags.

cld                    ; reset the direction flag

mov _TOS_, ecx

pop edi

ret

blueWord:   ; ( -- )    \ format the editor display screen using certain blue tokens. ToDo: This should ba a table...

_DUP_

mov al, [ v_seeb ]

cmp al, 0

mov _TOS_, [ ( edi * 4 ) - 0x04 ]   ; fetch the next token - then calls showShannonFano

jz .forward

call setBlue

_DUP_

call showShannonFano

.forward:

mov _TOS_, [ ( edi * 4 ) - 0x04 ]

call find_Blue_word_             ; returns the index TOS

call [ ( _TOS_ * 4 ) + BlueJumpTableROM ]

_DROP_

ret

; silverWord:   ; ( -- )    ; ToDo: find out what this is for (GreenArrays gray token?)

;     mov edx, [ ( edi * 4 ) - 0x04 ] ; load the value of the action from the current token in the pre-parsed source

;     sar edx, 0x05                   ; remove the token colour bits

;     _DUP_

;     mov _TOS_, colour_white

;     cmp dword [ x_numberDisplay ], dotDecimal

;     jz .forward

;     mov _TOS_, colour_silver

; .forward:

;    jmp short displayNumber

;    ret

silverWord:     ; display a silver word

call setSilver

jmp showSF_EDI_

displayShannonFanoActions:  ;    * = number

dd extension            ; 0     extension token, remove space from previous word, do not change the colour

dd yellowWord           ; 1     yellow "immediate" word

dd yellowNumber         ; 2  *  yellow "immediate" 32 bit number in the following pre-parsed cell

dd redWord              ; 3     red forth wordlist "colon" word

dd greenWord            ; 4     green compiled word

dd greenNumber          ; 5  *  green compiled 32 bit number in the following pre-parsed cell

dd greenShortNumber     ; 6  *  green compiled 27 bit number in the high bits of the token

dd cyanWord             ; 7     cyan macro wordlist "colon" word

dd yellowShortNumber    ; 8  *  yellow "immediate" 27 bit number in the high bits of the token

dd lowercase            ; 9     white lower-case comment

dd camelcase            ; A     first letter capital comment

dd uppercase            ; B     white upper-case comment

dd magentaVariable      ; C     magenta variable

dd silverWord           ; D

dd blueWord             ; E     editor formatting commands

dd nul                  ; F

v_lineOffset:

dd 1    ; the top line of the display

doColourBlind:  ; ( state -- )   \ add conventional Forth punctuation based on the new and last states

cmp byte [ v_colourBlindMode ], 0x00

jz .forward3

test byte [ v_blk ], 0x01       ; do not display colour-blind characters in odd numbered shadow blocks

jnz .forward3

call dword colourBlindAction    ; pass the new state to colourBlind so that extra characters can be added to the display

.forward3:

_DROP_

ret

doShowASCII:    ;

cmp byte [ v_show_ASCII ], 0x00

jz .forward4

call dword ShowASCIIAction    ; pass the new state to colourBlind so that extra characters can be added to the display

.forward4:

ret

plusList:   ; ( -- ) display the current colorForth block

_DUP_

xor _TOS_, _TOS_                        ; set Top Of Stack to 0

mov [ currentState ], _TOS_             ; initialise the colour-blind state machine

mov [ lastState ], _TOS_                ; initialise the colour-blind state machine

mov [ v_display_token_number ], _TOS_   ; initialise the displayed token nunber

_DROP_

call setupText_                 ; setup the clip window for this display

_DUP_

mov _TOS_, [ v_lcad ]

mov [ v_cad ], _TOS_

mov _TOS_, [ v_blk ]                    ; get the current block number to be edited

call blockToCellAddress                 ; add the RELOCATED block number offset and convert to cell address

mov edi, _TOS_

xor _TOS_, _TOS_

add edi, [ v_lineOffset ]

mov [ v_pcad ], edi

.back:

mov edx, dword [ ( edi * 4 ) + 0x00 ]   ; edi is the display pointer and is a cell address

call show_cursor                        ; show the PacMan-like cursor

inc edi

inc word [ v_display_token_number ]     ; count up the number of tokens displayed

cmp word [ v_display_token_number ], 0xE0   ; save the last 128 bytes for version information

jne .forward4

call setBlue

EMIT_IMM('<')   ; warm the user that we have hit the limit of the block display "<<"

EMIT_IMM('<')

ret     ; we have displayed enough tokens now

.forward4:

; adjust the number base according to bit 5 of the token value, only used by number display words

; this section of code displays numbers in the base that they were defined as, not according to the current base

mov dword [ x_numberDisplay ], dotDecimal   ; set the display base to decimal

test dl, 0x10

jz .forward2

mov dword [ x_numberDisplay ], dotHex       ; set the display base to hexadecimal (overwrites dotDecimal that was just set)

.forward2:

and edx, byte 0x0F

_DUP_

mov _TOS_, edx

call doColourBlind

call [ ( edx * 4 ) + displayShannonFanoActions ]

jmp short .back

refresh:                            ; refresh the editor display

call show                       ; set the screen task to execute the code following (in a repeating loop) :

call page_                      ; clear the screen

call displayBlockNumber         ; display the current block number on the screen, top right corner

call plusList                   ; list the contents of the block

_DUP_

mov _TOS_, 0x0F

call doColourBlind             ; display the final colour-blind punctuation, set up for next call of plusList

jmp dword displayTheKeypad_

align 4, db 0   ; fill the gap with 0's

actionColourTable:          ;    * = number

dd colour_orange        ; 0     extension token, remove space from previous word, do not change the colour

dd colour_yellow        ; 1     yellow "immediate" word

dd colour_yellow        ; 2  *  yellow "immediate" 32 bit number in the following pre-parsed cell

dd colour_red           ; 3     red forth wordlist "colon" word

dd colour_green         ; 4     green compiled cf2022

dd colour_green         ; 5  *  green compiled 32 bit number in the following pre-parsed cell

dd colour_green         ; 6  *  green compiled 27 bit number in the high bits of the token

dd colour_cyan          ; 7     cyan macro wordlist "colon" word

dd colour_yellow        ; 8  *  yellow "immediate" 27 bit number in the high bits of the token

dd colour_white         ; 9     white lower-case comment

dd colour_white         ; A     first letter capital comment

dd colour_white         ; B     white upper-case comment

dd colour_magenta       ; C     magenta variable

dd colour_silver        ; D

dd colour_blue          ; E     editor formatting commands

dd colour_black         ; F

vector:

dd 0  ; pointer to call table for keypad ( see keypd )

action:

db 1

align 4, db 0   ; fill the gap with 0's

cursorLeft:     ; ( -- )

dec dword [ v_curs ]

jns .forward

inc dword [ v_curs ]

.forward:

ret

limitToEndOfBlock:

call countTokens

cmp _TOS_, dword [ v_curs ]

jns .forward

mov dword [ v_curs ], _TOS_

.forward:

_DROP_

ret

cursorRight:

inc dword [ v_curs ]

call limitToEndOfBlock

ret

countAllTokens:     ; ( -- x ) \ counts red and magenta tokens and all tokens in the current block

_DUP_

xor _TOS_, _TOS_

mov dword [ v_numberOfMagentas ], _TOS_

mov dword [ v_numberOfRedAndMagentas ], _TOS_   ; count up Red and Magenta tokens

mov dword [ v_numberOfTokens ], _TOS_           ; count all tokens

mov dword [ v_numberOfBigConstants ], _TOS_     ; count of 32 bit literal tokens

mov ecx, 0x00100      ; 256 x 4 byte cells = 1 block

.loop:

_DUP_

mov _TOS_, [ v_numberOfTokens ]

call nth_to_token

mov _SCRATCH_, _TOS_

_DROP_

cmp _SCRATCH_, 0x00

je .forward         ; exit if the token value is 0, means end of block

inc dword [ v_numberOfTokens ]

and _SCRATCH_, 0x0F     ; look at the token type

cmp _SCRATCH_, 0x03     ; red token

jne .forwardRed

inc dword [ v_numberOfRedAndMagentas ]

.forwardRed:

cmp _SCRATCH_, 0x0C     ; magenta token

jne .forwardMagenta

inc dword [ v_numberOfRedAndMagentas ]

inc dword [ v_numberOfMagentas ]    ; correction for magenta variables

inc dword [ v_numberOfTokens ]      ; step over the Magenta variable data cell

.forwardMagenta:

cmp _SCRATCH_, 0x02     ; yellow 32 bit literal

jne .forwardBig

inc dword [ v_numberOfBigConstants ]    ; correction for literal constants

inc dword [ v_numberOfTokens ]          ; step over the data cell

.forwardBig:

cmp _SCRATCH_, 0x05     ; green 32 bit literal

jne .forwardBig2

inc dword [ v_numberOfBigConstants ]    ; correction for literal constants

inc dword [ v_numberOfTokens ]          ; step over the data cell

.forwardBig2:

loop .loop

.forward:               ; found the end of the block

;    mov _TOS_, dword [ v_numberOfRedAndMagentas ]

ret

countRedAndMagentaTokens:     ; ( -- n ) \ counts red and magenta tokens in the current block

call countAllTokens

mov _TOS_, dword [ v_numberOfRedAndMagentas ]

ret

countTokens:     ; ( -- n ) \ counts all tokens up to the end of the current block

call countAllTokens

mov _TOS_, dword [ v_numberOfTokens ]

sub _TOS_, dword [ v_numberOfMagentas ]

sub _TOS_, dword [ v_numberOfBigConstants ]

and _TOS_, 0x00003FF   ; limit the maximum numer of tokens, just in case

ret

; *****************************************************************************

cursorDownToNth:     ; ( -- ) \ step down to after the v_cursLine'th red or magenta token

_DUP_

xor _TOS_, _TOS_

mov dword [ v_numberOfMagentas ], _TOS_

mov dword [ v_curs ], _TOS_

mov dword [ v_numberOfBigConstants ], _TOS_

mov dword _TOS_, [ v_cursLine ]

mov dword [ v_curs_number_down ], _TOS_

mov ecx, 0x00100      ; 256 x 4 byte cells = 1 block

.loop:

cmp dword [ v_curs_number_down ], 0x00  ; test for zero

je .forward     ; jump to the end if  v_curs_number_down  reaches zero

_DUP_

mov _TOS_, [ v_curs ]

call nth_to_token

mov _SCRATCH_, _TOS_

_DROP_

cmp _SCRATCH_, 0x00

je .endOfBlock         ; exit if the token value is 0, means end of block

inc dword [ v_curs ]

and _SCRATCH_, 0x0F     ; look at the token type

cmp _SCRATCH_, 0x03     ; red token

jne .forwardRed

dec dword [ v_curs_number_down ]

.forwardRed:

;   cmp _SCRATCH_, 0x0E     ; blue token

;   jne .forwardBlue

;       dec dword [ v_curs_number_down ]

;   .forwardBlue:

cmp _SCRATCH_, 0x0C     ; magenta token

jne .forwardMagenta

dec dword [ v_curs_number_down ]

inc dword [ v_numberOfMagentas ]    ; correction for magenta variables

inc dword [ v_curs ]                ; step over the Magenta variable data cell

.forwardMagenta:

cmp _SCRATCH_, 0x02     ; yellow 32 bit literal

je .forwardBig

cmp _SCRATCH_, 0x05     ; green 32 bit literal

jne .forwardBig2

.forwardBig:

inc dword [ v_numberOfBigConstants ]    ; correction for literal constants

inc dword [ v_curs ]                    ; step over the data cell

.forwardBig2:

loop .loop

.forward:               ; found the right number of red or magenta tokens, so exit

mov _SCRATCH_, dword [ v_numberOfMagentas ]

add _SCRATCH_, dword [ v_numberOfBigConstants ]

sub dword [ v_curs ], _SCRATCH_  ; the correction for magenta variables

.endOfBlock:

call limitToEndOfBlock

_DROP_

ret

cursorUp:     ; ( -- ) \ step down to after the next red token, or after 0x16 steps, or until the end of the block

dec dword [ v_cursLine ]

jnz .forward

mov dword [ v_cursLine ], 0x00

.forward:

;    mov dword [ v_cursLine ], 0x03

call cursorDownToNth

ret

cursorDown:     ; ( -- ) \ step down to after the next red token, or after 0x16 steps, or until the end of the block

inc dword [ v_cursLine ]

call countRedAndMagentaTokens

inc dword _TOS_     ; add one so that we can go past the last token to the end of the block

cmp dword [ v_cursLine ], _TOS_

js .forward

mov dword [ v_cursLine ], _TOS_

.forward:

_DROP_

;   mov dword [ v_cursLine ], 0x02

call cursorDownToNth

ret

cursorEnd:     ; ( -- )

call countRedAndMagentaTokens

inc dword _TOS_     ; add one so that we can go past the last token to the end of the block

mov dword [ v_cursLine ], _TOS_

_DROP_

call cursorDownToNth

call limitToEndOfBlock

ret

cursorHome:     ; ( -- )

xor _SCRATCH_, _SCRATCH_

mov dword [ v_numberOfMagentas ], _SCRATCH_

mov dword [ v_curs ], _SCRATCH_                 ; the graphics cursor for drawing the block

mov dword [ v_lineOffset ], _SCRATCH_           ; the cursor position to start drawing the block

mov dword [ v_lineOffsetTablePtr ], _SCRATCH_   ; a pointer to the cursor for each line in the display

mov dword [ v_numberOfMagentas ], _SCRATCH_     ; count of Magenta variables displayed so far in the edited block

mov dword [ v_cursLine ], _SCRATCH_

ret

nextBlock:     ; ( -- )

add dword [ v_blk ], byte 0x02

call lineOffsetZero

ret

previousBlock:

cmp dword [ v_blk ], byte ( START_BLOCK_NUMBER + 2 )

js .forward

sub dword [ v_blk ], byte 0x02

.forward:

call lineOffsetZero

ret

otherBlock:

call swap_with_other_

ret

tog_show_ASCII:

not byte [ v_show_ASCII ]

ret

shadow:     ; alternate between source and shadow blocks

xor dword [ v_blk ], byte 0x01

ret

insert0:    ; ( ... -- )

mov ecx, [ v_lcad ]

add ecx, [ v_words ]

xor ecx, [ v_lcad ]

and ecx, 0xFFFFFF00

jz insert1

mov ecx, [ v_words ]

.back:

_DROP_

loop .back

ret

insert1:

push esi

mov esi, [ v_lcad ]

mov ecx, esi

dec esi

mov edi, esi

add edi, [ v_words ]

shl edi, 0x02

sub ecx, [ v_cad ]

js .forward

shl esi, 0x02

std

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

cld

.forward:

pop esi

shr edi, 0x02

inc edi

mov [ v_curs ], edi

mov ecx, [ v_words ]

.back:

dec edi

mov [ ( edi * 4 ) + 0x00 ], _TOS_

_DROP_

loop .back

ret

insert:

call insert0

mov cl, [ action ]

xor [ edi * 4 + 0x00 ],cl

cmp cl, 0x03                    ; if we are a red token

jnz .forward

mov byte [ action ], 0x04       ; switch to green

mov dword [ keypad_colour ], colour_green

mov word [ v_hintChar ], 'g'    ; mark the green keypad with a 'g'

.forward:

ret

_word1:

pop dword [ aword ]

mov dword [ aword ], ex1

ret

_word:

mov dword [ aword ], _word1

jmp dword quit_

tokenAction_1:

_DUP_

mov _TOS_, 0x01

cmp byte [ action ], 0x04

jz .forward2

mov al, 0x03

.forward2:

cmp dword [ base ], byte 0x0A

jz .forward

xor al, 0x10

.forward:

_SWAP_

mov dword [ v_words ], 0x02

jmp short insert

tokenAction:

test byte [ action ], 0x0A

jnz .forward

mov edx, _TOS_

and edx, 0xFC000000

jz .forward2

cmp edx, 0xFC000000

jnz tokenAction_1

.forward2:

shl _TOS_, 0x05

xor al, 0x02

cmp byte [ action ], 0x04

jz .forwardBack

xor al, 0x0B

.forwardBack:

cmp dword [ base ], byte 0x0A

jz .forward4

xor al, 0x10

.forward4:

mov dword [ v_words ], 0x01

jmp insert

.forward:

cmp byte [ action ],  0x09

jnz .forward3

mov edx, _TOS_

shl edx, 0x05

sar edx, 0x05

cmp edx, _TOS_

jz .forward5

.forward3:

_DROP_

ret

.forward5:

shl _TOS_, 0x05

xor al, 0x06

jmp short .forwardBack

enstack: ; ( ... n -- )   ; ctrlY action, delete the token at the cursor and put it into the trash buffer

_DUP_

mov _TOS_, [ v_cad ]

sub _TOS_, [ v_pcad ]

jz .forward

mov ecx, _TOS_

xchg _TOS_, edx

push esi

mov esi, [ v_cad ]

lea esi, [ (esi * 4) - 0x04 ]

mov edi, [ v_trash ]    ; setup EDI to point to the current trash buffer address

.back:

std

lodsd ; _DROP_          ; loads EAX with the value pointed to by EDI = [ v_trash ]

cld

stosd                   ; stores EAX into the location pointed to by EDI = [ v_trash ] and increments EDI

loop .back

xchg _TOS_, edx         ;

stosd                   ; stores EAX into the location pointed to by EDI and increments EDI

mov [ v_trash], edi     ; update the current trash buffer address

pop esi

.forward:

_DROP_

ret

deleteAction:

call enstack

mov edi, [ v_pcad ]

mov ecx, [ v_lcad ]

sub ecx, edi

shl edi, 0x02

push esi

mov esi, [ v_cad ]

shl esi, 0x02

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

pop esi

jmp dword cursorLeft

act0:

call enstack

jmp dword cursorLeft

yellowAction:

mov al, 0x01

jmp short actt

redAction:  ; red : start creating a new definition

mov al, 0x03

jmp short actt

greenAction:   ; green, start compiling an existing definition

mov al, 0x04

jmp short actt

textAction:

mov al, 0x09

jmp short actt

CapitalAction:

mov al, 0x0A

jmp short actt

capitalS_Action:

mov al, 0x0B

jmp short actt

grayAction:

mov al, 0x0D

jmp short actt

blueAction:

mov al, 0x0E

jmp short actt

cyanAction:

mov al, 0x07

actt:   ; ( action -- )

mov [ action ], al

mov dword [ aword ], insert

mov _TOS_, [ ( _TOS_ * 4 ) + actionColourTable ]

actn:

mov [ keypad_colour ], _TOS_

pop _TOS_

_DROP_

jmp dword quit_

magentaAction:   ; magenta variable action

mov byte [ action ], 0x0C

mov _TOS_, colour_magenta

mov dword [ aword ], .forward

jmp short actn

.forward:

_DUP_

xor _TOS_, _TOS_

inc dword [ v_words ]

jmp dword insert

editorExit:       ; ( -- )   \ leave the editor

pop _TOS_

_DROP_

mov dword [ aword ], ex1

mov dword [ anumber ], nul

mov byte [ alpha0 + ( 4 * 4 ) ], 0x00

mov dword [ alpha0 + 4 ], nul0

mov dword [ keypad_colour ], colour_yellow

mov byte [ v_quitMode ], 0x00

mov byte [ v_hintChar ], 0x00   ; no hint chararacter

jmp dword quit_

destack:                    ; ctrlZ action, insert the next token from the trash buffer

mov edx, [ v_trash ]

cmp edx, TRASH_BUFFER   ; do not insert if we have emptied the trash buffer

jnz .forward

ret

.forward:

sub edx, byte 0x08

mov ecx, [edx+0x04]

mov [ v_words ], ecx

.back:

_DUP_

mov _TOS_, [edx]

sub edx, byte 0x04

loop .back

add edx, byte 0x04

mov [ v_trash ], edx

jmp dword insert0

; *****************************************************************************

; Locate

; *****************************************************************************

locate:

mov ecx, [ v_blk ]

xchg ecx, [ v_locatedBlock ]

mov [ v_blk ], ecx

mov ecx, [ v_curs ]

xchg ecx, [ v_locatedCurs ]

mov [ v_curs ], ecx

ret

; *****************************************************************************

; Editor keypad and action table

; *****************************************************************************

editorActionTable:

dd nul           , deleteAction    , editorExit    , destack      ;

dd yellowAction  , redAction       , greenAction   , shadow       ; y r g *

dd cursorLeft    , cursorUp        , cursorDown    , cursorRight  ; l u d r

dd previousBlock , magentaAction   , cyanAction    , nextBlock    ; - m c +

dd nul           , capitalS_Action , CapitalAction , textAction   ; _ S C t

dd nul           , locate          , nul           , otherBlock   ; _ L _ j

ekbd0:

dd grayAction    , blueAction      , nul           , act0         ; a b _ _

db 'x'           , '.'             , 'i'           , 0x00         ; four characters to display on the bottom line of the keyboard

editorKeyTableHintChars:    ; display the current edit colour and mode in the bottom right hand corner of the keyboard

db '    '   ;

db 'yrg '   ; y r g _

db '    '   ; l u d r

db ' mc+'   ; - m c +

db ' SCt'   ; _ S C t

db ' l  '   ; _ _ _ j

db 'ab  '   ; a b _ _

; Editor keypad display

; _ S C t  y r g *

; < l > j  l u d r

; a b _ k  - m c +

;     x . i

editorKeypad:			; the main editor keyboard icons

db 'yrg*'  			; yellow, red, green, shadow

db 0x10, 0x11, 0x12, 0x13   ; 'ludr' arrow glyphs (left, up, down, right)

db '-mc+'			; previous block, magenta, cyan, next block

db ' SCt'			; ALL CAPITALS, Capital start, lower case text

db 0x14,'l', 0x17, 'j'      ; left glyph, "locate", right glyph, j = other block

db 'ab  '			; grAy, blue

set_e_main:

mov dword [ shiftAction ], ekbd0

mov dword [ currentKeypadIcons ], ( editorKeypad - 4 )

mov dword [ keypad_colour ], colour_yellow

ret

edit0:

_DROP_

jmp short edit2

save_edit_state:   ; ( n -- )   \ save edit block n

push ecx

mov ecx, [ v_blk ]

mov [ v_otherBlock ], ecx       ; save the current edit block to the "other" block variable

mov ecx, [ v_curs ]

mov [ v_otherCursor ], ecx      ; save the current edit block cursor to the "other" cursor variable

pop ecx

mov [ v_blk ], _TOS_            ; set the new edit block

_DROP_                          ; discard n

ret

swap_with_other_:

push ecx

mov ecx, [ v_blk ]

xchg ecx, [ v_otherBlock ]

mov [ v_blk ], ecx

mov ecx, [ v_curs ]

xchg [ v_otherCursor ], ecx      ; save the current edit block cursor to the "other" cursor variable

mov [ v_curs ], ecx

pop ecx

ret

edit_:   ; ( n -- )   \ edit block n

call save_edit_state

e_:

mov byte [ v_quitMode ], 0xFF

call refresh

plus_e:

mov dword [ anumber ], tokenAction

mov byte [ alpha0+4*4 ], 0x25

mov dword [ alpha0 + 4 ], edit0

edit2:

call set_e_main

.back:

call clearHintChar

call get_key_

push _TOS_

mov al, [ editorKeyTableHintChars + _TOS_ ]

mov [ v_hintChar ], _TOS_

pop _TOS_

call [ ( _TOS_ * 4 ) + editorActionTable ]

_DROP_

jmp short .back

convertAddress:     ; ( a32 -- )    set up the block at the given 32 bit cell address, including the cursor position

mov _SCRATCH_, _TOS_

and _SCRATCH_, 0x00FF

mov [ v_curs ], _SCRATCH_           ; cell offset in block

call cellAddressToBlock

mov [ v_blk ], _TOS_

_DROP_

ret

editAddress_:    ; ( a32 -- )    edit the block at the given 32 bit byte address, including the cursor position

mov _SCRATCH_, _TOS_

shr _SCRATCH_, 2

and _SCRATCH_, 0x00FF

mov [ v_curs ], _SCRATCH_           ; cell offset in block

sub _TOS_, RELOCATED                ; subtract the addess of block 0

shr _TOS_, 10                       ;

call edit_

ret

keypd_:    ; display the keypad vectors and display characters at the address on top of the return stack

pop edx                             ; keypd_ is followed by call table then keymap

mov [ vector ], edx                 ; edx points to the next colorForth word to be executed

add edx, ( 28 * 5 )                 ; 28 keys, 5 bytes per compiled call

mov [ currentKeypadIcons ], edx

sub edx, byte +16

mov [ shiftAction ], edx

.back:

call get_key_                       ; calls pause_ while waiting for a character

mov edx, [ vector ]

add edx, _TOS_

lea edx, [ ( _TOS_ * 4 ) + edx + 0x05 ]

add edx, [ edx - 0x04 ]

_DROP_

keypd1:

call edx

jmp short keypd_.back

; *****************************************************************************

; QWERTY support

; *****************************************************************************

qwertyKeyboard:

dd 0

dd 0

dd 0

dd 0x01040f17       ; 'qwer'

dd 0

dd 0

qwertToggleBase:

xor dword [ setCurrentBase ], ((setBase_decimal - $$) ^ (setBase_hex - $$))

xor byte [ ( numb0 + 12 ) ], 0x2F

qwertToggleBase1:

;    call [ setCurrentBase ]

;    mov dword [ qwertyKeyboard ], 0x00           ; '' => decimal

;    cmp dword [ base ], byte +0x10

;    jnz .forward

;    mov dword [ qwertyKeyboard ], 0x00150414     ; 'hex'

; .forward:

;    mov dword [ currentKeypadIcons ], keypd1

;    mov dword [ shiftAction ], qwertyKeyboard

ret

qwertyAction4:

call qwertToggleBase

jmp qwertyAction3

qwertyActionTable:

dd endn, endn, exitn_, qwertyAction3, qwertyAction4

qwertFunction1:

call right

db 0xC7

add _TOS_, ( qwertyKeyboard + 4 )

push es

push ss

or [_TOS_], _TOS_

call qwertToggleBase1

mov byte [ v_sign ], 0x00

mov _TOS_, [ v_digin ]

qwertyAction5:

call get_qwerty_key_

jz .forward4

jmp dword [ _TOS_ * 4 + qwertyActionTable - 0x200 ]

.forward4:

test _TOS_, _TOS_

jng qwertyAction3

cmp al, 0x23

jz .forward3

mov _TOS_, [ v_digin ]

cmp _TOS_, [ base ]

jns .forward2

test byte [ v_sign ], 0xFF

jz .forward

neg _TOS_

.forward:

mov edx, [ esi ]

imul edx, [ base]

add edx, _TOS_

mov [ esi ], edx

.forward2:

jmp short qwertyAction3

.forward3:

xor [ v_sign ], _TOS_

neg dword [ esi ]

qwertyAction3:

_DROP_

jmp short qwertyAction5

qwertToggleBaseTable2:

dd lj, lj, exit_

qwertyFunction2:

mov dword [ ( qwertyKeyboard + 4 ) ], 0x02150402  ; 'text'

call right

mov dword [ v_words ], 0x01

mov dword [ chars], 0x01

_DUP_

mov dword [ esi ], 0x00

mov byte [ bits_ ], 0x1C

.back:

jz .forward

cmp _TOS_, 0x83

jns .forward

jmp dword [ _TOS_*4 + qwertToggleBaseTable2 - 0x200 ]

.forward:

test _TOS_, _TOS_

jng .forward2

cmp _TOS_, 0x30

jns .forward2

_DUP_

call echo_

call pack_

inc dword [ chars]

.forward2:

_DROP_

call get_qwerty_key_

jmp short .back

qwertyAction2:

call qwertToggleBase

jmp dword nul0

qwertyAction1:

jmp dword [ alpha0 + 4 ]

qwertyTable1:

dd nul0

dd nul0

dd nul0

dd qwertyAction1

dd qwertyAction2

qwertyDoAction:

mov dword [ ( qwertyKeyboard + 4 ) ], 0x00   ; clear the 'text' string

mov dword [ shiftAction ], qwertyKeyboard

mov dword [ currentKeypadIcons ], keypd1

.back2:

call get_qwerty_key_

jz .forward

jmp dword [ ( _TOS_ * 4 ) + qwertyTable1 - 0x0200 ]

.forward:

cmp al, 0x30

jnz .back

mov dword [ ( qwertyKeyboard + 4 ) ], 0x02150402  ; 'text'

_DROP_

jmp short .back2

.back:

test _TOS_, _TOS_

jng .forward3

test dword [ ( qwertyKeyboard + 4 ) ], 0xFFFFFFFF

jnz .forward2

cmp byte [ v_digin ], 0x0A

js qwertFunction1

.forward2:

cmp _TOS_, 0x30

jns .forward3

call qwertyFunction2

call [ aword ]

_DUP_

.forward3:

_DROP_

jmp dword quit_

qwert:      ; selects QWERTY keyboard entry

mov dword [ x_qwerty ], qwertyDoAction

ret

; *****************************************************************************

abort_action:

cmp edi, ( RELOCATED / 4 )  ; if we are compiling a block, show the location of the error

; edi is a cell address, so divide by 4

jc .forward

_DUP_

mov _TOS_, [ v_blk ]

mov [ v_otherBlock ], _TOS_ ; save the last block to be edited

mov _TOS_, edi

call convertAddress

.forward:

mov esp, RETURN_STACK_0

cmp esi, ( DATA_STACK_0 + 4 )

jc .forward2

mov esi, ( DATA_STACK_0 + 4 )

.forward2:

mov dword [ tokenActions + ( 3 * 4 ) ], forthd

mov dword [ tokenActions + ( 4 * 4 ) ], qcompile

mov dword [ tokenActions + ( 5 * 4 ) ], cnum

mov dword [ tokenActions + ( 6 * 4 ) ], cshort

mov _TOS_, 0x3F     ; '?' character to follow the display of the unknown word

call echo_

;    jmp abort_e2

jmp dword quit_

; *****************************************************************************

rquery: ; r?

_DUP_

mov _TOS_, RETURN_STACK_0

sub _TOS_, esp

shr _TOS_,1

shr _TOS_,1

ret

boot:

; see http://wiki.osdev.org/PS2_Keyboard#CPU_Reset

mov al, 0xFE

out 0x64, al

jmp short $         ; we should never get here, because the processor will be rebooted... stop here just in case

wipe:  ; ( -- )    \ wipe the currently edited block

_DUP_

mov _TOS_, [ v_blk ]

mov ecx, 0x40

wipe2:

push edi

call blockToCellAddress     ; add the RELOCATED block number offset and convert to cell address

shl _TOS_, 2                ; convert to byte address

mov edi, _TOS_

xor _TOS_, _TOS_

rep stosd  ; stores  eax  into the location pointed to by  edi  then increments  edi  by 4, does this  ecx  times

pop edi

_DROP_

ret

wipes:  ; ( startblock# #blocks -- )    \ wipes #blocks starting from block startblock#   ( was erase )

mov ecx, _TOS_

shl ecx, 0x06               ;  convert blocks to cells, multiply by 64

_DROP_

jmp wipe2

copy_:   ; ( blk -- )    \ copy the given block (and shadow) to the currently displayed block (and shadow)

cmp _TOS_, byte 0x0C        ; below block 12 is machine code

jc abort_

push edi

push esi

push ecx

call blockToCellAddress     ; source block

shl _TOS_, 0x02             ; convert cell address to byte address

mov esi, _TOS_

mov _TOS_, [ v_blk ]

call blockToCellAddress     ; destination block

shl _TOS_, 0x02             ; convert cell address to byte address

mov edi, _TOS_

mov ecx, 0x0200

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

pop ecx

pop esi

pop edi

_DROP_

ret

debug:

mov dword [ v_gr_xy ], 0x302B5

_DUP_

mov _TOS_, [ main ]

push dword [_TOS_]

call dotHex

_DUP_

pop _TOS_

call dotHex

_DUP_

mov _TOS_, [ draw ]

call dotHex

_DUP_

mov _TOS_, esi

jmp dword dotHex

; *****************************************************************************

tic0:

dec dword [ v_words ]

jz .forward

_DROP_

jmp short tic0

.forward:

ret

tic_:   ; ( -- a )   \ return the byte address of the next word entered

call _word          ; allow user to enter the word to  search for

call tic0           ; remove the entered word from the stack

call find_          ; find the word in the dictionary, return its index in  ecx

jnz abort_

mov _TOS_, [ ( ecx * 4 ) + ForthJumpTable ]     ; return the word's address from the jump table

ret

itick:

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call find_

mov _TOS_, [ ( ecx * 4 ) + ForthJumpTable ]

ret

; *****************************************************************************

plusList_words:   ; ( -- ) display the current colorForth block

_DUP_

xor _TOS_, _TOS_

mov [ currentState ], _TOS_

mov [ lastState ], _TOS_

mov [ v_curs ], _TOS_           ; set the cursor to top left (0, 0) ToDo: Note : does not work!

_DROP_

call setupText_                 ; setup the clip window for this display

_DUP_

mov _TOS_, [ v_lcad ]

mov [ v_cad ], _TOS_

mov _TOS_, [ v_blk ]            ; get the current block number to be edited

call blockToCellAddress         ; add the RELOCATED block number offset and convert to cell address

mov edi, _TOS_

xor _TOS_, _TOS_

add edi, [ v_lineOffset ]

mov [ v_pcad ], edi

.back:

mov edx, dword [ ( edi * 4 ) + 0x00 ]   ; edi is the display pointer and is a cell address

call show_cursor                        ; show the PacMan-like cursor

call space_

inc edi

and edx, byte 0x0F

_DUP_

mov _TOS_, 0x09

; call doColourBlind

call [ ( edx * 4 ) + displayShannonFanoActions ]

jmp short .back

refresh_words:                      ; refresh the editor display to show all Forth words

call show                       ; set the screen task to execute the code following :

call page_                      ; clear the screen

; call displayBlockNumber       ; display the current block number on the screen

call plusList_words             ; list the contents of the block

_DUP_

; mov _TOS_, 0x0F

; call doColourBlind             ; display the final colour-blind punctuation, set up for next call of plusList

jmp dword displayTheKeypad_

words_:     ; \ show the ForthNames array as if it is a sequence of blocks

_DUP_

mov _TOS_, ForthNames

call a2blk_

call save_edit_state

call refresh_words

ret

; *****************************************************************************

; Int 0x13 AH Return Code error type

; 0x00 Success

; 0x01 Invalid Command

; 0x02 Cannot Find Address Mark

; 0x03 Attempted Write On Write Protected Disk

; 0x04 Sector Not Found

; 0x05 Reset Failed

; 0x06 Disk change line 'active'

; 0x07 Drive parameter activity failed

; 0x08 DMA overrun

; 0x09 Attempt to DMA over 64kb boundary

; 0x0A Bad sector detected

; 0x0B Bad cylinder (track) detected

; 0x0C Media type not found

; 0x0D Invalid number of sectors

; 0x0E Control data address mark detected

; 0x0F DMA out of range

; 0x10 CRC/ECC data error

; 0x11 ECC corrected data error

; 0x20 Controller failure

; 0x40 Seek failure

; 0x80 Drive timed out, assumed not ready

; 0xAA Drive not ready

; 0xBB Undefined error

; 0xCC Write fault

; 0xE0 Status error

; 0xFF Sense operation failed

; *****************************************************************************

; 16 bit BIOS disk read/write from 32 bit

; *****************************************************************************

; set the required parameters into the DAP buffer for the LBA BIOS extended read/write calls.

; Also set up the extra DAP buffer values for use by the CHS BIOS calls, if the LBA call fails.

; This is to avoid returning from 16 bit mode to calculate the values.

setupDAP_:   ; ( sector n cmd -- )   \ setup the DAP for the given LBA sector number

push edi

xor ecx, ecx

mov edi, (data_area - $$ + BOOTOFFSET)   ; setup the data index pointer

mov cx, [ word di + ( driveinfo_Drive_DX - data_area ) ]  ; restore the boot drive into dl

mov edi, DAP_BUFFER

mov word [ edi + o_Int13_DAP_saved_DX ], cx     ; setup DX value returned by the BIOS

mov word [ edi + o_Int13_DAP_readwrite ], ax    ; set the read/write cmd value, 0x0000 or 0x0001

_DROP_

; limit the number of sectors to the size of the SECTOR_BUFFER

cmp _TOS_, ( SECTOR_BUFFER_SIZE / 0x0200 )

js .forward

mov _TOS_, ( SECTOR_BUFFER_SIZE / 0x0200 )

.forward:

mov word [ edi + o_Int13_DAP_num_sectors ], ax

_DROP_

mov dword [ edi + o_Int13_DAP_LBA_64_lo ], eax

push eax    ; save for later

xor eax, eax

mov dword [ edi + o_Int13_DAP_LBA_64_hi ], eax

; buffer within low 16 bits of address space

mov word [ edi + o_Int13_DAP_segment ], ax

mov ax, ( SECTOR_BUFFER )

mov word [ edi + o_Int13_DAP_address ], ax

; set the configuration buffer values from the registers

mov eax, 0x0010

mov word [ edi + o_Int13_DAP_size ], ax  ; setup DAP buffer size

; setup values for CHS BIOS disk calls

pop eax                         ; restore the start sector number

add eax, [ bootsector - $$ + BOOTOFFSET]    ; add the bootsector from the drive parameter table

push eax                        ; save it while we calculate heads*sectors-per-track

mov al, [ driveinfo_Head - $$ + BOOTOFFSET]      ; index of highest-numbered head

inc al                          ; 1-base the number to make count of heads

mul byte [ driveinfo_SectorsPertrack - $$ + BOOTOFFSET]     ; sectors per track

mov ebx, eax

pop eax

xor edx, edx                    ; clear high 32 bits

div ebx                         ; leaves cylinder number in eax, remainder in edx

mov ecx, eax                    ; store cylinder number in another register

mov eax, edx                    ; get remainder into AX

mov bl, [ driveinfo_SectorsPertrack - $$ + BOOTOFFSET]      ; number of sectors per track

div bl                          ; head number into AX, remainder into DX

mov bl, al                      ; result must be one byte, so store it in BL

rol ecx, 8                      ; high 2 bits of cylinder number into high 2 bits of CL

shl cl, 6                       ; makes room for sector number

or cl, ah                       ; merge cylinder number with sector number

inc cl                          ; one-base sector number

mov word [ edi + o_Int13_DAP_saved_CHS_CX ], cx  ; also save the calculated CX value

mov cx, [ driveinfo_Drive_DX - $$ + BOOTOFFSET]    ; drive number in low 8 bits

mov ch, bl                      ; place head number in high bits

mov word [ edi + o_Int13_DAP_saved_CHS_DX ], cx  ; also save the calculated DX value

pop edi

_DROP_

ret

; *****************************************************************************

; BIOS read/write 512 byte LBA sectors

; *****************************************************************************

BIOS_ReadWrite_Sector_LBA: ; ( -- )   \ try to read or write using the extended disk BIOS calls,

; \ if that fails, try the CHS BIOS call. Parameters are in the DAP buffer.

pushf   ; save the processor flags, especially interrupt enable

%ifdef NOT_BOCHS

call restore_BIOS_idt_and_pic   ;

%endif

_DUP_

xor _TOS_, _TOS_

call lidt_                      ; Load the BIOS Interrupt Descriptor Table

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

mov si, DAP_BUFFER

mov byte ah, [ si + o_Int13_DAP_readwrite ]  ; 0x00 for read, 0x01 for write

or  ah, 0x42                    ; BIOS extended read/write

mov al, 0x00

mov dx, [ si + o_Int13_DAP_saved_DX ]

int 0x13

cli                             ; BIOS might have left interrupts enabled

mov word [ si + o_Int13_DAP_returned_AX ], ax  ; save the value in AX that the BIOS call returned

jnc .forward

mov si, DAP_BUFFER

mov byte ah, [ si + o_Int13_DAP_readwrite ]  ; 0x00 for read, 0x01 for write

or  ah, 0x02                ; CHS BIOS mode, read  al  sectors, set above

mov al, byte [ si + o_Int13_DAP_num_sectors ]   ; restore the number of sectors saved by setupDAP_

mov word cx, [ si + o_Int13_DAP_saved_CHS_CX ]  ; restore the CX value calculated by sector_chs

mov word dx, [ si + o_Int13_DAP_saved_CHS_DX ]  ; restore the DX value calculated by sector_chs

mov word bx, [ si + o_Int13_DAP_address ]       ; restore the address saved by setupDAP_

int 0x13

cli                         ; BIOS might have left interrupts enabled

mov si, DAP_BUFFER

mov word [ si + o_Int13_DAP_returned_AX ], ax  ; the BIOS call returned AX

mov ax, 0x0001

jc .forward2

mov ax, 0x0000

.forward:

mov [ si + o_Int13_DAP_returned_carry_flag ], ax  ; the BIOS call returned carry flag

.forward2:

mov [ si + o_Int13_DAP_returned_carry_flag ], ax  ; the BIOS call returned carry flag

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

%ifdef NOT_BOCHS

call restore_new_idt_and_pic

%endif

_DUP_

mov _TOS_, INTERRUPT_VECTORS

call lidt_                      ; Load the new Interrupt Descriptor Table

popf   ; restore the processor flags, especially interrupt enable

ret

Read_Sector_LBA:    ; ( sector n -- )   "rlba"   GetFlag returns 0 for success

_DUP_

mov eax, 0x0000                 ; read command

call setupDAP_                  ; setup up the DAP table using 3 items from the stack ( start n cmd -- )

cli                             ; disable interrupts

pushad                          ; Pushes all general purpose registers onto the stack

call BIOS_ReadWrite_Sector_LBA

popad                           ; restore the registers pushed by  pushad

ret

Write_Sector_LBA:   ; ( sector n -- )   "wlba"

_DUP_

mov eax, 0x0001                 ; write command

call setupDAP_                  ; setup up the DAP table using 3 items from the stack ( start n cmd -- )

cli                             ; disable interrupts

pushad                          ; Pushes all general purpose registers onto the stack

call BIOS_ReadWrite_Sector_LBA

popad                           ; restore the registers pushed by  pushad

ret

ReadSectors:    ; ( a sector n -- a' )  \ read  n  sectors from  sector  into address  a

call Read_Sector_LBA            ; reads  n  sectors starting from  sector  into the SECTOR_BUFFER

push esi                        ; esi is changed by rep movsw

mov esi, DAP_BUFFER

xor ecx, ecx

mov word cx, [ si + o_Int13_DAP_num_sectors ]   ; restore the number of sectors saved by setupDAP_

mov ebx, ecx                    ; save number of sectors for later

mov esi, SECTOR_BUFFER          ; source address

mov edi, eax                    ; destination address

shl ecx, 0x07                   ; 512 bytes in cells = 2 ** 7

rep movsd                       ; does not change AX , it moves DS:SI to ES:DI and increments SI and DI

; ( a -- a' )

mov ecx, ebx

shl ecx, 0x09                   ; 512 bytes in bytes = 2 ** 9

add eax, ecx                    ; increment the address that is TOS

pop esi

; ( a -- a' sector' )

_DUP_

push esi

mov esi, DAP_BUFFER             ; esi is changed by rep movsd above

xor ecx, ecx

mov word cx, [ si + o_Int13_DAP_LBA_64_lo ]   ; restore the start sector

pop esi

mov eax, ecx

add eax, ebx

;    call GetFlag

ret

WriteSectors:    ; ( a sector n -- a' ) \ write  n  sectors starting at  sector  from address  a

push ecx

push edx

mov edx, [ esi + 4 ]            ; save  a  from stack in  edx

push esi                        ; esi is also changed by rep movsw

mov esi, DAP_BUFFER

xor ecx, ecx

mov word cx, [ si + o_Int13_DAP_num_sectors ]   ; restore the number of sectors saved by setupDAP_

mov ebx, ecx                    ; save number of sectors for later

shl ecx, 0x07                   ; 512 bytes in cells = 2 ** 7

mov esi, edx                    ; source address

mov edi, SECTOR_BUFFER          ; destination address

rep movsd                       ; does not change AX , it moves DS:SI to ES:DI and increments SI and DI

pop esi

push ebx

call Write_Sector_LBA    ; writes  n  sectors starting from  sector  from the SECTOR_BUFFER

pop ebx

;    push esi                        ; esi is also changed by rep movsw

; ( a -- a' )

mov ecx, ebx

shl ecx, 0x09                   ; 512 bytes in bytes = 2 ** 9

add eax, ecx                    ; increment the address that is TOS

;    pop esi

; ( a -- a' sector' )

_DUP_

push esi

mov esi, DAP_BUFFER             ; esi is changed by rep movsd above

xor ecx, ecx

mov word cx, [ si + o_Int13_DAP_LBA_64_lo ]   ; restore the start sector

pop esi

mov eax, ecx

add eax, ebx

pop edx

pop ecx

ret

SaveAll_:    ; ( -- ) "sss"

pushf   ; save the processor flags, especially interrupt enable

cli

_DUP_

xor eax, eax

call block_

_DUP_

xor eax, eax

mov ecx, 0x21   ; 32 x 16 Kbytes= 512 + 32 Kbytes

.back:

_DUP_

mov eax, 0x20   ; 32 x 512 byte sectors = 16 Kbytes

call WriteSectors   ; ( a sector n -- a' ) \ write  n  sectors starting at  sector  from address  a

loop .back

_DROP_

_DROP_

;    ; repeat the first group of sectors, to flush the save

;    _DUP_

;    xor eax, eax

;    call block_

;    _DUP_

;    xor eax, eax

;    mov ecx, 0x01   ; 1 x 16 Kbytes= 162 Kbytes

;    .back2:

;    _DUP_

;    mov eax, 0x20   ; 32 x 512 byte sectors = 16 Kbytes

;    call WriteSectors   ; ( a sector n -- a' ) \ write  n  sectors starting at  sector  from address  a

;    loop .back2

;    _DROP_

;    _DROP_

;    ; repeat the last sector, to flush the save

;    _DUP_

;    ; address

;    mov eax, LAST_BLOCK_NUMBER

;    call block_

;    _DUP_

;    ; sector number

;    mov eax, ( LAST_BLOCK_NUMBER * 2 )  ; 01 x 512 byte sectors = 512 bytes, just write one sector

;    _DUP_

;    ; number of 512 byte sectors to write

;    mov eax, 0x01   ; 01 x 512 byte sectors = 512 bytes, just write one sector

;    call WriteSectors   ; ( a sector n -- a' ) \ write  n  sectors starting at  sector  from address  a

;    _DROP_

;    _DROP_

popf   ; restore the processor flags, especially interrupt enable

ret

GetFlag: ; ( -- error | 0 )   0 for success, else the error type ( eax == 0x100 is Invalid Command )

_DUP_

xor eax, eax

push edi

mov edi, DAP_BUFFER

mov ax, [ edi + o_Int13_DAP_returned_carry_flag ]  ; the BIOS call returned carry flag

add ax, 0

jz .forward

mov ax, [ edi + o_Int13_DAP_returned_AX ]  ; the BIOS call returned error value in ax

.forward:

pop edi

ret

%if 0

BIOS_Read_Sector_CHS:

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

mov si, DAP_BUFFER

mov al, byte [ si + o_Int13_DAP_num_sectors ]   ; setup the number of sectors saved by setupDAP_

;    and al, 0x0F        ; limit to 16 sectors

mov ah, 0x02        ; CHT BIOS mode, read  al  sectors, set above

mov word cx, [ si + o_Int13_DAP_saved_CHS_CX ]  ; setup the CX value calculated by sector_chs

mov word dx, [ si + o_Int13_DAP_saved_CHS_DX ]  ; setup the DX value calculated by sector_chs

mov word bx, [ si + o_Int13_DAP_address ]       ; setup the address saved by setupDAP_

int 0x13

cli                             ; BIOS might have left interrupts enabled

mov si, DAP_BUFFER

mov word [ si + o_Int13_DAP_returned_AX ], ax  ; the BIOS call returned AX

mov ax, 0x0001

jc .forward

mov ax, 0x0000

.forward:

mov [ si + o_Int13_DAP_returned_carry_flag ], ax  ; the BIOS call returned carry flag

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

ret

; rchs:

Read_Sector_CHS:    ; ( sector n -- f )   "rchs"  returns 0 for success

call setupDAP_              ; ( start n -- ) store the sector number into the Disk Address Packet

cli                         ; disable interrupts

pushad                      ; Pushes all general purpose registers onto the stack

call BIOS_Read_Sector_CHS

popad                       ; restore the registers pushed by  pushad

;    _DROP_

jmp GetFlag

; wcht:

Write_Sector_CHS:   ; ( sector -- )   "wcht"

call setupDAP_              ; store the sector number into the Disk Address Packet

cli                         ; disable interrupts

pushad                      ; Pushes all general purpose registers onto the stack

call BIOS_Read_Sector_CHS

popad                       ; restore the registers pushed by  pushad

ret

%endif

; *****************************************************************************

; *****************************************************************************

%if 0

[BITS 16]                           ; Real Mode code (16 bit)

storeBefore:    ; ( -- )   \ store registers to the V_REGS array

mov word [ V_REGS + 0x00 ], ax

mov word [ V_REGS + 0x04 ], bx

mov word [ V_REGS + 0x08 ], cx

mov word [ V_REGS + 0x0C ], dx

mov word [ V_REGS + 0x10 ], si

mov word [ V_REGS + 0x14 ], di

mov word [ V_REGS + 0x18 ], bp

push ax                         ; save eax

pushfd                          ; push the 32 bit eflags register onto the stack

pop ax                          ; and pop it off into eax

mov word [ V_REGS + 0x1C ], ax  ; eflags

pop ax

mov word [ V_REGS + 0x1E ], ax  ; eflags top 16 bits

pop ax                          ; restore eax

ret

storeAfter:     ; ( -- )   \ store registers to the V_REGS array

mov word [ V_REGS + 0x20 ], ax

mov word [ V_REGS + 0x24 ], bx

mov word [ V_REGS + 0x28 ], cx

mov word [ V_REGS + 0x2C ], dx

mov word [ V_REGS + 0x30 ], si

mov word [ V_REGS + 0x34 ], di

mov word [ V_REGS + 0x38 ], bp

push ax                         ; save eax

pushfd                          ; push the 32 bit eflags register onto the stack

pop ax                          ; and pop it off into eax

mov word [ V_REGS + 0x3C ], ax  ; eflags

pop ax

mov word [ V_REGS + 0x3E ], ax  ; eflags top 16 bits

pop ax                          ; restore eax

ret

[BITS 32]                           ; Protected Mode code (32 bit)

BIOS_thunk:     ; ( -- )   \ call the BIOS - registers will have previously been setup

call setRealModeAPI

[BITS 16]                           ; Real Mode code (16 bit)

push ax

push es                         ; this operation messes with ES

push di                         ; and DI

call storeBefore

int 0x13

jc $                            ; stop here on error

call storeAfter

pop di

pop es

pop ax

cli                             ; BIOS might have left interrupts enabled

call setProtectedModeAPI        ; called from 16 bit code, returns to 32 bit code

[BITS 32]                           ; Protected Mode code (32 bit)

ret

%endif

%if 0

th_:        ; ( ax bx cx dx si di es -- w )   \ th ( thunk to BIOS Int 0x13 )

; eax = 0x DH DL AH AL , returns in same order

cli                             ; disable interrupts

pushad  ; Pushes all general purpose registers onto the stack in the following order:

; EAX, ECX, EDX, EBX, ESP, EBP, ESI, EDI. The value of ESP is the value before the actual push of ESP

;  7    6    5    4    3    2    1    0   offset in cells from ESP

;    call setupDAP_

push edi

mov di, (data_area - $$ + BOOTOFFSET)   ; setup the data index pointer

mov dx, [ byte di + ( driveinfo_Drive_DX - data_area) ]  ; restore the boot drive from dx (and head? )

;    mov dl, 0x80

mov ebx, SECTOR_BUFFER

mov eax, ( 0x0200 + ( ( SECTOR_BUFFER_SIZE / 512 ) & 0xFF ) ) ; read n sectors to fill the buffer

mov ecx, 0x0201                  ; cylinder | sector

call BIOS_thunk

pop edi

popad   ; restore the stack values pushed by  pushad

ret

%endif

%if 0

XXXrsect_:     ; ( sector -- ax )   \

pushad  ; Pushes all general purpose registers onto the stack

push edi

;    call sector_chs                 ; store th sector number into the Disk Address Packet

mov di, (data_area - $$ + BOOTOFFSET)   ; setup the data index pointer

mov dx, [ byte di + ( driveinfo_Drive_DX - data_area) ]  ; restore the boot drive from dx (and head? )

;    mov dl, 0x80

cli                             ; disable interrupts

;    mov esi, DAP_BUFFER

;    _DUP_

mov eax, 0x0201     ; BIOS read, one sector

mov bx, SECTOR_BUFFER

call BIOS_thunk

pop edi

popad   ; restore the stack values pushed by  pushad

ret

%endif

; *****************************************************************************

; *****************************************************************************

%define FORTH_INITIAL_WORD_COUNT  ( ( ForthJumpTableROM_end - ForthJumpTableROM ) / 4 )     ; in cells

%define MACRO_INITIAL_WORD_COUNT  ( ( MacroJumpTableROM_end - MacroJumpTableROM ) / 4 )     ; in cells

%define BLUE_INITIAL_WORD_COUNT   ( ( BlueJumpTableROM_end  - BlueJumpTableROM )  / 4 )     ; in cells

warm:   ; warm start

mov _SCRATCH_, STACK_MEMORY_START ; start of stack memory area

mov ecx, ( TOTAL_STACK_SIZE >> 2 )  ; number of 32 bit cells to fill with the pattern

.back:

mov dword [ _SCRATCH_ ], 0x55555555             ; fill with this pattern

add _SCRATCH_, 0x04

loop .back

xor ecx, ecx                    ; assumed by initshow to have been previously zeroed

;    call initshow                  ; sets up do-nothing "show" task

call refresh                    ; starts the editor display task

call initserv1_                 ; sets up do-nothing "serv1" task

call initserv2_                 ; sets up do-nothing "serv2" task ToDo: fix the serv2 task...

mov dword [ v_ForthWordCount ], FORTH_INITIAL_WORD_COUNT ; initial #words

mov dword [ v_MacroWordCount ], MACRO_INITIAL_WORD_COUNT ; initial #macros

mov dword [ v_BlueWordCount ],  BLUE_INITIAL_WORD_COUNT  ; initial #blues

mov dword [ v_trash ], TRASH_BUFFER

push esi

;Forth wordlist

lea esi, [ ForthNamesROM ]

mov edi, ForthNames

mov ecx, [ v_ForthWordCount ]

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

lea esi, [ ForthJumpTableROM ]

mov edi, ForthJumpTable

mov ecx, [ v_ForthWordCount ]

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

; Macro wordlist

lea esi, [ MacroNamesROM ]

mov edi, MacroNames

mov ecx, [ v_MacroWordCount ]

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

lea esi, [ MacroJumpTableROM ]

mov edi, MacroJumpTable

mov ecx, [ v_MacroWordCount ]

rep  movsd                  ; copy ecx 32 bit words from ds:esi to es:edi

pop esi

mov dword [ v_H ], H0

mov dword [ x_qwerty ], 0x00        ; select non-qwerty mode

mov dword [ v_offset ], ( RELOCATED >> ( 2 + 8 ) ) ; 0x10000 >> 2 >> 8, offset of RELOCATED block 0 as 1024 byte block number

; Historical note. This bug took about 15 hours to find and fix...

; Below is code to track down a bug : Block 64 offset 0x7C contained 0x800

; The code with the two test functions was re-compiled using the cf2022Ref.img file

; Then block 64 was manually fixed ": rtc 94 ld ;"

; The source blocks were saved with "sa"

; cf2022 was restarted without recompilation

; Looking at blocks 506 and 507 showed that the bug occurred between the two copy functions

;

;    ; OK at this point

;    mov _SCRATCH_, [ v_blk ]

;    mov dword [ v_blk ], 506   ; block 506 shows corruption

;    mov _TOS_, 64

;    call copy_

;    mov [ v_blk ], _SCRATCH_

; setup  v_bytesPerLine

mov _TOS_, [ vesa_XResolution ]

and _TOS_, 0xFFFF

imul _TOS_, BYTES_PER_PIXEL

mov [ v_bytesPerLine ], _TOS_

; was :     mov [ v_bytesPerLine + RELOCATED ], _TOS_   <--- BUG!!!

;    ; NOT OK at this point

;    mov _SCRATCH_, [ v_blk ]

;    mov dword [ v_blk ], 507   ; block 506 shows corruption???

;    mov _TOS_, 64

;    call copy_

;    mov [ v_blk ], _SCRATCH_

; set up  fov

mov _TOS_, [ vesa_YResolution ]

and _TOS_, 0x0000FFFF

mov _SCRATCH_, _TOS_

shl _SCRATCH_, 1

shr _TOS_, 1

add _TOS_, _SCRATCH_

imul _TOS_, 10

mov [ v_fov ], _TOS_

; select which code to use, depending on the display mode

mov byte [ displayMode ], 0

cmp word [ vesa_XResolution ], scrnw1

jz .forward

mov byte [ displayMode ], 1

.forward:

; *****************************************************************************

; miscellaneous setup

; *****************************************************************************

call randInit_      ; initialise the Marsaglia Pseudo Random Number Generator

call initIconSize   ; sets up the size of an icon (glyph) according to the 800x600 or 1024x768 display size

call cursorHome     ; setup the initial cursor location

call c_             ; clear the stack

; *****************************************************************************

; erase the DAP buffer, for the Int 0x13 Disk Address Packet (DAP)

; *****************************************************************************

_DUP_

mov _TOS_, SECTOR_BUFFER

_DUP_

mov _TOS_, SECTOR_BUFFER_SIZE

call erase_

; *****************************************************************************

; load the colorForth source starting at the first colorForth source block

; *****************************************************************************

_DUP_

mov _TOS_, START_BLOCK_NUMBER

_DUP_                           ; not sure why we need this...

call _load_

jmp dword quit_

; *****************************************************************************

; *****************************************************************************

pad_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_pad

ret

v_srch:     ; variables to search for a token name

dd 0xC4B80000   ; token name "pad"

dd 0            ; token name extension (optional)

dd 0            ; current found address

dd 0            ; last found address

dd ( START_BLOCK_NUMBER * 1024 )        ; start searching from here

dd ( ( LAST_BLOCK_NUMBER + 1 ) * 1024 ) ; end the search here

vsrch_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_srch

ret

srch_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_srch

ret

; align 4, db 0  ; variables must be on dword boundary so that "dump" can show them correctly

;

; hsvv:    ; the start address of the pre-assembled high level Forth words

;     dd 0

;     times 0x28 db 0

xy_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_gr_xy

ret

x_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_x

ret

y_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_y

ret

z_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_z

ret

lblk_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_lblk

ret

fov_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_fov

ret

tokenActions_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS tokenActions

ret

last_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS last

ret

version_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS version

ret

vframe_:     ; ( -- a )  \ return the video frame address, where we create the image to be displayed

_DUP_

mov _TOS_, [ vframe ]

ret

vars_:

_DUP_

LOAD_RELATIVE_ADDRESS vars

ret

base_:

_DUP_

LOAD_RELATIVE_ADDRESS base

ret

hex_:

mov byte [ base ], 16

ret

decimal_:

mov byte [ base ], 10

ret

block_: ; ( block -- address )   \ : block ( n -- n ) $400 * ; block number to byte address of block

shl _TOS_, 0x0A

add _TOS_, RELOCATED

ret

a2blk_: ; ( address -- block )   \ byte address of block to block number

sub _TOS_, RELOCATED

shr _TOS_, 0x0A

ret

scrnw_:    ; ( -- n )   screen width ( number of horizontal pixels )

_DUP_

xor _TOS_, _TOS_

mov word ax, [ vesa_XResolution ]

ret

scrnh_:    ; ( -- n )    screen height ( number of vertical pixels )

_DUP_

xor _TOS_, _TOS_

mov word ax, [ vesa_YResolution ]     ; v_scrnh

ret

bpp_:    ; ( -- n )    bits per pixel

_DUP_

xor _TOS_, _TOS_

mov byte al, [ vesa_BitsPerPixel ]     ; v_bitsPerPixel

ret

iconw_:    ; ( -- n )    icon width ( number of pixels between characters, fixed font width )

_DUP_

mov _TOS_, [ v_iconw ]

ret

iconh_:    ; ( -- n )    icon height ( number of pixels between lines )

_DUP_

mov _TOS_, [ v_iconh ]

ret

counter_:   ; ( -- n )    roughly 1 ms counter

_DUP_

RDTSC   ; Read Time-Stamp Counter  https://c9x.me/x86/html/file_module_x86_id_278.html

mov ecx, 1000

idiv ecx

ret

; : drop ( lodsd, flags unchanged, why sp is in ESI )

; : a! ?lit if $BA 1, , ; then $D08B 2, drop ;

; : p@ ( a-n ) qdup a! $EC 1, ;

; : p! ( na- ) a! $EE 1, drop ;

; ( Real Time Clock )

; : rtc@ ( t-c ) $70 p! $71 p@ ;

; : rtc! ( ct- ) $70 p! $71 p! ;

; : hi ( -- ) #10 rtc@ $80 and drop 0if hi ; then ;

; : lo ( -- ) #10 rtc@ $80 and drop if lo ; then ;

; : calkhz ( -- ) hi lo counter hi lo counter swap -

;    dup onesec ! #1 rshift #250 + #500 / dup khz ! ;

; : ms ( n- ) khz @ * counter + begin pause dup counter

;    invert + drop -if drop ; then end drop ;

p70_fetch:  ;  ( reg -- c )

mov edx, 0x70   ; db 0xBA  dd 0x70

IN AL, DX       ; db 0xEC

ret

p70_store:  ;  ( c reg -- )

mov edx, 0x70   ; db 0xBA  dd 0x70

OUT DX, AL      ; db 0xEE

_DROP_

_DROP_

ret

p71_fetch:  ;  ( reg -- c )

mov edx, 0x71   ; db 0xBA  dd 0x71

IN AL, DX       ; db 0xEC

ret

p71_store:  ;  ( c reg -- )

mov edx, 0x71   ; db 0xBA  dd 0x71

OUT DX, AL      ; db 0xEE

_DROP_

_DROP_

ret

rtc_fetch_:  ;  ( reg -- c )

mov edx, 0x70   ; db 0xBA  dd 0x70

OUT DX, AL      ; db 0xEE

mov edx, 0x71   ; db 0xBA  dd 0x71

IN AL, DX       ; db 0xEC

ret

rtc_store_:  ;  ( c reg -- )

mov edx, 0x70   ; db 0xBA  dd 0x70

OUT DX, AL      ; db 0xEE

mov edx, 0x71   ; db 0xBA  dd 0x71

_DROP_

OUT DX, AL      ; db 0xEE

_DROP_

ret

rtc_hi:     ; ( -- )   wait for the RTC second pulse to go high

_DUP_

.back:

mov _TOS_, 10   ; Update in progress" flag (bit 7 of Status Register A).

call rtc_fetch_

and al, 0x80

jz .back

_DROP_

ret

rtc_lo:     ; ( -- )   wait for the RTC second pulse to go low

_DUP_

.back:

mov _TOS_, 10   ; Update in progress" flag (bit 7 of Status Register A).

call rtc_fetch_

and al, 0x80

jnz .back

_DROP_

ret

get_proc_clk:   ; ( -- d )  get the processor clock counter

_DUP_

RDTSC                           ; Read Time-Stamp Counter  https://c9x.me/x86/html/file_module_x86_id_278.html

_DUP_

mov _TOS_, edx                  ; put the high cell in TOS

ret

calck_:         ; ( -- )   calibrate the ms counter clock

call rtc_hi                     ; wait for the RTC second pulse to go high

call rtc_lo                     ; wait for the RTC second pulse to go low

call get_proc_clk

call d_negate_                  ; so the d_plus_ later subtracts this value

call rtc_hi                     ; wait for the RTC second pulse to go high

call rtc_lo                     ; wait for the RTC second pulse to go low

call get_proc_clk

call d_plus_                    ; double number "subtract"

mov [ v_onesec ], _TOS_

mov _SCRATCH_, [ esi ]

mov [ v_onesec + 4 ], _SCRATCH_ ; put the result in onesec

ret

ms_:        ; ( n -- )   delay n milli seconds

_DROP_

ret

onesec_:    ; ( -- a )    return the address of the onesec variable

_DUP_

LOAD_RELATIVE_ADDRESS v_onesec ;

ret

khz_:       ; ( -- a )    return the address of the khz variable

_DUP_

LOAD_RELATIVE_ADDRESS v_khz ;

ret

font_:    ; ( -- n )    return the address of the font pointer

_DUP_

LOAD_RELATIVE_ADDRESS v_font ; font16x24

ret

last:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_lastToken

ret

blk_:       ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_blk

ret

seeb:       ; ( -- )    \ toggle the display of blue words in the editor

not byte [ v_seeb ]

ret

colourBlindModeToggle:   ; ( -- )    \ toggle the editor display colorForth / ANS style

not byte [ v_colourBlindMode ]

ret

curs:       ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_curs

ret

; analyse stack usage

; the stack areas are initialised to all 'U's at power up

; areas of 8 bytes that re not all 'U's are marked by a byte in a 512 byte buffer

; analyse_eight_bytes:    ; ( a -- a' )   zero flag is true if all 'U's

;     xor edx, edx

;     cmp dword [ _TOS_ ], 0x55555555

;     jz .forward

;         inc edx     ; not all 'U's

;     .forward:

;     inc dword _TOS_     ; next address

;     cmp dword [ _TOS_ ], 0x55555555

;     jz .forward2

;         inc edx     ; not all 'U's

;     .forward2:

;     inc dword _TOS_     ; next address

;     add edx, 0

;     ret

analyse_stacks:

mov _TOS_, STACK_MEMORY_START

mov _SCRATCH_, STACK_ANALYSIS_BUFFER

mov ecx, ( TOTAL_STACK_SIZE / 8 ) ;  0x200

.back:

; call analyse_eight_bytes

xor edx, edx

cmp dword [ _TOS_ ], 0x55555555

jz .forward

inc edx     ; not all 'U's

.forward:

add _TOS_, 4    ; next address

cmp dword [ _TOS_ ], 0x55555555

jz .forward2

inc edx     ; not all 'U's

.forward2:

add _TOS_, 4    ; next address

add edx, 0

jz .forward3

mov byte [ _SCRATCH_], 0x2E

jmp .forward4

.forward3:

mov byte [ _SCRATCH_], 0x55

.forward4:

inc _SCRATCH_   ; next address in the results buffer

loop .back

ret

stacks_:    ; ( -- a n )

call analyse_stacks

_DUP_

mov _TOS_, STACK_ANALYSIS_BUFFER

ret

_DUP_

mov _TOS_, STACK_MEMORY_START

_DUP_

mov _TOS_, TOTAL_STACK_SIZE

ret

%if 0

stacks_:   ;  ( -- a )   \ return the address of the stack memory information ( see v_stack_info for details )

;RETURN_STACK_SIZE

;DATA_STACK_SIZE

;STACK_MEMORY_START       ; bottom of stack memory

;TOTAL_STACK_SIZE

_DUP_

mov _TOS_, RETURN_STACK_0 - 0x3C           ; top of task 0 return stack

_DUP_

mov _TOS_, DATA_STACK_0   - 0x3C           ; top of task 0 data stack

_DUP_

mov _TOS_, RETURN_STACK_1 - 0x3C           ; top of task 1 return stack

_DUP_

mov _TOS_, DATA_STACK_1   - 0x3C           ; top of task 1 data stack

_DUP_

mov _TOS_, RETURN_STACK_2 - 0x3C           ; top of task 2 return stack

;    _DUP_

;    mov _TOS_, DATA_STACK_2   - 0x3C           ; top of task 2 data stack

;    LOAD_RELATIVE_ADDRESS v_stack_info

ret

%endif

ekt:   ; ( -- a ) ; editor key table - variable containing vectors for editor keys beginning with null

; and the shift keys.  Then follows right hand top, middle, bottom rows,

; and left hand top, middle, bottom rows. (from ColorForth2.0a.doc)

_DUP_

LOAD_RELATIVE_ADDRESS editorActionTable

ret

vword_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_words

ret

;vregs_:   ; ( -- a )

;    _DUP_

;    mov _TOS_, V_REGS

;    ret

ivec_:   ; ( -- a )

_DUP_

mov _TOS_, INTERRUPT_VECTORS

ret

pic_:   ; ( -- a )

_DUP_

mov _TOS_, IDT_AND_PIC_SETTINGS

ret

%if 0

From : https://pdos.csail.mit.edu/6.828/2014/readings/hardware/8259A.pdf

The following registers can be read via OCW3 (IRR and ISR or OCW1 [IMR]).

Interrupt Request Register (IRR):

8-bit register which contains the levels requesting an interrupt to be acknowledged.

The highest request level is reset from the IRR when an interrupt is acknowledged. (Not affected by IMR.)

In-Service Register (ISR):

8-bit register which contains the priority levels that are being serviced.

The ISR is updated when an End of Interrupt Command is issued.

Interrupt Mask Register:

8-bit register which contains the interrupt request lines which are masked.

The IRR can be read when, prior to the RD pulse, a Read Register Command is issued with OCW3 (RR = 1, RIS = 0.)

The ISR can be read, when, prior to the RD pulse, a Read Register Command is issued with OCW3 (RR = 1, RIS = 1).

There is no need to write an OCW3 before every status read operation,

as long as the status read corresponds with the previous one; i.e., the 8259A 'remembers' whether

the IRR or ISR has been previously selected by the OCW3.

This is not true when poll is used.

After initialization the 8259A is set to IRR.

For reading the IMR, no OCW3 is needed.

The output data bus will contain the IMR whenever RD is active and A0 = 1 (OCW1).

Polling overrides status read when P = 1, RR = 1 in OCW3.

From : https://en.wikibooks.org/wiki/X86_Assembly/Programmable_Interrupt_Controller

Remapping

Another common task, often performed during the initialization of an operating system, is remapping the PICs.

That is, changing their internal vector offsets, thereby altering the interrupt numbers they send.

The initial vector offset of PIC1 is 8, so it raises interrupt numbers 8 to 15.

Unfortunately, some of the low 32 interrupts are used by the CPU for exceptions

(divide-by-zero, page fault, etc.), causing a conflict between hardware and software interrupts.

The usual solution to this is remapping the PIC1 to start at 32, and often the PIC2 right after it at 40.

This requires a complete restart of the PICs, but is not actually too difficult, requiring just eight 'out's.

mov al, 0x11

out 0x20, al                        ; restart PIC1

out 0xA0, al                        ; restart PIC2

mov al, 0x20

out 0x21, al                        ; PIC1 now starts at 32

mov al, 0x28

out 0xA1, al                        ; PIC2 now starts at 40

mov al, 3

out 0x21, al                        ; setup cascading

mov al, 0x02

out 0xA1, al

mov al, 0x01

out 0x21, al

out 0xA1, al                        ;done!

From: cf2019 Forth block 244

: p!   pc! ;  \ 8 bit port store

: pic1! $21 p! ;

: pic2! $A1 p! ;

: !pic cli

( init )       $11 dup $20 p! $A0 p!

( irq )        $20 pic1! $28 pic2!

( master )     #4 pic1!

( slave )      #2 pic2!

( 8086 mode )  #1 dup pic1! pic2!

( mask irqs )  $FF pic2! $FA pic1! ;

Re-factored :

: !pic cli

\ PIC1

( init )     $11 $20 p!

( irq )      $20 $21 p!

( master )   $04 $21 p!

( 8086 mode) $01 $21 p!

( mask irqs) $FA $21 p!

\ PIC2

( init )     $11 $A0 p!

( irq )      $28 $A1 p!

( slave )    $02 $A1 p!

( 8086 mode) $01 $A1 p!

( mask irqs) $FF $A1 p!

;

%endif

dap_:   ; ( -- a )

_DUP_

mov _TOS_, DAP_BUFFER

ret

sect_:   ; ( -- a )

_DUP_

mov _TOS_, SECTOR_BUFFER

ret

digin:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_digin

ret

actc:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS actionColourTable

ret

tickh:   ; ( -- a )    HERE variable address

_DUP_

LOAD_RELATIVE_ADDRESS v_H

ret

md5buf_:   ; ( -- a )    \ the address of the 16 byte MD5 output buffer

_DUP_

mov _TOS_, MD5_OUTPUT_BUFFER

ret

; *****************************************************************************

; wordlist addresses and lengths

; *****************************************************************************

maca_:    ; ( -- a )  \ the address of the Macro wordlist

_DUP_

mov _TOS_,  MacroNames

ret

macn_:   ; ( -- a )   the number of words in the Macro wordlist

_DUP_

LOAD_RELATIVE_ADDRESS v_MacroWordCount

ret

macl_:    ; ( -- a )  \ the address of the Macro Locates list

_DUP_

mov _TOS_,  MacroLocates

ret

mact_:    ; ( -- a )  \ the address of the Macro Jump Table

_DUP_

mov _TOS_,  MacroJumpTable

ret

ftha_:    ; ( -- a )  \ the address of the Forth wordlist

_DUP_

mov _TOS_,  ForthNames

ret

fthn_:   ; ( -- a )   the number of words in the Forth wordlist

_DUP_

LOAD_RELATIVE_ADDRESS v_ForthWordCount

ret

fthl_:    ; ( -- a )  \ the address of the Forth Locates list

_DUP_

mov _TOS_,  ForthLocates

ret

ftht_:    ; ( -- a )  \ the address of the Forth Jump Table

_DUP_

mov _TOS_,  ForthJumpTable

ret

; the blue wordlist is not extensible, at the moment

blua_:    ; ( -- a )  \ the address of the Blue wordlist

_DUP_

mov _TOS_,  BlueNames

ret

blun_:   ; ( -- a )   the number of words in the Blue wordlist

_DUP_

LOAD_RELATIVE_ADDRESS v_BlueWordCount

ret

; *****************************************************************************

; loc , see and fnd

; *****************************************************************************

loc_:   ; ( token# -- a )

mov dword _TOS_, [ ( _TOS_ * 4 ) + ForthLocates ]

; ret

cmp _TOS_, 0x0000000

jz .forward                 ; do nothing is the locate field is 0

jmp editAddress_

.forward:

ret

see_:   ; ( token# -- a )

mov dword _TOS_, [ ( _TOS_ * 4 ) + ForthJumpTable ]

; call dmp_

ret

fnd_:       ; ( sf -- token# )

and _TOS_, byte -0x10 ; (saves 2 bytes compared to 'and _TOS_, 0xFFFFFFF0' )

call find_

mov _TOS_, 0x00000000

jnz .skip

mov _TOS_, ecx

.skip:

ret

dmp_:

mov _TOS_, [ ( _TOS_ * 4 ) + ForthNames ]

ret

; *****************************************************************************

; *****************************************************************************

offset_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_offset

ret

vesa:   ; ( -- a )

_DUP_

mov _TOS_, VESA_BUFFER

ret

vesamode_:   ; ( -- u )

_DUP_

xor _TOS_, _TOS_

mov word ax, [ vesa_SavedMode ]     ; the saved  VESA video mode value

ret

fetchDX_:   ; ( -- c )

_DUP_

xor _TOS_, _TOS_

push edi

mov edi, DAP_BUFFER

mov _TOS_l_, [ edi + o_Int13_DAP_saved_DX ]     ; setup DX value returned by the BIOS

pop edi

ret

trash_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_trash

ret

buffer_:   ; ( -- a )

_DUP_

mov _TOS_, SECTOR_BUFFER ;0x25300

ret

cad:   ; ( -- a )   \ the address of the cursor as an offset from the start of the currently displayed block

_DUP_

LOAD_RELATIVE_ADDRESS v_cad

ret

pcad:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS v_pcad

ret

; hsvv_:   ; ( -- a )

;     _DUP_

;     LOAD_RELATIVE_ADDRESS hsvv

;     ret

displ:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS displayShannonFanoActions

ret

cBlindAddr_:   ; ( -- a )

_DUP_

LOAD_RELATIVE_ADDRESS x_colourBlind

ret

; *****************************************************************************

; memory operators

; *****************************************************************************

cFetch_:      ; ( a -- c ) \ c@

xor _SCRATCH_, _SCRATCH_

mov byte _SCRATCH_l_, [ _TOS_ ] ;

mov _TOS_, _SCRATCH_

ret

wFetch_:      ; ( a -- w ) \ w@

xor _SCRATCH_, _SCRATCH_

mov word _SCRATCH_x_, [ _TOS_ ] ;

mov _TOS_, _SCRATCH_

ret

fetch_:       ; ( a -- u ) \ @

mov dword _TOS_, [ _TOS_ ] ;

ret

two_fetch_:     ; ( a -- x1 x2 )

sub esi, 4                       ; make room on stack

mov _SCRATCH_, [ _TOS_ + 4 ]     ; read x1 from addr+4

mov [ esi ], _SCRATCH_           ; write onto stack

mov _TOS_, [ _TOS_ ]             ; read x2 from addr+0, replacing tos

ret

cStore_:      ; ( c a -- )          \ c!

mov _SCRATCH_, [ esi ]

mov byte [ _TOS_ ], _SCRATCH_l_

_DROP_

_DROP_

ret

wStore_:      ; ( w a -- )          \ w!

mov _SCRATCH_, [ esi ]

mov word [ _TOS_ ], _SCRATCH_x_

_DROP_

_DROP_

ret

store_:       ; ( u a -- )          \ !

mov _SCRATCH_, [ esi ]

mov dword [ _TOS_ ], _SCRATCH_

_DROP_

_DROP_

ret

two_store_:     ; ( x1 x2 a -- )    \ 2!

mov _SCRATCH_, [ esi ]          ; x2 into scratch

mov [ _TOS_ ], _SCRATCH_        ; write x2 to addr+0

mov _SCRATCH_, [ esi + 4 ]      ; x1 into scratch

mov [ _TOS_ + 4 ], _SCRATCH_    ; write x1 to addr+4

_DROP_                          ; drop the stack

_DROP_

_DROP_

ret

plus_store_:  ; ( n addr -- )       \ +!

mov _SCRATCH_, [ esi ]          ; copy the value n into the scratch register

add [ _TOS_ ], _SCRATCH_        ; add to value at addr

_DROP_                          ; drop the stack

_DROP_

ret

; *****************************************************************************

; double number operators

; *****************************************************************************

d_negate_:  ; ( d1 -- d2 )

not dword [ esi ]                ; invert d1-lo

not _TOS_                        ; invert d1-hi

add dword [ esi ], 1             ; make two's complement

adc _TOS_, 0                     ; from invert + 1

ret

d_plus_:   ; ( d1 d2 -- d3 )   add d2 to d1 to give d3

mov _SCRATCH_, [ esi ]           ; get d2-lo

add _SCRATCH_, [ esi + 8 ]       ; add d1-lo

adc _TOS_, [ esi + 4 ]           ; add d1-hi and carry to d2-hi

mov [ esi + 8 ], _SCRATCH_       ; write d3-low

add esi, 8                       ; and clean up stack

ret

d_minus_:   ; ( d1 d2 -- d3 )   subtract d2 from d1 to give d3

call d_negate_

call d_plus_

ret

; *****************************************************************************

; stack operators

; *****************************************************************************

two_dup_:       ; ( a b -- a b a b )

;    sub esi, byte 0x08 ; lea esi, [ esi - 0x08 ]    ; pre-decrement the stack pointer, adding 2 cells

;    mov [ esi + 4 ], _TOS_  ; copy x2 to Third On Stack ( second on the real stack )

;    mov _SCRATCH_, [ esi + 8 ]    ; copy x1 to register ebx

;    mov [ esi ], _SCRATCH_        ; copy register ebx to Fourth On Stack

_OVER_

_OVER_

ret

two_drop_:      ; ( a b -- )

_DROP_

_DROP_

ret

two_swap_:      ; ( a b c d -- c d a b )

mov _SCRATCH_, [ esi + 8 ]

xchg _SCRATCH_, [ esi ]

mov  [ esi + 8 ], _SCRATCH_

xchg _TOS_, [ esi + 4 ]

ret

two_over_:      ; ( a b c d -- a b c d a b )

lea esi, [ esi - 8 ]

mov [ esi + 4 ], _TOS_

mov _SCRATCH_, [ esi + 0x10 ]

mov [esi], _SCRATCH_

mov _TOS_, [ esi + 0x0C ]

ret

rot_:           ; ( a b c -- b c a)

mov _SCRATCH_,[ esi + 4 ]

mov ebp, [ esi ]

mov [ esi + 4 ], ebp

mov [ esi ],_TOS_

mov _TOS_, _SCRATCH_

ret

minus_rot_:     ; -rot ( a b c -- c b a)

mov _SCRATCH_, [ esi + 4 ]

mov ebp, [ esi ]

mov [ esi + 4 ], _TOS_

mov [esi], _SCRATCH_

mov _TOS_, ebp

ret

tuck_:          ; ( a b -- b a b )

_SWAP_

_OVER_

ret

pick_:          ; ( ... n -- ... u ) where u is the n'th stack item

mov _TOS_, [ esi + ( _TOS_ * 4 ) ]

ret

%define CELL_WIDTH  0x04    ; this is a 32 bit wide system = 4 bytes

cell_:          ; ( -- c )

_DUP_

mov _TOS_, CELL_WIDTH

ret

cell_minus_:    ; ( u -- u' )

sub _TOS_, CELL_WIDTH

ret

cell_plus_:    ; ( u -- u' )

add _TOS_, CELL_WIDTH

ret

cells_:         ; ( u -- u' )

add _TOS_, _TOS_    ; this code must be changed if CELL_WIDTH is changed

add _TOS_, _TOS_

ret

; *****************************************************************************

; save and restore the Interrupt Descriptor Table and Interrupt Mask Registers

; *****************************************************************************

lidt_:  ; ( a -- )  \ set a into the Interrupt Descriptor Table (IDT) register

cli

push ebp

mov ebp, ( PIC_BIOS_IDT_SETTINGS )   ; 6 bytes of RAM used to store the IDT info

mov word [ ebp ], 0x03B7

mov [ ebp + 2 ], _TOS_   ; save IDT base address from eax

lidt [ ebp ]  ; db 0x0F, 0x01, 0x18

_DROP_

pop ebp

ret

sidt_:  ; ( -- a )  \ return the address contained in the Interrupt Descriptor Table (IDT) register

cli

_DUP_

push ebp

mov ebp, ( IDT_AND_PIC_SETTINGS_PAD )   ; 6 bytes of RAM used to interface to the stack

sidt [ ebp ]        ; write the 6-byte IDT to memory location pointed to by  ebp

mov _TOS_, [ ebp + 2 ]   ; save IDT base address to eax

pop ebp

ret

save_BIOS_idt:  ; ( -- )  \ save the Interrupt Descriptor Table (IDT) register value

cli

push ebp

mov ebp, ( PIC_BIOS_IDT_SETTINGS )   ; 6 bytes of RAM used to save the values in

sidt [ ebp ]        ; write the 6-byte IDT to memory location pointed to by  ebp

pop ebp

ret

restore_BIOS_idt:  ; ( -- )  \ restore the saved IDT value into the Interrupt Descriptor Table (IDT) register

cli

push ebp

mov ebp, ( PIC_BIOS_IDT_SETTINGS )   ; 6 bytes of RAM used to restore from

lidt [ ebp ]  ; db 0x0F, 0x01, 0x18

pop ebp

ret

save_BIOS_idt_and_pic: ; ( -- )   \ save the PIC1 and PIC2 IMR values into IDT_AND_PIC_SETTINGS at startup

cli

call save_BIOS_idt

push ebp

mov ebp, ( PIC_BIOS_IMR_SETTINGS )   ; 2 bytes of RAM used to save the IMR for PIC1 and PIC2

; PIC1

in al, 0x21     ; read PIC1's IMR value

mov [ ebp ], al

; PIC2

inc ebp

in al, 0xA1     ; read PIC 2's IMR value

mov [ ebp ], al

pop ebp

ret

restore_BIOS_idt_and_pic: ; ( -- )   \ restore the saved BIOS PIC and IMR values into PIC1 and PIC2

cli

call restore_BIOS_idt

push ebp

mov ebp, ( PIC_BIOS_IMR_SETTINGS )   ; 2 bytes of RAM used to save the IMR for PIC1 and PIC2

; PIC1

mov al, 0x11    ; init command

out 0x20, al    ; init PIC1                 ( $11 $20 p! )

mov al, 0x00    ; PIC1 Interrupt Vector table start address

out 0x21, al    ; PIC1 now starts at 0x00   ( $00 $21 p! )

mov al, 0x04    ; master mode command

out 0x21, al    ; set PIC1 as master, sets up cascading of PIC1 and PIC2 ( $04 $21 p! )

mov al, 0x01    ; 8086 command

out 0x21, al    ; set 8086 mode         ( $01 $21 p! )

mov al, [ ebp ] ; Interrupt Mask Register ( IMR )

out 0x21, al    ; set PIC1's IMR, BIOS = 0xB8 ( $xx $21 p! )

; PIC2

inc ebp

mov al, 0x11    ; init command

out 0xA0, al    ; init PIC2

mov al, 0x08    ; PIC2 Interrupt Vector table start address

out 0xA1, al    ; PIC2 now starts at 0x08   $08 $A1 p!

mov al, 0x02    ; slave mode command

out 0xA1, al    ; set PIC2 as slave     ( $02 $A1 p! )

mov al, 0x01    ; 8086 command

out 0xA1, al    ; set 8086 mode         ( $01 $A1 p! )

mov al, [ ebp ] ; Interrupt Mask Register ( IMR )

out 0xA1, al    ; set PIC2's IMR, BIOS = 0x8F ( $xx $A1 p! )

pop ebp

ret

restore_new_idt_and_pic: ; ( -- )   \ restore the new IDT and PIC IMR values

cli

push ebp

mov ebp, ( PIC_NEW_IMR_SETTINGS )   ; 2 bytes of RAM used to save the IMR for PIC1 and PIC2

; PIC1

mov al, 0x11    ; init command

out 0x20, al    ; init PIC1                 ( $11 $20 p! )

mov al, 0x20    ; PIC1 Interrupt Vector table start address

out 0x21, al    ; PIC1 now starts at 0x20   ( $20 $21 p! )

mov al, 0x04    ; master mode command

out 0x21, al    ; set PIC1 as master, sets up cascading of PIC1 and PIC2 ( $04 $21 p! )

mov al, 0x01    ; 8086 command

out 0x21, al    ; set 8086 mode         ( $01 $21 p! )

mov al, [ ebp ] ; Interrupt Mask Register ( IMR )

out 0x21, al    ; set PIC1's IMR, BIOS = 0xB8 ( $xx $21 p! )

; PIC2

inc ebp

mov al, 0x11    ; init command

out 0xA0, al    ; init PIC2

mov al, 0x28    ; PIC2 Interrupt Vector table start address

out 0xA1, al    ; PIC2 now starts at 0x28   $28 $A1 p!

mov al, 0x02    ; slave mode command

out 0xA1, al    ; set PIC2 as slave     ( $02 $A1 p! )

mov al, 0x01    ; 8086 command

out 0xA1, al    ; set 8086 mode         ( $01 $A1 p! )

mov al, [ ebp ] ; Interrupt Mask Register ( IMR )

out 0xA1, al    ; set PIC2's IMR, BIOS = 0x8F ( $xx $A1 p! )

pop ebp

ret

init_default_PIC_IMRs:  ; ( -- )

pushf

cli

pusha

mov esi, 0x0000                 ; source address = the BIOS interrupt vector table

mov edi, INTERRUPT_VECTORS      ; destination address

mov ecx, ( 1024 / 4 )           ; 1024 bytes in cells

rep movsd                       ; does not change AX , it moves DS:SI to ES:DI and increments SI and DI

; now copy Interrupts 0x00 to 0x0F up to 0x20 to 0x2F

mov esi, 0x0000                 ; source address = the BIOS interrupt vector table

mov edi, ( INTERRUPT_VECTORS + ( 0x20 * 4 ) )  ; destination address

mov ecx, ( 0x10 )               ; 16 vectors in cells

rep movsd                       ; does not change AX , it moves DS:SI to ES:DI and increments SI and DI

popa

push ebp

mov ebp, ( PIC_NEW_IMR_SETTINGS )   ; 2 bytes of RAM used to save the IMR for PIC1 and PIC2

mov byte [ ebp ] , 0xFA   ; Interrupt Mask Register ( IMR ) saved value for PIC1

inc ebp

mov byte [ ebp ] , 0xFF   ; Interrupt Mask Register ( IMR ) saved value for PIC2

pop ebp

popf

ret

set_PIC1_IMR:   ; ( c -- )   \ set the Interrupt Mask Register for PIC1 and copy to PIC_NEW_IMR_SETTINGS

pushf

cli

push ebp

mov ebp, ( PIC_NEW_IMR_SETTINGS )   ; 1 byte of RAM used to save the IMR for PIC1

mov [ ebp ] , al    ; Interrupt Mask Register ( IMR )

out 0x21, al        ; set PIC1's IMR ( $xx $21 p! )

pop ebp

popf

_DROP_

ret

set_PIC2_IMR:   ; ( c -- )   \ set the Interrupt Mask Register for PIC2 and copy to PIC_NEW_IMR_SETTINGS+1

pushf

cli

push ebp

mov ebp, ( PIC_NEW_IMR_SETTINGS + 1 )   ; 1 byte of RAM used to save the IMR for PIC1

mov [ ebp ] , al    ; Interrupt Mask Register ( IMR )

out 0xA1, al        ; set PIC2's IMR ( $xx $A1 p! )

pop ebp

popf

_DROP_

ret

; *****************************************************************************

; lp support for GRaphics demo

; *****************************************************************************

lp_:

;; test what lodsd actually does

;    push esi

;    mov esi, [ v_trash ]    ; setup EDI to point to the current trash buffer address

;    mov [ v_lcad ], esi

;    lodsd  ; loads a 32 bit dword from [ds:esi] into _TOS_, increments  esi  by 4 : true

;    mov [ v_pcad ], esi

;    pop esi

;    _DUP_

;    mov _TOS_, [ v_lcad ]

;    _DUP_

;    mov _TOS_, [ v_pcad ]

;    ret

nop

nop

nop

db 0x8B , 0xE8 ; mov ebp,eax

lodsd  ; loads a 32 bit dword from [ds:esi] into _TOS_, increments  esi  by 4

db 0x8B , 0xC8 ; mov ecx,eax

lodsd  ; loads a 32 bit dword from [ds:esi] into _TOS_, increments  esi  by 4

mov ebx,[edx+0x20]

.back:

mov [ebx],bp

db 0x23 , 0xC0 ; and eax,eax   21C0                        and eax,eax

js .forward

add eax,[edx]

add ebx,[edx+0x18]

.forward:

add eax,[edx+0x8]

add ebx,[edx+0x10]

loop .back

;    dd 0x8B909090 , 0xC88BADE8 , 0x205A8BAD , 0x232B8966

;    dd 0x030578C0 , 0x185A0302 , 0x03084203 , 0xECE2105A

ret

; *****************************************************************************

; maths operators

; The ANSI/ISO Forth Standard (adopted in 1994) mandates the minimal set

; of arithmetic operators +  -  *  /  MOD  */  /MOD  */MOD and  M* .

; *****************************************************************************

two_slash_:                         ; ( n -- n' )   "2/" arithmetic divide by 2

sar _TOS_, 0x01

ret

u_two_slash_:                       ; ( u -- u' )   "u2/" unsigned divide by 2

shr _TOS_, 0x01

ret

rshift_:                            ; ( u c -- u' )   shift TOS right by c bits

mov ecx, _TOS_

_DROP_

shr _TOS_, cl

ret

lshift_:                            ; ( u c -- u' )   shift TOS left by c bits

mov ecx, _TOS_

_DROP_

shl _TOS_, cl

ret

%if 0

; untested!!!

mod_:                               ; ( n1 n2 -- n3 )

mov _TOS_, [ esi ]               ; get dividend

cdq                              ; sign extend dividend

idiv _SCRATCH_                   ; do the divide

add esi, 4                       ; clean up stack

mov _TOS_, edx                   ; and return remainder in tos

ret

%endif

; "idiv ecx" divides the signed double dividend EDX:EAX, by the divisor in ECX

; and stores the remainder in EDX and quotient in EAX

slash_mod_:                         ; /mod ( n1 n2 -- r q )

mov ecx, _TOS_                   ; get n2 the divisor

mov _TOS_, [ esi ]               ; get n1 the dividend

cdq                              ; sign extend into edx

idiv ecx                         ; do the divide

mov [ esi ], edx                 ; and remainder to stack

ret

; "imul ecx" multiplies ECX by EAX and stores the result in EDX:EAX

; ToDo: fix and test this properly...

star_slash_mod_:                    ; */mod ( n1 n2 n3 -- r q )

push _TOS_

mov ecx, [ esi ]                 ; get n2

mov _TOS_, [ esi + 4 ]           ; get n1

imul ecx                         ; n1*n2 => edx:eax

add esi, 4                       ; clean up stack

pop ecx

idiv ecx                         ; n1*n2/n3

mov [ esi ], edx                 ; remainder to stack

ret

star_slash_:                        ; */ ( n1 n2 n3 -- n )

push _TOS_

mov ecx, [ esi ]                 ; get n2

mov _TOS_, [ esi + 4 ]           ; get n1

imul ecx                         ; n1*n2 => edx:eax

add esi, 8                       ; clean up stack

pop ecx

idiv ecx                         ; n1*n2/n3

ret

; U*/  is an unsigned  */  with the twist of rounding up

; It adds one less than the divisor ( u3 ) to the dividend before dividing

; ToDo: fix and test this properly...

u_star_slash_:                      ; U*/ ( u1 u2 u3 -- u )

mov _SCRATCH_, _TOS_

dec _SCRATCH_                    ; divisor

mov ecx, [ esi ]                 ; get n2

mov edx, [ esi + 4 ]             ; get n1

mul ecx                          ; u1 * u2

add _TOS_, _SCRATCH_               ; round up

adc edx, 0                       ;

inc _SCRATCH_                    ; restore the original u3 divisor

div _SCRATCH_                    ; do the division

add esi, 8                       ; clean up stack

ret

cmove_:                             ; ( from to count -- )

test _TOS_, _TOS_

jz .forward

mov _SCRATCH_, _TOS_

mov edx, [ esi + 0 ]

mov ecx, [ esi + 0x04 ]

.back:

mov byte al, [ ecx + 0 ]

mov byte [ edx + 0 ], al

inc ecx

inc edx

dec _SCRATCH_

jnz .back

.forward:

mov _TOS_, [ esi + 0x08 ]

add esi, 0x0C

ret

two_star_:                          ; 2* ( u -- u' )    u' = 2 * u

shl _TOS_, 1

ret

two_star_star_:                     ; 2** ( c -- u )    u  = 2 ** c

mov ecx, _TOS_

mov _TOS_, 0x00000001

shl _TOS_, cl

ret

; *****************************************************************************

; Random and Pseudo Random Number Generators

; *****************************************************************************

GetCPUIDsupport:    ; ( -- )  equal flag is set if no CPUID support

; check to see if CPUID is supported

pushfd              ; save EFLAGS

pop eax             ; store EFLAGS in EAX

mov ebx, eax        ; save in EBX for later testing

xor eax, 00200000h  ; toggle bit 21

push eax            ; push to stack

popfd               ; save changed EAX to EFLAGS

pushfd              ; push EFLAGS to TOS

pop eax             ; store EFLAGS in EAX

cmp eax, ebx        ; see if bit 21 has changed

ret

GetRDRANDsupport:   ; zero flag is set if no support for RDRAND, the hardware Random Number generator

mov _TOS_, 0x00000001     ; select the 'features' CPU information

CPUID           ; get CPU information into eax, ebx, ecx and edx

test eax, 0x40000000 ; Bit 30 of ECX returned by CPUID => RDRAND present if true

ret

GetCPUID_:   ; ( -- u )

_DUP_

mov _TOS_, 0x00000001     ; select the 'features' CPU information

CPUID           ; get CPU information into eax, ebx, ecx and edx

ret

rdtsc_:  ; ( -- u )   \ return the current processor instruction counter

_DUP_

rdtsc ; db 0x0F, 0x31

ret

randInit_:

call rdtsc_

push ebp

mov ebp, v_random

xor [ ebp ], _TOS_      ; vRandom ! , if the value was 0

pop ebp

_DROP_

ret

%if 0

\ Marsaglia, "Xorshift RNGs".  http://www.jstatsoft.org/v08/i14/paper

: Random32 ( -- u )

vRandom @

dup 0= or

dup 6 lshift xor

dup 21 rshift xor

dup 7 lshift xor

dup vRandom ! ;

%endif

; \ Marsaglia, "Xorshift RNGs".  http://www.jstatsoft.org/v08/i14/paper

getRandMarsaglia: ; ( -- u )   \ load a 32 bit pseudo random number into TOS

_DUP_

push ebp

mov ebp, v_random

mov _TOS_, [ ebp ]      ; vRandom @

test _TOS_, _TOS_

jnz .forward            ; dup 0= or

mov _TOS_, 0xFFFFFFFF

.forward:

mov _SCRATCH_, _TOS_    ; dup 6 lshift xor

shl _SCRATCH_, 0x06

xor _TOS_, _SCRATCH_

mov _SCRATCH_, _TOS_    ; dup 21 rshift xor

shr _SCRATCH_, 0x15

xor _TOS_, _SCRATCH_

mov _SCRATCH_, _TOS_    ; dup 7 lshift xor

shl _SCRATCH_, 0x07

xor _TOS_, _SCRATCH_

mov [ ebp ], _TOS_      ; vRandom !

pop ebp

ret

rand_:  ; ( -- u )   \ load a 32 bit true random number into TOS

_DUP_

call GetCPUIDsupport

je .NO_CPUID             ; if no change to bit 21, no CPUID

; CPUID is supported, so check if RDRAND is supported

call GetRDRANDsupport

jz .NO_CPUID         ; test for RDRAND support

RDRAND _TOS_    ; supported, so call the instruction

ret

.NO_CPUID:

_DROP_

call getRandMarsaglia

ret

randq_:     ; ( -- f )   \ returns true if the processor supports the RDRAND random number instruction

_DUP_

call GetCPUIDsupport

jz .NO_CPUID             ; if no change, no CPUID

; CPUID is supported, so check if RDRAND is supported

call GetRDRANDsupport

jz .NO_CPUID         ; test for RDRAND support

mov _TOS_, 0xFFFFFFFF

ret

.NO_CPUID:

xor _TOS_, _TOS_

ret

; *****************************************************************************

; CRC32 Cyclic Redundancy Checksum (32 bit)

; The International Standard 32-bit cyclical redundancy check defined by :

; [ITU-T-V42] International Telecommunications Union, "Error-correcting

; Procedures for DCEs Using Asynchronous-to-Synchronous Conversion",

; ITU-T Recommendation V.42, 1994, Rev. 1.

; and

; [ISO-3309]

; International Organization for Standardization,

; "Information Processing Systems--Data Communication High-Level Data Link

; Control Procedure--Frame Structure", IS 3309, October 1984, 3rd Edition.

; *****************************************************************************

crc32_table:

dd 000000000h, 077073096h, 0EE0E612Ch, 0990951BAh, 0076DC419h, 0706AF48Fh, 0E963A535h, 09E6495A3h, 00EDB8832h, 079DCB8A4h

dd 0E0D5E91Eh, 097D2D988h, 009B64C2Bh, 07EB17CBDh, 0E7B82D07h, 090BF1D91h, 01DB71064h, 06AB020F2h, 0F3B97148h, 084BE41DEh

dd 01ADAD47Dh, 06DDDE4EBh, 0F4D4B551h, 083D385C7h, 0136C9856h, 0646BA8C0h, 0FD62F97Ah, 08A65C9ECh, 014015C4Fh, 063066CD9h

dd 0FA0F3D63h, 08D080DF5h, 03B6E20C8h, 04C69105Eh, 0D56041E4h, 0A2677172h, 03C03E4D1h, 04B04D447h, 0D20D85FDh, 0A50AB56Bh

dd 035B5A8FAh, 042B2986Ch, 0DBBBC9D6h, 0ACBCF940h, 032D86CE3h, 045DF5C75h, 0DCD60DCFh, 0ABD13D59h, 026D930ACh, 051DE003Ah

dd 0C8D75180h, 0BFD06116h, 021B4F4B5h, 056B3C423h, 0CFBA9599h, 0B8BDA50Fh, 02802B89Eh, 05F058808h, 0C60CD9B2h, 0B10BE924h

dd 02F6F7C87h, 058684C11h, 0C1611DABh, 0B6662D3Dh, 076DC4190h, 001DB7106h, 098D220BCh, 0EFD5102Ah, 071B18589h, 006B6B51Fh

dd 09FBFE4A5h, 0E8B8D433h, 07807C9A2h, 00F00F934h, 09609A88Eh, 0E10E9818h, 07F6A0DBBh, 0086D3D2Dh, 091646C97h, 0E6635C01h

dd 06B6B51F4h, 01C6C6162h, 0856530D8h, 0F262004Eh, 06C0695EDh, 01B01A57Bh, 08208F4C1h, 0F50FC457h, 065B0D9C6h, 012B7E950h

dd 08BBEB8EAh, 0FCB9887Ch, 062DD1DDFh, 015DA2D49h, 08CD37CF3h, 0FBD44C65h, 04DB26158h, 03AB551CEh, 0A3BC0074h, 0D4BB30E2h

dd 04ADFA541h, 03DD895D7h, 0A4D1C46Dh, 0D3D6F4FBh, 04369E96Ah, 0346ED9FCh, 0AD678846h, 0DA60B8D0h, 044042D73h, 033031DE5h

dd 0AA0A4C5Fh, 0DD0D7CC9h, 05005713Ch, 0270241AAh, 0BE0B1010h, 0C90C2086h, 05768B525h, 0206F85B3h, 0B966D409h, 0CE61E49Fh

dd 05EDEF90Eh, 029D9C998h, 0B0D09822h, 0C7D7A8B4h, 059B33D17h, 02EB40D81h, 0B7BD5C3Bh, 0C0BA6CADh, 0EDB88320h, 09ABFB3B6h

dd 003B6E20Ch, 074B1D29Ah, 0EAD54739h, 09DD277AFh, 004DB2615h, 073DC1683h, 0E3630B12h, 094643B84h, 00D6D6A3Eh, 07A6A5AA8h

dd 0E40ECF0Bh, 09309FF9Dh, 00A00AE27h, 07D079EB1h, 0F00F9344h, 08708A3D2h, 01E01F268h, 06906C2FEh, 0F762575Dh, 0806567CBh

dd 0196C3671h, 06E6B06E7h, 0FED41B76h, 089D32BE0h, 010DA7A5Ah, 067DD4ACCh, 0F9B9DF6Fh, 08EBEEFF9h, 017B7BE43h, 060B08ED5h

dd 0D6D6A3E8h, 0A1D1937Eh, 038D8C2C4h, 04FDFF252h, 0D1BB67F1h, 0A6BC5767h, 03FB506DDh, 048B2364Bh, 0D80D2BDAh, 0AF0A1B4Ch

dd 036034AF6h, 041047A60h, 0DF60EFC3h, 0A867DF55h, 0316E8EEFh, 04669BE79h, 0CB61B38Ch, 0BC66831Ah, 0256FD2A0h, 05268E236h

dd 0CC0C7795h, 0BB0B4703h, 0220216B9h, 05505262Fh, 0C5BA3BBEh, 0B2BD0B28h, 02BB45A92h, 05CB36A04h, 0C2D7FFA7h, 0B5D0CF31h

dd 02CD99E8Bh, 05BDEAE1Dh, 09B64C2B0h, 0EC63F226h, 0756AA39Ch, 0026D930Ah, 09C0906A9h, 0EB0E363Fh, 072076785h, 005005713h

dd 095BF4A82h, 0E2B87A14h, 07BB12BAEh, 00CB61B38h, 092D28E9Bh, 0E5D5BE0Dh, 07CDCEFB7h, 00BDBDF21h, 086D3D2D4h, 0F1D4E242h

dd 068DDB3F8h, 01FDA836Eh, 081BE16CDh, 0F6B9265Bh, 06FB077E1h, 018B74777h, 088085AE6h, 0FF0F6A70h, 066063BCAh, 011010B5Ch

dd 08F659EFFh, 0F862AE69h, 0616BFFD3h, 0166CCF45h, 0A00AE278h, 0D70DD2EEh, 04E048354h, 03903B3C2h, 0A7672661h, 0D06016F7h

dd 04969474Dh, 03E6E77DBh, 0AED16A4Ah, 0D9D65ADCh, 040DF0B66h, 037D83BF0h, 0A9BCAE53h, 0DEBB9EC5h, 047B2CF7Fh, 030B5FFE9h

dd 0BDBDF21Ch, 0CABAC28Ah, 053B39330h, 024B4A3A6h, 0BAD03605h, 0CDD70693h, 054DE5729h, 023D967BFh, 0B3667A2Eh, 0C4614AB8h

dd 05D681B02h, 02A6F2B94h, 0B40BBE37h, 0C30C8EA1h, 05A05DF1Bh, 02D02EF8Dh

; CRC-32 with polynomial $04c11db7, as specified in IEEE 802.3 ( Ethernet )

crc32_:    ; ( a n -- u )   \ CRC32 Cyclic Redundancy Checksum

push    _SCRATCH_

push    ecx

push    edx

mov ecx, _TOS_

_DROP_

mov _SCRATCH_, _TOS_

; address in ebx, count in ecx, result in eax

xor edx, edx

mov  _TOS_, 0xFFFFFFFF     ; initial CRC value

test ecx, ecx

jz .forward

.back:

mov dl, byte [_SCRATCH_]

xor dl, al

shr _TOS_, 8

xor _TOS_, dword [ crc32_table + ( 4 * edx ) ]

inc _SCRATCH_

dec ecx

jnz .back

not _TOS_     ; invert the final CRC value

.forward:

pop edx

pop ecx

pop _SCRATCH_

ret

; *****************************************************************************

; MD5

; *****************************************************************************

; From : https://github.com/rwfpl/rewolf-md5/blob/master/nasm/rewolf_md5.inc

;----------------------------------------------------------------------------

;|                     The MD5 Message-Digest Algorithm                     |

;----------------------------------------------------------------------------

;|   Description:                                                           |

;|   ============                                                           |

;|                                                                          |

;|   The MD5 algorithm is designed to be quite fast on 32-bit machines. In  |

;|   addition,  the MD5 algorithm  does not require any large substitution  |

;|   tables, the algorithm can be coded quite compactly.                    |

;|                                                                          |

;|   The MD5 algorithm is an extension of the MD4 message-digest algorithm  |

;|   1,2]. MD5 is  slightly slower than MD4, but is more "conservative" in  |

;|   design. MD5  was designed  because it  was felt  that MD4 was perhaps  |

;|   being adopted  for use more  quickly than  justified by  the existing  |

;|   critical  review, because MD4  was designed to be exceptionally fast,  |

;|   it  is "at the edge"  in terms of  risking  successful  cryptanalytic  |

;|   attack.  MD5 backs off  a bit, giving up a little in speed for a much  |

;|   greater   likelihood  of  ultimate  security.  It  incorporates  some  |

;|   suggestions  made  by  various  reviewers,  and  contains  additional  |

;|   optimizations. The MD5 algorithm is being placed in the public domain  |

;|   for review and possible adoption as a standard.                        |

;|                                                                          |

;----------------------------------------------------------------------------

;|   Implementation based on rfc1321 (fully rewritten in asm, not ripped :))|

;----------------------------------------------------------------------------

;|   Usage:                                                                 |

;|   ======                                                                 |

;|                                                                          |

;|   Simply include this file to your project:                              |

;|   exp: include \..path..\rewolf_md5.inc                                  |

;|                                                                          |

;|   Target compiler...: NASM-YASM                                          |

;|   Calling convention:                                                    |

;|                                                                          |

;|       push    size of datablock                                          |

;|       push    datablock                                                  |

;|       push    destHash                                                   |

;|       call    _rwf_md5                                                   |

;|                                                                          |

;|   datablock -> (input)  -> buffer that contains data to hash             |

;|   destHash  -> (output) -> 16-bytes buffer for hashed data               |

;|                                                                          |

;|   Modified registers: none                                               |

;|   Stack is automatically cleared                                         |

;----------------------------------------------------------------------------

;|   Coder.: ReWolf^HTB                                                     |

;|   Date..: 17.XII.2004                                                    |

;|   E-mail: rewolf@poczta.onet.pl                                          |

;|   WWW...: http://www.rewolf.prv.pl                                       |

;----------------------------------------------------------------------------

;|   Adaptation for NASM/YASM:  Ange Albertini                              |

;----------------------------------------------------------------------------

S11 equ 7

S12 equ 12

S13 equ 17

S14 equ 22

S21 equ 5

S22 equ 9

S23 equ 14

S24 equ 20

S31 equ 4

S32 equ 11

S33 equ 16

S34 equ 23

S41 equ 6

S42 equ 10

S43 equ 15

S44 equ 21

%macro FF 7 ;a,b,c,d,k,s,i

mov edi,%2

mov ebp,%2

and edi,%3

not ebp

and ebp,%4

or  edi,ebp

lea %1, [%1+edi+%7]

add %1, dword [esi+%5*4]

rol %1,%6

add %1,%2

%endmacro

%macro GG 7

mov edi,%4

mov ebp,%4

and edi,%2

not ebp

and ebp,%3

or  edi,ebp

lea %1, [%1+edi+%7]

add %1, dword [esi+%5*4]

rol %1,%6

add %1,%2

%endmacro

%macro  HH  7

mov ebp,%2

xor ebp,%3

xor ebp,%4

lea %1, [%1+ebp+%7]

add %1,dword [esi+%5*4]

rol %1,%6

add %1,%2

%endmacro

%macro  II  7

mov ebp,%4

not ebp

or  ebp,%2

xor ebp,%3

lea %1, [%1+ebp+%7]

add %1,dword [esi+%5*4]

rol %1,%6

add %1,%2

%endmacro

;|       push    size of datablock                                          |

;|       push    datablock                                                  |

;|       push    destHash

md5_:    ; ( a n -- md5_address )

push _TOS_      ; [ a -- ]

_DROP_

push _TOS_  ; [ a n -- ]

mov _SCRATCH_, MD5_OUTPUT_BUFFER

push _SCRATCH_  ; [ a n md5_output -- ]

call _rwf_md5

mov _TOS_, MD5_OUTPUT_BUFFER

ret

_rwf_md5:   ; ( a n outputPtr 0 -- )

pushad

mov esi,dword [esp+04h+8*4]

mov dword [esi], 067452301h

mov dword [esi+04h], 0efcdab89h

mov dword [esi+08h], 098badcfeh

mov dword [esi+0Ch], 010325476h

mov eax,dword [esp+0Ch+8*4]

push eax

xor edx,edx

mov ecx,64

div ecx

inc eax

pop edx

sub esp,64

mov ebx,esp

mov esi,dword [esp+08h+24*4]

xchg    eax,edx

_n0:

mov edi,ebx

dec edx

jne _n1

test eax,eax

js  _nD

mov byte [ebx+eax],80h

jmp _nC

_nD:

xor eax,eax

dec eax

_nC:

mov ecx,64

sub ecx,eax

add edi,eax

push eax

xor eax,eax

inc edi

dec ecx

rep stosb

pop eax

test eax,eax

js  _nB

cmp eax,56

jnb _nE

_nB:

push eax

mov eax,dword [esp+0Ch+25*4]

push edx

xor edx,edx

mov ecx,8

mul ecx

mov dword [ebx+56],eax

mov dword [ebx+60],edx

pop edx

pop eax

jmp _n1

_nE:

inc edx

_n1:

test eax,eax

js  _nA

cmp eax,64

jnb _n2

jmp _n10

_nA:

xor eax,eax

_n10:

mov ecx,eax

jmp _n3

_n2:

mov ecx,64

_n3:

mov edi,ebx

rep movsb

push eax

push edx

push ebx

push esi

lea esi, [esp+10h]

mov edi, dword [esp+4+28*4]

push edi

mov eax, dword [edi]

mov ebx, dword [edi+04h]

mov ecx, dword [edi+08h]

mov edx, dword [edi+0Ch]

FF  eax, ebx, ecx, edx, 0, S11, 0d76aa478h

FF  edx, eax, ebx, ecx, 1, S12, 0e8c7b756h

FF  ecx, edx, eax, ebx, 2, S13, 0242070dbh

FF  ebx, ecx, edx, eax, 3, S14, 0c1bdceeeh

FF  eax, ebx, ecx, edx, 4, S11, 0f57c0fafh

FF  edx, eax, ebx, ecx, 5, S12, 04787c62ah

FF  ecx, edx, eax, ebx, 6, S13, 0a8304613h

FF  ebx, ecx, edx, eax, 7, S14, 0fd469501h

FF  eax, ebx, ecx, edx, 8, S11, 0698098d8h

FF  edx, eax, ebx, ecx, 9, S12, 08b44f7afh

FF  ecx, edx, eax, ebx, 10, S13, 0ffff5bb1h

FF  ebx, ecx, edx, eax, 11, S14, 0895cd7beh

FF  eax, ebx, ecx, edx, 12, S11, 06b901122h

FF  edx, eax, ebx, ecx, 13, S12, 0fd987193h

FF  ecx, edx, eax, ebx, 14, S13, 0a679438eh

FF  ebx, ecx, edx, eax, 15, S14, 049b40821h

GG  eax, ebx, ecx, edx, 1, S21, 0f61e2562h

GG  edx, eax, ebx, ecx, 6, S22, 0c040b340h

GG  ecx, edx, eax, ebx,11, S23, 0265e5a51h

GG  ebx, ecx, edx, eax, 0, S24, 0e9b6c7aah

GG  eax, ebx, ecx, edx, 5, S21, 0d62f105dh

GG  edx, eax, ebx, ecx,10, S22, 002441453h

GG  ecx, edx, eax, ebx,15, S23, 0d8a1e681h

GG  ebx, ecx, edx, eax, 4, S24, 0e7d3fbc8h

GG  eax, ebx, ecx, edx, 9, S21, 021e1cde6h

GG  edx, eax, ebx, ecx,14, S22, 0c33707d6h

GG  ecx, edx, eax, ebx, 3, S23, 0f4d50d87h

GG  ebx, ecx, edx, eax, 8, S24, 0455a14edh

GG  eax, ebx, ecx, edx,13, S21, 0a9e3e905h

GG  edx, eax, ebx, ecx, 2, S22, 0fcefa3f8h

GG  ecx, edx, eax, ebx, 7, S23, 0676f02d9h

GG  ebx, ecx, edx, eax,12, S24, 08d2a4c8ah

HH  eax, ebx, ecx, edx, 5, S31, 0fffa3942h

HH  edx, eax, ebx, ecx, 8, S32, 08771f681h

HH  ecx, edx, eax, ebx,11, S33, 06d9d6122h

HH  ebx, ecx, edx, eax,14, S34, 0fde5380ch

HH  eax, ebx, ecx, edx, 1, S31, 0a4beea44h

HH  edx, eax, ebx, ecx, 4, S32, 04bdecfa9h

HH  ecx, edx, eax, ebx, 7, S33, 0f6bb4b60h

HH  ebx, ecx, edx, eax,10, S34, 0bebfbc70h

HH  eax, ebx, ecx, edx,13, S31, 0289b7ec6h

HH  edx, eax, ebx, ecx, 0, S32, 0eaa127fah

HH  ecx, edx, eax, ebx, 3, S33, 0d4ef3085h

HH  ebx, ecx, edx, eax, 6, S34, 004881d05h

HH  eax, ebx, ecx, edx, 9, S31, 0d9d4d039h

HH  edx, eax, ebx, ecx,12, S32, 0e6db99e5h

HH  ecx, edx, eax, ebx,15, S33, 01fa27cf8h

HH  ebx, ecx, edx, eax, 2, S34, 0c4ac5665h

II  eax, ebx, ecx, edx, 0, S41, 0f4292244h

II  edx, eax, ebx, ecx, 7, S42, 0432aff97h

II  ecx, edx, eax, ebx,14, S43, 0ab9423a7h

II  ebx, ecx, edx, eax, 5, S44, 0fc93a039h

II  eax, ebx, ecx, edx,12, S41, 0655b59c3h

II  edx, eax, ebx, ecx, 3, S42, 08f0ccc92h

II  ecx, edx, eax, ebx,10, S43, 0ffeff47dh

II  ebx, ecx, edx, eax, 1, S44, 085845dd1h

II  eax, ebx, ecx, edx, 8, S41, 06fa87e4fh

II  edx, eax, ebx, ecx,15, S42, 0fe2ce6e0h

II  ecx, edx, eax, ebx, 6, S43, 0a3014314h

II  ebx, ecx, edx, eax,13, S44, 04e0811a1h

II  eax, ebx, ecx, edx, 4, S41, 0f7537e82h

II  edx, eax, ebx, ecx,11, S42, 0bd3af235h

II  ecx, edx, eax, ebx, 2, S43, 02ad7d2bbh

II  ebx, ecx, edx, eax, 9, S44, 0eb86d391h

pop edi

add dword [edi],eax

add dword [edi+04h],ebx

add dword [edi+08h],ecx

add dword [edi+0Ch],edx

pop esi

pop ebx

pop edx

pop eax

sub eax,64

test edx,edx

jne _n0

add esp,64

popad

ret 0Ch

; *****************************************************************************

; *****************************************************************************

align 4, nop

tens:

dd 10

dd 100

dd 1000

dd 10000

dd 100000

dd 1000000

dd 10000000

dd 100000000

dd 1000000000

x_numberDisplay:    ; either dotDecimal or dotHex , depending on the BASE to use to display numbers

dd dotDecimal

v_help_counter:     ; cycles through the help screens used by "help" ( F1 key )

dd 0

v_saved_v_blk:      ; the block number saved by "help"

dd 0xFF

v_locatedBlock:     ; the block of the located word

dd 510          ; demo only to see if the interface is OK - toggling the located word on "L" in the editor keypad

v_locatedCurs:      ; the address of the located word within its block

dd 10           ; demo only to see if the interface is OK - toggling the located word on "L" in the editor keypad

v_blk:              ; the currently edited block

dd START_BLOCK_NUMBER ; the default edited block

v_curs:             ; the offset in cells of the cursor within the currently edited block

dd 0

v_otherBlock:       ; the previously edited block

dd 510          ; the default other block is the highest non-shadow block saved by 'sa'

v_otherCursor:      ; the v_curs cursor position in the other block

dd 0

; v_otherBlocks:      ; the previously edited help block array

;     dd START_BLOCK_NUMBER           ; the default edited block

;     dd START_BLOCK_NUMBER + 1       ; the default other block is the shadow of the default edited block

;     dd START_BLOCK_NUMBER + 2       ; the default other block is the shadow of the default edited block

;     dd START_BLOCK_NUMBER + 3       ; the default other block is the shadow of the default edited block

v_cursPtr:          ; variable to count the cursor offset from the start of the block

dd 0

v_cursLine:         ; which line we want to display the cursor on

dd 0

v_curs_number_down: ; to limit the steps down

dd 0

v_display_token_number: ; how many tokens we have displayed in the editor screen

dd 0

v_numberOfMagentas:

dd 0

v_numberOfBigConstants:

dd 0

v_numberOfRedAndMagentas:

dd 0

v_numberOfTokens:   ; in the current block

dd 0

v_cad:              ; the address of the cursor as an offset from the start of the currently displayed block

dd 0

v_pcad:             ; saved pointer to current cursor address (?)

dd 0

v_lcad:             ; saved length of 32 bit cells to move (?)

dd 0

v_trash:                            ; pointer to "trash" buffer, saves words deleted while editing

dd TRASH_BUFFER

v_offset:

dd ( RELOCATED >> ( 2 + 8 ) )

v_bitsPerPixel:

dd 16   ; default, set using VESA info

v_iconw:

dd 0 ; iconw

v_iconh:

dd 0 ; iconh

v_keypadY_iconh:

dd 0    ; keypadY * iconh

v_nine_iconw:

dd 0

v_twentytwo_iconw:  ; width of 12 history characters, 1 space and 9 keypad characters

dd 0            ; to calculate the start of the history display, subtracted from the right edge of the screen

v_10000_iconw:

dd 0    ; iconw*0x10000

x_qwerty:           ; selects non-QWERTY if set to 0, else jumps to the address

dd 0xFFFFFFFF   ;

x_abort:

dd abort_action

x_colourBlind:  ; ( state -- state )

dd colourBlindAction

; byte variables

v_seeb:             ; if = 255, show blue words in editor

db 0            ; 255 enable, 0 disable

v_colourBlindMode:  ; if = 255, select ANS style editor display

db 0            ; 255 enable, 0 disable

v_not_cr:   ; true to disable the  cr  before a red word is displayed in the editor

db 0

v_quitMode:         ; if non zero, the keypad is in Edit mode, else immediate (TIB) mode

db 0            ; 255 enable, 0 disable

v_hintChar:         ; the character to display in the bottom right hand corner of the keyboard

dd 0            ; as a hint to the colour being used

v_random:           ; the current Marsaglia Pseudo Random Number Generator state

dd 0

v_show_ASCII:       ; if true show the ASCII keyboard entry field at the cursor

db 0

; align 4, nop    ; so we can read these variables easily from Forth

; md5_output: times 4 dd 0    ; the MD5 hash output

align 4

currentKeypadIcons:

dd ( alphaKeypad - 4 )

shiftAction:        ; the table of Forth words to execute for the current keypad

dd alpha0

vars:       ; colorForth system variables start here

base:

dd 10

setCurrentBase:                     ; set the base to either decimalor hexadecimal

dd setBase_decimal

keypad_colour:

dd colour_yellow   ; current key colour for displaying key presses

chars:

dd 1

aword:

dd ex1

anumber:

dd nul

v_words:

dd 1

v_qwerty_key:

dd 0

v_digin:

dd 0

lit:

dd adup

v_washColour:

dd colour_background

v_spare:        ; just to align v_pad to a 16 byte boundary

dd 0

dd 0

dd 0

mark_MacroWordCount:

dd MACRO_INITIAL_WORD_COUNT ; initial #macros

; number of Macro words, saved by  mark , empty restores to this value

mark_v_ForthWordCount:

dd FORTH_INITIAL_WORD_COUNT ; initial #words

; number of Forth words, saved by  mark , empty restores to this value

mark_v_BlueWordCount:

dd BLUE_INITIAL_WORD_COUNT ; initial #words

; number of Blue words, saved by  mark , empty restores to this value

mark_H:

dd H0       ; top of dictionary pointer H , saved by  mark , empty restores to this value

v_H:

dd H0       ; variable H , dictionary pointer HERE, where new definitions go

v_lblk:

dd 0        ; the last block loaded by  ld

; this variable address is 16 below pad

v_x:            ; general purpose variable e.g. for  dump

dd 0

v_y:            ; general purpose variable e.g. for  dump

dd 0

v_z:            ; general purpose variable

dd 0

v_lastToken:

dd 0

v_font:         ; a pointer to the current font table

dd ( font16x24 )           ; default font

v_onesec:       ; one second's worth of counter counts

dd 0

dd 0

v_khz:          ; the Processor clock, scaled down to kHz

dd 0

v_mhz:          ; the Processor clock, scaled down to MHz

dd 0

v_lastAddress:

dd 0

v_lastAddress_copy:

dd 0                    ; saves a copy of v_lastAddress

v_ForthWordCount:

dd FORTH_INITIAL_WORD_COUNT ; initial #words  ; number of words in the Forth wordlist, empty resets this value

v_MacroWordCount:

dd MACRO_INITIAL_WORD_COUNT ; initial #macros  ; number of words in the Macro wordlist, empty resets this value

v_BlueWordCount:

dd BLUE_INITIAL_WORD_COUNT ; initial #macros  ; number of words in the Blue wordlist, empty resets this value

v_test1:                            ; two 32 bit variables to save test values in. Use ' pad 8 - 2@ ' to view the values

dd 0

v_test2:

dd 0

v_pad:                              ; the standard Forth PAD, 84 bytes long

times 84 db 0x00

tokenActions:       ;

dd qignore      ; 0  extension token

dd execute_lit  ; 1

dd num          ; 2

adefine: ; compile time action for each type of token, 3 to 12

dd forthd       ; 3

dd qcompile     ; 4

dd cnum         ; 5

dd cshort       ; 6

dd compile      ; 7

dd short_       ; 8

dd nul          ; 9

dd nul          ; A

dd nul          ; B

dd m_variable   ; C magenta variable

dd nul          ; D

dd nul          ; E

dd nul          ; F

v_gr_xy:  ; variable that holds the XY position for drawing characters, ( 0, 0 ) is top left

v_gr_y:

dw 0x0003

v_gr_x:

dw 0x0003

v_leftMargin:

dd 0x00000003  ; left margin

v_rightMargin:

dd 0           ; right margin

; xycr:

; dd 0

v_fov:  ; abstract display scale

dd 0    ; 10 * ( 2 * scrnh + scrnh / 2 )

vframe:  ; pointer to display frame buffer where we create our image, down from top of 32 Mbytes RAM ( 0x2000000 )

dd 0x2000000 - ( MAX_SCREEN_WIDTH * MAX_SCREEN_HEIGHT * BYTES_PER_PIXEL )

; v_frameBuffer:          ; framebuffer address

;    dd 0x00000000       ;

v_foregroundColour:

dd 0x00000000       ; the display foreground colour, set by  set_color_

v_xc:

dd 0x00000000       ;

v_yc:

dd 0x00000000          ;

MacroNamesROM:

dd 0xF0000000       ; semicolon ";"

dd 0xC19B1000       ; dup

dd 0xCF833620       ; qdup

;    dd 0xFF833620       ; ?dup

dd 0xC0278800       ; drop

dd 0x2C88C000       ; then

dd 0xC6957600       ; begin_

; MacroNamesROM_end:

MacroJumpTableROM:         ; jump table for the macro wordlist

dd semicolon        ; ;

dd cdup             ; compile dup

dd qdup             ; qdup

dd cdrop            ; compile drop

dd then             ;

dd begin_           ;

MacroJumpTableROM_end:

ForthNamesROM:      ; displayed using cf2ansi

dd 0xC6664000   ; boot

dd 0xBA8C4000   ; warm

dd 0xC4B9A080   ; pause

dd 0x8AC84C00   ; macro     macro_

dd 0xB1896400   ; forth     forth_

dd 0x90000000   ; c

dd 0x1A635000   ; rlba      Read_Sector_LBA

dd 0xBD31A800   ; wlba      Write_Sector_LBA

dd 0x145C1000   ; reads     ReadSectors

dd 0xB8B92400   ; writes    WriteSectors

dd 0x84200000   ; sss       SaveAll_

;    dd 0x2C800000   ; th        th_ ( thunk to BIOS Int 0x13 )

dd 0x145C0000   ; read      bios_read

dd 0xB8B92000   ; write     bios_write

;    dd 0x18248800   ; rsect

dd 0xF9832800   ; @dx       fetchDX_

dd 0xF5817100   ; !dap

dd 0x59100000   ; act(tivate)

dd 0x8643B800   ; show

dd 0xA1AE0000   ; load

dd 0x6A1AE000   ; nload

dd 0xF7435C00   ; +load

dd 0x2C839800   ; thru

dd 0xF6590730   ; +thru

dd 0x963A7400   ; cblk      return the block number currently being compiled, calculated from  edi

dd 0x1C74E800   ; rblk      return the block number offset of the RELOCATED address

dd 0x5C74E800   ; ablk      4 / cellAddressToBlock

dd 0x41582000   ; erase

dd 0xC8828000   ; here      here_

dd 0xFF472000   ; ?lit

dd 0xD7F80000   ; 3,

dd 0xD5F80000   ; 2,

dd 0xD3F80000   ; 1,

dd 0x97E00000   ; c,

dd 0xFC000000   ; ,

dd 0xA2420000   ; less

dd 0xE59A3880   ; jump

dd 0xCF99C800   ; quit_      was accept = dd 0x59493110

dd 0xC4B80000   ; pad_

dd 0xC3019640   ; vsrch_

dd 0x80CB2000   ; srch_

dd 0xE893C580   ; keypd_

dd 0xBBE24000   ; wipe

dd 0xBBE24800   ; wipes     was erase

dd 0x91E29800   ; copy

dd 0x8A8F4000   ; mark

dd 0x48E22980   ; empty

dd 0x48B90000   ; emit

dd 0x29E24000   ; type   type_

dd 0xC0F57200   ; digit

dd 0xD4917200   ; 2emit

dd 0xEA000000   ; .

dd 0xC9D75000   ; h.2    dotHex2_

dd 0xC9D76000   ; h.4    dotHex4_

dd 0xC9D40000   ; h.     dotHex8_

dd 0xC9D58000   ; h.n

dd 0x90800000   ; cr

dd 0x86259200   ; space

dd 0xC0776000   ; down

dd 0x4C0E4000   ; edit

dd 0x40000000   ; e

dd 0xA4400000   ; lm

dd 0x18800000   ; rm

dd 0xA8AE2C80   ; graph

dd 0x24CA4000   ; text

dd 0xE893C4A0   ; keypa(d)  displayTheKeypad_  was 0xE893C660 keybo(ard)

dd 0xC098F300   ; debu(g)

dd 0x52000000   ; at

dd 0xF6A40000   ; +at

dd 0xCB300000   ; xy

dd 0xC4B54000   ; page

dd 0x84851180   ; screen

dd 0xB1E10000   ; fov

;    dd 0xB3D8C000   ; fifo

dd 0xC6794000   ; box

dd 0xA3B20000   ; line

dd 0x91D0C400   ; color     set_color_

dd 0x3912B100   ; octant

dd 0x86200000   ; sp

dd 0xA2C08000   ; last

dd 0xCCD89640   ; unpac(k)

dd 0xC4B2E800   ; pack

dd 0xC74E8000   ; blk

dd 0x8485AE00   ; scrnw   screen width in pixels

dd 0x8485B200   ; scrnh   screen height in pixels

dd 0xC78B1000   ; bpp     bits per pixel

dd 0xB1B10000   ; font    address of font pointer, containing by default font16x24

dd 0x791B5C00   ; iconw   icon width in pixels

dd 0x791B6400   ; iconh   icon height in pixels

dd 0x91E66240   ; counte(r) counter

dd 0x8C000000   ; ms      ms_

dd 0x36482480   ; onesec  onesec_

dd 0xE993B000   ; khz     khz_

dd 0x1297C000   ; rtc@    rtc_fetch_

dd 0x1297A000   ; rtc!    rtc_store_

dd 0x92D25D00   ; calck   calck_

dd 0xC2820000   ; ver

dd 0x96618000   ; curs

dd 0xC7439740   ; block_    block number to address

dd 0x5D58F9D0   ; a2blk_    address to block number

dd 0xC36158A0   ; vframe  video frame address, where we create the image to be displayed

dd 0xC2A30000   ; vars

; new words

dd 0x82263000   ; seeb     ( see blue words, toggle )

dd 0x812CBA40   ; stacks_

dd 0xC0650B00   ; dotsf    type a ShannonFano token

dd 0xA22E1400   ; leave

;    dd 0x12312310   ; txtq

dd 0x1AE30000   ; rgb

dd 0xC7340000   ; bye

dd 0xB98E0000   ; word

dd 0x4E840000   ; ekt

dd 0x5C662400   ; abort_

dd 0x27974C80   ; tickh  HERE variable address

dd 0xC79AD640   ; buffe(r)  buffer_

dd 0x3B5A0840   ; offset

dd 0x27900000   ; tic   tic_

dd 0xC2905000   ; vesa

dd 0xC2905880   ; vesam

dd 0x21586400   ; trash trash_

;    dd 0xC90C3840   ; hsvv_

dd 0xC3731C00   ; vword

;    dd 0xC2295800   ; vregs

dd 0x7C292000   ; ivec

dd 0x14863000   ; resb  restore_BIOS_idt_and_pic

dd 0xC4F20000   ; pic

dd 0xC0B88000   ; dap

dd 0x82488000   ; sect

dd 0xB98E0800   ; words

dd 0xE8930000   ; key

dd 0xCFD12600   ; qkey

dd 0xC0F57600   ; digin

dd 0xCF741200   ; qwert

dd 0x1FE00000   ; r?

dd 0x6CD40000   ; nul

dd 0x92E00000   ; cad

dd 0xC525C000   ; pcad

dd 0xC0F0C540   ; displ(ay)

dd 0x59148000   ; actc

dd 0xF7478100   ; +list

dd 0x72797400   ; itick

dd 0xA3C00000   ; lisl

dd 0xF6800000   ; +e

dd 0x820E1D20   ; serv1     serv1_

dd 0x780E1D20   ; isrv1_    initserv1_

dd 0x820E1D40   ; serv2     serv2_

dd 0x780E1D40   ; isrv2_    initserv2_

dd 0x4C0E4A00   ; edita     editAddress_

dd 0x963A3B60   ; cblind

dd 0x97C00000   ; c@        cFetch_

dd 0xBFC00000   ; w@        wFetch_

dd 0xF8000000   ; @         fetch_          no longer replaced by optimising verson in block 68

; dd 0xB214B200   ; fetch     fetch_          was replaced by optimising verson in block 68

dd 0xD5F00000   ; 2@        two_fetch_      no longer replaced by optimising verson in block 68

dd 0x97A00000   ; c!        cStore_

dd 0xBFA00000   ; w!        wStore_

dd 0xF4000000   ; !         store_          no longer replaced by optimising verson in block 68

dd 0xF7E80000   ; +!        plus_store_

dd 0xD5E80000   ; 2!        two_store_      no longer replaced by optimising verson in block 68

dd 0xC0C95000   ; dneg      d_negate_

dd 0xC1EC0000   ; d+        d_plus_

dd 0xC1CC0000   ; d-        d_minus_

dd 0xD5833620   ; 2dup      two_dup_

dd 0xD5804F10   ; 2drop     two_drop_	bug fix from Marco Nicola

dd 0xD50BAE20   ; 2swap     two_swap_

dd 0xD4785040   ; 2over     two_over_

dd 0x13200000   ; rot       rot_

dd 0xE6264000   ; -rot      minus_rot_

dd 0x2CD2E800   ; tuck      tuck_

dd 0xC4F2E800   ; pick      pick_

dd 0x92528000   ; cell      cell_

dd 0x92529CC0   ; cell-     cell_minus_

dd 0x92529EC0   ; cell+     cell_plus_

dd 0x92529000   ; cells     cells_

dd 0xA6200000   ; lp        lp_

dd 0xA3E02000   ; lidt      lidt_

dd 0x83E02000   ; sidt      sidt_

dd 0xD5DC0000   ; 2/        two_slash_

dd 0xCDABB800   ; u2/       u_two_slash_

dd 0x18647B10   ; rshift    rshift_

dd 0xCDABB800   ; lshif     lshift_

dd 0xFBDC0000   ; */        star_slash_

dd 0xCDF7B800   ; u*/       u_star_slash_

dd 0xeF13C000   ; /mod      slash_mod_

dd 0xFBDE2780   ; */mod     star_slash_mod_

dd 0x944F0A00   ; cmove_    cmove

dd 0xD5F40000   ; 2*        two_star_

dd 0xD5F7E800   ; 2**       two_star_star_

;    dd 0xD5DC0000  ; u/        u/_

dd 0x962CCF80   ; cpuid     GetCPUID_

dd 0x1C050900   ; rdtsc

dd 0x156C0000   ; rand      rand_

dd 0x156C1DC0   ; rand/     randInit_

dd 0x156C19C0   ; randq     randq_

dd 0x90CB5EA0   ; crc32     crc32_

dd 0x8E0DA000   ; md5       md5_

dd 0x8E0DB8C0   ; md5b      md5buf_

;    dd 0xB18C5480   ; format

;    dd 0xC5270000   ; pci

;    dd 0x68248000   ; nsec      was devic(e)

dd 0x85DCA590   ; switch    switch

dd 0xB0A27640   ; freeze    freeze

dd 0x23C40000   ; top       top_

dd 0x8AC94000   ; maca      maca_

dd 0x8AC98000   ; macn      macn_

dd 0x8ACA8000   ; macl      macl_

dd 0x8AC88000   ; mact      mact_

dd 0xB1645000   ; ftha      ftha_

dd 0xB1646000   ; fthn      fthn_

dd 0xB164A000   ; fthl      fthl_

dd 0xB1642000   ; ftht      ftht_

dd 0xC74CCA00   ; blua      blua_

dd 0xC74CCC00   ; blun      blun_

dd 0xB63A6640   ; fblue     find_Blue_word_

dd 0xA1C80000   ; loc       loc_

dd 0x82200000   ; see       see_

dd 0xB3600000   ; fnd       fnd_

dd 0xC11C4000   ; dmp       dmp_

dd 0x4CA92000   ; exec      execute_

dd 0xCA000000   ; x         x_

dd 0x98000000   ; y         y_

dd 0xEC000000   ; z         z_

dd 0xA63A7400   ; lblk      lblk_

dd 0            ; terminating null at the end of the list

ForthJumpTableROM:  ; jumptable:

dd boot         ;

dd warm         ;

dd pause_       ; pause

dd macro_       ; macro     select the macto wordlist

dd forth_       ; forth     select the forth wordlist

dd c_           ; c

dd Read_Sector_LBA   - $$ + BOOTOFFSET  ; jmp Read_Sector_LBA

dd Write_Sector_LBA  - $$ + BOOTOFFSET  ; jmp Write_Sector_LBA

dd ReadSectors  - $$ + BOOTOFFSET  ; jmp ReadSectors    reads

dd WriteSectors - $$ + BOOTOFFSET  ; jmp WriteSectors   writes

dd SaveAll_     - $$ + BOOTOFFSET  ; jmp SaveAll_

;    dd th_        - $$ + BOOTOFFSET ; jmp th_ ( thunk to BIOS Int 0x13 )

dd bios_read  - $$ + BOOTOFFSET ; jmp bios_read   'read'

dd bios_write - $$ + BOOTOFFSET ; jmp bios_write  'write'

;    dd XXXrsect_  - $$ + BOOTOFFSET ; jmp rsect_  'rsect'

dd fetchDX_     ; @dx

dd setupDAP_    ; !dap

dd activate     ; act

dd show         ;

dd _load_       ;

dd nload        ; nload

dd plusLoad     ; +load

dd thru_        ; thru

dd plusThru_    ; +thru

dd cblk_        ;           return the block number currently being compiled, calculated from  edi

dd rblk_        ;           return the block number offset of the RELOCATED address

dd ablk_        ;           convert byte address to block number

dd erase_       ;

dd here_        ; here      returns the current dictionary pointer

dd qlit         ; ?lit

dd comma3_      ; 3,

dd comma2_      ; 2,

dd comma1_      ; 1,

dd comma1_      ; c,

dd comma_       ; ,

dd less         ; less

dd jump         ; jump

dd quit_        ; quit

dd pad_         ; pad

dd vsrch_       ; vsrch

dd srch_        ; srch

dd keypd_       ; keypd  ( alias of pad )

dd wipe         ;

dd wipes        ;

dd copy_        ; copy

dd mark         ;

dd empty_       ; empty

dd emit_        ; emit

dd type_        ; type

dd digit        ;

dd two_emit     ; 2emit

dd dotDecimal   ; .

dd dotHex2_     ; h.2

dd dotHex4_     ; h.4

dd dotHex8_     ; h.

dd h_dot_n      ; h.n

dd cr_          ; cr

dd space_       ; space

dd down         ;

dd edit_        ;

dd e_           ; e

dd lm           ;

dd rm           ;

dd graphAction  ; graph

dd setupText_   ; text

dd displayTheKeypad_ ; keypa (d)

dd debug        ;

dd _at          ; at

dd plus_at      ; +at

dd xy_          ;

dd page_        ; page

dd screen_      ; screen

dd fov_         ;

;    dd fifo         ;

dd box_         ; box

dd line_        ; line

dd set_color_    ; color

dd octant       ;

dd tokenActions_ ; tokenActions table

dd last         ;

dd unpack       ;

dd pack_        ;

dd blk_         ;

dd scrnw_       ; scrnw  screen width in pixels

dd scrnh_       ; scrnh  screen height in pixels

dd bpp_         ; bpp    bits per pixel

dd font_        ; font   address of font pointer, containing by default font16x24

dd iconw_       ; iconw  icon width in pixels

dd iconh_       ; iconh  icon height in pixels

dd counter_     ; counter

dd ms_          ; ms

dd onesec_      ; onesec

dd khz_         ; khz

dd rtc_fetch_   ; rtc@

dd rtc_store_   ; rtc!

dd calck_       ; calclk  calibrate the clock for ms

dd version_     ; ver

dd curs         ; curs

dd block_       ; block

dd a2blk_       ; a2blk

dd vframe_      ; vframe

dd vars_        ; vars

; new words

dd seeb         ; seeb

dd stacks_      ;

dd dotsf_       ; dotsf

dd leave_       ; leave

;    dd txtq_        ;

dd rgb          ; rgb

dd bye_         ; bye

dd _word        ;

dd ekt          ;

dd abort_       ;

dd tickh        ;

dd buffer_      ; buffe(r)

dd offset_      ;

dd tic_         ; tic

dd vesa         ;

dd vesamode_    ;

dd trash_       ; trash

;    dd hsvv_        ; hsvv

dd vword_       ; ('%s')", DB_NAME,

;    dd vregs_       ; vregs

dd ivec_         ; ivec

dd restore_BIOS_idt_and_pic ; resb

dd pic_         ; pic  Programmable Interrupt Controller settings, as set by the BIOS

dd dap_         ; dap

dd sect_        ; sect

dd words_       ; words

dd get_key_     ; key

dd get_qwerty_key_ ; qkey

dd digin        ;

dd qwert        ;

dd rquery       ; r?

dd nul          ;

dd cad          ;

dd pcad         ;

dd displ        ;

dd actc         ;

dd plusList     ; +list

dd itick        ;

dd refresh      ; lis

dd plus_e       ; +e

dd serv1_       ; serv1

dd initserv1_   ; isrv1_

dd serv2_       ; serv2

dd initserv2_   ; isrv1_

dd editAddress_ ; edita

dd cBlindAddr_  ; cblind

dd cFetch_      ; c@

dd wFetch_      ; w@

dd fetch_       ; @     was replaced by optimising verson in block 68

dd two_fetch_   ; 2@    was replaced by optimising verson in block 68

dd cStore_      ; c!

dd wStore_      ; w!

dd store_       ; !     was replaced by optimising verson in block 68

dd plus_store_  ; +!

dd two_store_   ; 2!    was replaced by optimising verson in block 68

dd d_negate_    ; dneg

dd d_plus_      ; d+

dd d_minus_     ; d-

dd two_dup_     ; 2dup

dd two_drop_    ; 2drop

dd two_swap_    ; 2swap

dd two_over_    ; 2over

dd rot_         ; rot

dd minus_rot_   ; -rot

dd tuck_        ; tuck

dd pick_        ; pick

dd cell_        ; cell

dd cell_minus_  ; cell-

dd cell_plus_   ; cell+

dd cells_       ; cells

dd lp_          ; lp

dd lidt_        ; lidt

dd sidt_        ; sidt

dd two_slash_   ; 2/

dd u_two_slash_ ; u2/

dd rshift_      ; rshift

dd lshift_      ; lshif     lshift

dd star_slash_  ; */

dd u_star_slash_; u*/

dd slash_mod_   ; /mod

dd star_slash_mod_ ; */mod

dd cmove_       ; cmove

dd two_star_    ; 2*

dd two_star_star_ ; 2**   _

;    dd u/_          ; u/

dd GetCPUID_    ; cpuid

dd rdtsc_       ; rdtsc

dd rand_        ; rand

dd randInit_    ; rand/

dd randq_       ; randq

dd crc32_       ; crc32

dd md5_         ; md5

dd md5buf_      ; md5b      md5buf_

;    dd format       ;

;    dd pci          ;

;    dd device       ;

dd switch       ;

dd freeze       ;

dd top_         ;

dd maca_        ; maca

dd macn_        ; macn

dd macl_        ; macl

dd mact_        ; mact

dd ftha_        ; ftha

dd fthn_        ; fthn

dd fthl_        ; fthl

dd ftht_        ; ftht

dd blua_        ; blua

dd blun_        ; blun

dd find_Blue_word_ ; fblue

dd loc_         ; loc

dd see_         ; see

dd fnd_         ; fnd

dd dmp_         ; dmp

dd execute_     ; exec

dd x_           ; x

dd y_           ; y

dd z_           ; z

dd lblk_        ; lblk

ForthJumpTableROM_end:

; times 200 NOP   ;  enable this line to see how much space is left. If NASM reports :

; "cf2022.nasm:6282: error: TIMES value -28 is negative" with "times 200" you have (200 - 28) bytes left

; fill with no-ops to 55AA at end of boot sector, less $40 for the info string

times ( ( START_BLOCK_NUMBER - SIZE_OF_FONT_IN_BLOCKS ) * 0x400 ) - ($ - $$)  NOP

; the above produces a 26K boot image, we then add the 6K font and colorForth source blocks:

font16x24:

; incbin "cf2023_font.img"

; incbin "cf2023Ref.img",( OFFSET_OF_FONT +(SIZE_OF_FONT_IN_BLOCKS *1024) ), ( 512 * 1024 ) ; append the font and colorForth source blocks from the reference image, skip the kernel code

; colorForth:                         ; the colorForth source blocks

incbin "cf2023Ref.img", OFFSET_OF_FONT, ( ( 512 - START_BLOCK_NUMBER + SIZE_OF_FONT_IN_BLOCKS ) * 1024 ); append the font and colorForth source blocks from the reference image, skip the kernel code

times 32768 db 'U'

; end of file

# Appendix C colorForth Source Code

\ C:\Users\User\Dropbox\colorForth\cf2023\cf2023Ref.img converted by colorForthScan V1.0 2023 Jan 29

\ File MD5 = 649C8F306CE629E514E5A0812BF1C912  roomed-zebra

\ MagentaV  is the colorForth Magenta Variable

: MagentaV ( initial -- )   create , ; \ Runtime: ( -- a )

\ Block 64

( colorforth cf2023 2023 Apr 04 )

( processor clock ) #0 MagentaV mhz

( dump ) #5952 MagentaV x #0 MagentaV y ( ld ) #64 MagentaV lblk

#2 #12 +thru

: dump #78 load ;

: words #112 ld ;

: icons #80 ld ;

: serve #506 ld ;

: north #92 ld ;

: rtc #96 ld ;

: lan #98 ld ;

: colors #102 ld ;

: wood #106 ld ;

: mand #108 ld ;

: sound #114 ld ;

: gr #118 ld ;

: eth #176 ld ;

: life #272 ld ;

: ed #252 ld ;

: slime #246 ld ;

: int #288 ld ;

: xx #278 load ;

: info ver dump ;

: staks #504 ld ;

( hardware ) #0 MagentaV rng

: chm ( -- ) #0 mhz ! md5b x ! #0 y ! #64 lblk ! $00010000

: ch ( n-- ) #64 block swap md5 dump ;

: hlp randq rng ! logo pause calkhz

onesec @ #1000 / mhz ! e ;

mark empty hlp

( Press the * key to see the comment block )

( Press F1 )

\ Block 65

\ ( Based on colorforth 2001 Jul 31 by Chuck Moore )

\ ( released into the Public Domain. )

\ ( This block is loaded at power up. Press F1 for help )

\ : dump ( instant compile version of DUMP )

\ : icons ( edit the character font icons )

\ : north ( North Bridge PCI chip display )

\ : rtc ( Real Time Clock display )

\ : colors ( 3-axis rgb colour display )

\ : wood ( imitation pine blockboard )

\ : mand ( display the Mandeldrot set )

\ : sound ( control the PC speaker )

\ : gr ( graphics - type ok to run the demo )

\ : life ( Conways game of life )

\ : ed ( the editor partly converted to colorforth )

\ : slime ( watch out for the slugs! )

\ : int ( 1000 Hz timer interrupt )

\ : xx ( colorforth explorer )

\ : ch ( show MD5 of n bytes starting at block 64 )

\ : chm ( show MD5 of system blocks )

\ : help ( press the space bar to leave the editor, then type the keys indicated in the keypad in the bottom right of the

\ screen, then the space bar to execute the word. Type ) e ( or ) #64 edit ( to run the editor. )

\ info ( to view the boot system version )

\ seeb ( to toggle display of blue words )

\ hlp ( shows help and clock speed )

\ Block 66

macro

: ?f $C021 2, ;

: 0if $75 2, here ;

: +if $78 2, here ;

: 1+ ( n-n ) $40 1, ;

: 1- ( n-n ) $48 1, ;

: 2/ ( n-n ) $F8D1 2, ;

: time ( -u ) qdup $310F 2, ;

: shl ( uc-u ) ?lit $E0C1 2, 1, ;

: shr ( uc-u ) ?lit $E8C1 2, 1, ;

: r@ qdup $8B 1, $C7 1, ;

: sti $FB 1, ; ( enable interrupts )

: cli $FA 1, ; ( disable interrupts )

forth

: cli cli ;

: sti sti ;

: nul ;

: time time ;

\ Block 67

\ ( Pentium macros: )

\ : ?f ( set flags to reflect tos )

\ : 0if ( if zero ... then jnz aids in clarity )

\ : +if ( js, this complements the set )

\ : 1- ( subtract 1 )

\ : 2/ ( divide by 2 )

\ : qdup ( is the new name for ?dup )

\ : time ( return Pentium instruction counter )

\ : lshift ( shift u left c places )

\ : rshift ( shift u right c places )

\ : r@ ( copies the top of the return stack to TOS )

\ : sti ( enable device interrupts )

\ : cli ( disable them )

\ : a,

\ Block 68

( more macros ) macro

: swap $168B 2, $C28B0689 , ;

: 0 qdup $C031 2, ;

: if $74 2, here ;

: -if $79 2, here ;

: a qdup $C28B 2, ;

: a! ?lit if $BA 1, , ; then $D08B 2, drop ;

: 1@ $8A 2, ;

: 1! a! $0288 2, drop ;

: p@ ( a-n ) qdup a! $EC 1, ;

: p! ( na- ) a! $EE 1, drop ;

: 2* $E0D1 2, ;

: a, , ;

: @@ ?lit if qdup $058B 2, , ; then $8B 2, 0 , ;

: !! ?lit if ?lit if $05C7 2, swap , , ; then $0589 2, , drop ; then a! $0289 2, 0 , drop ;

: nip $0004768D 3, ;

: + ?lit if $05 1, , ; then $0603 2, nip ;

: xor $0633

: binary ?lit if swap #2 + 1, , ; then 2, nip ;

: and $0623 binary ;

: or $060B binary ;

: u+ ?lit if $0681 2, , ; then $00044601 3, drop ;

: ? ?lit $A9 1, , ;

\ Block 69

\ ( Pentium macros: 1, 2, 3, , compile 1-4 bytes )

\ : drop ( lodsd, flags unchanged, why sp is in ESI )

\ : over ( sp 4 + @ )

\ : swap ( sp xchg )

\ : 0 ( 0 0 xor, macro 0 identical to number 0 )

\ : a ( 2 0 mov, never used? )

\ : a! ( 0 2 mov, unoptimized )

\ : 1@ ( fetch byte from byte address )

\ : 1! ( store byte to byte address )

\ : p@ p-n ( fetch byte from port )

\ : p! np ( store byte to port )

\ : @ ( EAX 4 *, unoptimized )

\ : ! ( EDX 4 * )

\ : nop ( used to thwart look-back optimization )

\ : - ( ones-complement )

\ : 2*

\ : 2/

\ : if ( jz, flags set, max 127 bytes, leave address )

\ : -if ( jns, same )

\ : then ( fix address - in kernel )

\ : push ( EAX push )

\ : pop ( EAX pop )

\ : u+ ( add to 2nd number, literal or value )

\ : ? ( test bits, set flags, literal only! )

\ Block 70

( even more macros )

: over qdup $0004468B 3, ;

: push $50 1, drop ;

: pop qdup $58 1, ;

: invert ( n-n ) $D0F7 2, ;

: for push begin ;

: *next swap

: next $75240CFF

: 0next , here invert + 1, $0004C483 3, ;

: -next $79240CFF 0next ;

: i qdup $0024048B 3, ;

: *end swap

: end $EB 1, here invert + 1, ;

: +! ?lit if ?lit if $0581 2, swap , , ; then $0501 2, , drop ; then a! $0201 2, drop ;

: nop $90 1, ;

: align here invert #3 and drop if nop align ; then ;

: or! a! $00950409 3, 0 , drop ;

: * $0006AF0F 3, nip ;

: */ $C88B 2, drop $F9F72EF7 , nip ;

: /mod swap $99 1, $16893EF7 , ;

: / /mod nip ;

: mod /mod drop ;

\ Block 71

\

\ : - n-n ( ones complement negate , xor )

\ : for n ( push count onto return stack, falls into ) begin

\ : begin -a ( current code address - byte )

\ : *next aa-aa ( swap ) for ( and ) if ( addresses )

\ : next a ( decrement count, jnz to ) for, ( pop return stack when done )

\ : -next a ( same, jns - loop includes 0 )

\ : i -n ( copy loop index to data stack )

\ : end a ( jmp to ) begin

\ : +! na ( add to memory, 2 literals optimized )

\ : align ( next call to end on word boundary )

\ : or! na ( inclusive-or to memory, unoptimized )

\ : * mm-p ( 32-bit product )

\ : */ mnd-q ( 64-bit product, then quotient )

\ : /mod nd-rq ( remainder and quotient )

\ : / nd-q ( quotient )

\ : mod nd-r ( remainder )

\ : time -n ( Pentium cycle counter, calibrate to get actual clock rate )

\ Block 72

( Compiled macros ) forth

: r@ ( -n ) r@ ;

: @@ ( a-n ) @ ;

: !! ( an- ) ! ;

( Arithmetic )

: + ( nn-n ) + ;

: 1+ ( u--u ) 1+ ;

: 1- ( u--u ) 1- ;

: invert ( n-n ) invert ;

: */ ( nnn-n ) */ ;

: * ( nn-n ) * ;

: / ( nn-n ) / ;

: 2* ( n-n ) 2* ;

: 2/ ( n-n ) 2/ ;

: drop ( n- ) drop ;

: dup ( n-nn ) dup ;

: swap ( nn-nn ) swap ;

: over over ;

: negate ( n-n ) invert #1 + ;

: - ( nn-n ) negate + ;

: min ( nn-n ) less if drop ; then swap drop ;

: abs ( n-u ) dup negate

: max ( nn-n ) less if swap then drop ;

: v+ ( vv-v ) push u+ pop + ;

: save sss ;

: sa sss e ;

\ Block 73

\ ( These macros may be ) yellow, ( others may not )

\ : block n-a ( block number to word address )

\ : r@ ( copies the top of the return stack to stack )

\ : @ etc ( Arithmetic )

\ : negate n-n ( when you just cant use ) -

\ : min nn-n ( minimum )

\ : abs n-u ( absolute value )

\ : max nn-n ( maximum )

\ : v+ vv-v ( add 2-vectors )

\ : save ( write colorforth to a bootable USB drive )

\ : sa ( save, then show edit screen )

\ Block 74

( Relative load blocks )

: ll ( -- ) blk @ load ;

: sect ( --asn ) blk @ block blk @ 2* #2 ;

: ss ( -- ) sect writes drop drop ;

: uu ( -- ) sect reads drop drop ;

: ld ( n- ) dup lblk ! load ;

: vv ( -- ) lblk @ edit ;

: help ( -- ) lblk @ #1 + edit ;

( Real Time Clock )

: rtc@ ( t-c ) $70 p! $71 p@ ;

: rtc! ( ct- ) $70 p! $71 p! ;

: hi ( -- ) #10 rtc@ $80 and drop 0if hi ; then ;

: lo ( -- ) #10 rtc@ $80 and drop if lo ; then ;

: calkhz ( -- ) hi lo counter hi lo counter swap -

dup onesec ! #1 rshift #250 + #500 / dup khz ! ;

: ms ( n- ) khz @ * counter + begin pause dup counter

invert + drop -if drop ; then end drop ;

: secs ( n- ) for pause lo hi next ; macro

: swapb ( w-w ) $E086 2, ; forth

: split ( w--cc ) dup swapb $FF and swap $FF and ;

\ Block 75

\

\ : nload ( loads the next source block : b+2 )

\ : +load ( loads the source block : b+n )

\ : blk ( where the current blk happens to be kept )

\ : ll ( load the current edit blk )

\ : ss ( save the sector containing the current edit block to the floppy disc )

\ : lblk ( holds the last block loaded by )

\ : ld

\ : vv ( edits the last block loaded by ld )

\ : rtc@ reg-n ( fetch reg from rtc )

\ : rtc! n reg- ( store in rtc register )

\ : hi ( wait till Update In Progress bit is high )

\ : lo ( wait till UIP bit is low )

\ : calkhz ( calibrate the processor clock using the RTC )

\ : ms ( wait for n milliseconds )

\ : secs ( wait for n seconds )

\ : swapb ( swap the two low bytes )

\ : split ( split the low two bytes )

\ : vframe ( byte address of the video frame buffer )

\ Block 76

( Colors etc )

: white $00FFFFFF rgb color ; : red $00FF0000 rgb color ;

: green $FF00 rgb color ; : blue $FF rgb color ;

: silver $00BFBFBF rgb color ; : yellow $FFE0 color ;

: orange $00E04000 rgb color ; : black $00 rgb color ;

: 5* #5 for 2emit next ;

: cf #25 dup at red $72 $6F $6C $6F $63 5* green $68 $74 $72 $6F $46 5* ;

: logo show black screen #800 #710 blue box #600 #50 at #1024 #620 red box #200 #100 at #700 #500 green box text cf keypa

d ;

: noshow show keypad ;

: lshift ( uc-u ) $1F and ?f 0if drop ; then for #1 shl next ;

: rshift ( uc-u ) $1F and ?f 0if drop ; then for #1 shr next ;

: rand32 ( -n ) time dup #16 lshift xor ;

: string pop ;

: 1@ ( a-c ) 1@ $0F and ;

: 1! ( ac- ) 1! ;

\ Block 77

\

\ : colors ( specified as rgb: 888 )

\ : screen ( fills screen with current color )

\ : at xy ( set current screen position )

\ : box xy ( lower-right of colored rectangle )

\ : 5* ( displays 5 large characters )

\ : cf ( displays ) colorforth

\ : logo ( displays colorforth logo )

\ : empty ( also displays the logo )

\ : lshift ( shift u left c places )

\ : rshift ( shift u right c places )

\ : show ( background task executes following code repeatedly )

\ : keypad ( displays keypad and stack )

\ : string ( returns the address of the string following )

\ : rand32 ( returns a 3 bit random number )

\ Block 78

( Dump names )

: .cell ( a-a ) orange dup @ #4 for dup $FF and emit $0100 / next drop white ;

: one dup dup @ dup push h. space dup h. pop space swap .cell drop space space space space dup dotsf drop white cr ;

: lines for one #4 + next drop ;

: dump ( a- ) $0FFFFFFC and x !

: r show black screen x @ #16 text lines cr x @ #16 for .cell #4 + next drop keypad ;

: it @ + @ dup h. space ;

: lines for white i x it i y it xor drop if red then i . cr -next ;

: cmp show blue screen text #19 lines red x @ h. space y @ h. keypad ;

: u $40

: +xy dup x +! y +! ;

: d $FFFFFFC0 +xy ;

: ati $F4100000 ( ff7fc000 ) xor

: byte #4 / dump ;

: fix for #0 over ! #1 + next ; dump

\ Block 79

\ ( Does not say empty, compiles on top of application )

\ : x -a ( current address )

\ : one a-a ( line of display )

\ : lines an

\ : dump a ( background task continually displays memory : decodes the value as a name and ASCII )

\ : u ( increment address )

\ : d ( decrement )

\ : ati ( address of AGP graphic registers )

\ : byte a ( byte address dump )

\ : fix an-a ( test word )

\ : ver ( show the kernel version information )

\ : cmp ( shows data at both ) x ( and ) y ( addresses )

\ Block 80

( App: Icons font editor ) empty

( icon number ) #28 MagentaV ic ( cursor ) #351 MagentaV cu

macro : @w $8B66 3, ; : !w a! $00028966 3, drop ;

: *byte $C486 2, ; forth

: sq xy @ $00010000 /mod #16 + swap #16 + box #17 #0 +at ;

: loc ic @ $FF and

: tofont ( n--a ) #16 #24 #8 */ * font @ + ;

: 0/1 $8000 ? if green sq ; then blue sq ;

: row dup @w *byte #16 for 0/1 2* next drop #-17 #16 * #17 +at ;

: cpl #32 ;

: showall ( -- ) #2 lm iconw cpl * rm ic @ cpl /mod iconh * #448 #2 - + swap iconw * swap over over at red #16 #4 + u+ #24

+ #4 + box white

#0 #2 #448 at #256 for dup emit #1 + next drop ;

: ikon loc #24 for row #2 + next drop ;

: adj #17 * swap ;

: cursor cu @ #16 /mod adj adj over over at red #52 u+ #52 + box ;

: ok show page cursor #18 dup at ikon text blue #400 #400 at    ef #416 #424 box #400 #400 at white    ef ic @ dup emit space

dup green . $30 emit $78 emit #2 h.n showall keypad ;

: fcopy tofont swap tofont swap #16 #24 #8 */ cmove ;

nload ok h

\ Block 81

\ ( Draw big-bits icon )

\ : @w a-n ( fetch 16-bit word from byte address )

\ : !w na ( store same )

\ : *byte n-n ( swap bytes )

\ : ic -a ( current icon )

\ : cu -a ( cursor )

\ : sq ( draw small square )

\ : xy -a ( current screen position, set by ) at

\ : loc -a ( location of current icons bit-map )

\ : 0/1 n-n ( color square depending on bit 15 )

\ : row a-a ( draw row of icon )

\ : +at nn ( relative change to screen position )

\ : ikon ( draw big-bits icon )

\ : adj nn-nn ( magnify cursor position )

\ : cursor ( draw red box for cursor )

\ : ok ( background task to continually draw icon, icon number at bottom )

\ Block 82

( Edit icon )

: icmv ( n-- ) ic @ + $FF and ic ! ;

: +ic #1 icmv ;

: -ic #-1 icmv ;

: ++ic cpl icmv ;

: --ic cpl negate icmv ;

: bit cu @ 2/ 2/ 2/ 2/ 2* loc + $00010000 cu @ $0F and #1 + for 2/ next *byte ;

: toggle bit over @w xor swap !w ;

: td toggle

: d #16

: wrap cu @ + #16 #24 * dup u+ /mod drop cu ! ;

: tu toggle

: u #-16 wrap ;

: tr toggle

: r #1 wrap ;

: tl toggle

: l #-1 wrap ;

: nul ;

: h keypd

nul nul quit nul        tl tu td tr

l u d r                 -ic --ic ++ic +ic

nul nul nul nul         nul nul nul toggle

nul nul nul nul

$2500 , $13121110 dup , , $2B16152D , #0 , $80000000 , #0 ,

\ Block 83

\ ( Edit icon )

\ : t ( toggles the current pixel )

\ : ludr ( left up down right )

\ : . ( top row toggles and moves )

\ : -+ ( select icon to edit )

\ Block 84

( Print PNG to disk ) #1024 MagentaV w #768 MagentaV h #1 MagentaV d

#6 +load #4 +load #2 +load

: -crc ( a ) here over negate + crc . ;

: crc -crc ;

: wd ( -a ) here #3 and drop if #0 1, wd ; then here #2 2/s ;

: bys ( n-a ) . here swap , ;

: plte $45544C50 #48 bys $00 3, $00FF0000 3, $FF00 3, $00FFFF00 3, $FF 3, $00FF00FF 3, $FFFF 3, $00FFFFFF 3, $00 3, $00C00000

3, $C000 3, $00C0C000 3, $C0 3, $00C000C0 3, $C0C0 3, $00C0C0C0 3, crc ;

: png ( awh ) d @ / h ! d @ / w ! wd swap $474E5089 , $0A1A0A0D , ( ihdr ) $52444849 #13 bys w @ . h @ . $0304 , $00 1, crc

plte ( idat ) $54414449 #0 bys swap deflate crc ( iend ) $444E4549 #0 bys crc wd over negate + ;

: at #1024 * + 2* vframe + ;

: full #4 d ! #0 dup at #1024 #768 png ;

: pad #1 d ! #46 #-9 + #22 * nop #25 #-4 + #30 * at #9 #22 * nop #4 #30 * png ;

: go #1 d ! #1024 w ! #768 h ! #0 #0 at #1024 #768 png raw ; go e

\ Block 85

\ ( Print PNG to disk )

\ : frame ( the video frame buffer )

\ : -crc ( a )

\ : crc

\ : wd ( -a )

\ : bys ( n-a )

\ : plte

\ : png ( awh )

\ : at

\ : full

\ : pad

\ : go ( copy the screen image as a PNG file to the floppy disk block 270 and up. )

\ Block 86

( lz77 ) macro

: @w $8B66 3, ;

: *byte $C486 2, ;

: !b a! $0289 2, drop ; forth

: *bys dup #16 2/s *byte swap $FFFF and *byte $00010000 * + ;

: . *bys , ;

: +or over invert and or ;

: 0/1 $10 ? if $1E and $1E or drop if #7 ; then $0F ; then #0 and ;

: 4b dup 0/1 #9 and over #6 2/s 0/1 $0A and +or swap #11 2/s 0/1 $0C and +or $08 or ;

: pix dup @w d @ 2* u+ 4b ;

: row 1, dup w @ 2/ dup #1 + dup 2, invert 2, #0 dup 1, +adl for pix #16 * push pix pop or dup 1, +adl next drop +mod d @

#1024 #2 * * + ;

: deflate $0178 2, #1 #0 adl! h @ #-1 + for #0 row next #1 row drop ad2 @ *byte 2, ad1 @ *byte 2, here over #4 + negate +

*bys over #-4 + !b ;

\ Block 88

( Crc ) macro

: 2/s ?lit $E8C1 2, 1, ;

: 1@ $8A 2, ; forth #36054 MagentaV ad1 #54347 MagentaV ad2

: array ( -a ) pop #2 2/s ;

: bit ( n-n ) #1 ? if #1 2/s $EDB88320 or ; then #1 2/s ;

: fill ( nn ) for dup #8 for bit next , #1 + next drop ;

: table ( -a ) align array #0 #256 fill

: crc ( an-n ) #-1 swap for over 1@ over or $FF and table + @ swap #8 2/s or #1 u+ next invert nip ;

: +adl ( n ) $FF and ad1 @ + dup ad2 @ +

: adl! ad2 ! ad1 ! ;

: +mod ad1 @ #65521 mod ad2 @ #65521 mod adl! ;

\ Block 90

( DOS file )

: blks #256 * ;

: w/c #18 blks ;

: buffer block ;

: size ( -a ) buffer #0 #1 reads buffer $098F + ;

: set ( n ) ! buffer     s #1 writes ;

: cyls ( n-nn ) #1 swap w/c #-1 + + w/c / ;

: put ( an ) dup 2* 2* size set cyls writes /flop ;

: raw ( an- ) #15 swap 2* 2* w/c #-1 + + w/c / writes /flop ;

: get ( a ) size @ #3 + 2/ 2/ cyls reads /flop ;

: .com #0 #63 blocks put ;

\ Block 91

\

\ : blks n-n ( size in blocks to words )

\ : w/c -n ( words per cylinder )

\ : buffer -a ( 1 cylinder required for floppy dma )

\ : size -a ( locate size of 2nd file. Floppy has first FILLER then FILE allocated. FILLER is 2048 bytes, to fill out cylind

\er 0. Names at most 8 letters, all caps. Directory starts at ) buffer $0980 +

\ : set n ( size. FILE must be larger than your file. )

\ : cyls n-nn ( starting cylinder 1 and number of cylinders )

\ : raw an ( write raw data to cyl 15 , block 270 )

\ : put an ( write file from address )

\ : get a ( read file to address )

\ Block 92

( App: North Bridge ) empty macro

: 4@ dup $ED 1, ;

: 4! $EF 1, drop ; forth #2048 MagentaV dev

: nb $00 dev ! ;

: sb $3800 dev ! ;

: agp $0800 dev ! ;

: ess $6800 dev ! ;

: ric $7800 dev ! ;

: win $8000 dev ! ;

: ati $00010000 dev ! ;

: add $0CF8 a! 4! $0CFC a! ;

: q $80000000 + add 4@ ;

: en $8004 q #-4 and xor 4! ;

: dv dup $0800 * q swap #1 + ;

: regs dev @ #19 #4 * + #20 for dup q h. space dup h. cr #-4 + next drop ;

: devs #0 #33 for dup q dup #1 + drop if dup h. space drop dup #8 + q dup h. space over h. cr then drop $0800 + next drop

;

: ok show black screen text regs keypad ;

: ko show black screen text devs keypad ;

: u $40 dev +! ;

: d #-64 dev +! ;

: test $FF00 + a! 4@ ; ok

\ Block 93

\ ( Display the PCI interface chip registers )

\ Block 94

( ASCII )

: cf-ii string ( 0*00 ) $6F747200 , $696E6165 , $79636D73 , $7766676C , ( 0*10 ) $62707664 , $71757868 , $33323130 , $37363534

, ( 0*20 ) $2D6A3938 , $2F7A2E6B , $2B213A3B , $3F2C2A40 , ( 0*30 ) $4F545200 ,

: ch $FFFFFFF0 and unpack cf-ii + 1@ $FF and ;

: ii-cf string ( 0x20 ) $64632A00 , $7271706F , $2B2D6E6D , $2725232E , ( 0x30 3210 ) $1B1A1918 , ( 7654 ) $1F1E1D1C , ( ..98

) $28292120 , $2F6C6B6A , ( 0x40 CBA@ ) $3A43352C , ( GFED ) $3D3E3440 , ( KJIH ) $54523744 , ( ONML ) $3336393C , ( 0x50

SRQP ) $38314742 , ( WVUT ) $3F414632 , ( .ZYX ) $58563B45 , $75745973 , ( 0x60 cba. ) $0A130576 , ( gfed ) $0D0E0410 , ( kjih

) $24220714 , ( onml ) $0306090C , ( 0x70 srqp ) $08011712 , ( wvut ) $0F111602 , ( .zyx ) $77260B15 , $62617879 ,

: chc $FFFFFFE0 + ii-cf + 1@ $FF and ;

: tst #2000 block dup #4 * #-1 + $60 for $01 + $80 i negate + over 1! next drop dump ; #51 MagentaV qch

: rr ( c-c ) qch ! $20 $60 for $01 + dup chc qch @ negate + drop 0if pop drop ; then next $7F and ;

\ Block 95

\ ( Convert colorforth chars to and from ASCII )

\ : cf-ii ( conversion table )

\ : ch ( convert colorforth character to ASCII )

\ : ii-cf ( conversion table )

\ : chc ( convert ASCII to colorforth )

\ : tst ( create a table of ASCII characters )

\ : r ( scan the ii-cf table to perform cf-ii . Used to cross-reference the two tables )

\ : info ( display the ASCII version information in the last 64 bytes of block 11 . Type u to see more . )

\    ( dump takes a byte address )

\ Block 96

( App: RTC Real Time Clock ) empty

: bcd ( -c ) rtc@ #16 /mod #10 * + ;

: hms ( -n ) lo #4 bcd #100 * #2 bcd + #100 * #0 bcd + ;     s

: ymd ( -n ) lo #9 bcd #2000 + #100 * #8 bcd + #100 * #7 bcd + ;

: day ( -c ) lo #6 bcd ;

: crlf ( Port Dump )

: one ( n-n ) space yellow dup rtc@ h.2 blue space dup . cr ;

: lines ( sn- ) for one #-1 + next drop ;

: ok show page text cr #15 #16 lines white cr ymd .

hms . day . keypad ;

: h

keypd nul nul quit      nul nul nul nul

nul nul nul nul         nul nul nul nul

nul nul nul nul         nul nul nul nul

nul nul nul nul nul

$00250000 , #0 , #0 , #0 , #0 , #0 , #0 ,

ok

\ Block 97

\ ( RTC Real Time Clock )

\ : . ( displays the PC clock registers )

\ : bcd bcd-n ( bcd to binary )

\ : hms -n ( hours+mins+secs )

\ : ymd -n ( year+month+day )

\ : day -n ( day of the week )

\ : rtc ( display the Real Time Clock registers )

\ : one ( display one line )

\ : lines ( display n lines starting at s )

\ : ok ( display task )

\ Block 98

( LAN ) empty $03F8 nload init

: no block #4 * #1024 ;

: send no for dup 1@ xmit #1 + next drop ;

: receive no for rcv over 1! #1 + next drop ;

: no #18 #7 #18 * ;

: backup no for dup send #1 + next drop ;

: accept no for dup receive #1 + next drop ;

\ Block 99

\

\ Block 100

( Serial 3f8 2e8 1050 ) macro

: 1@ $8A 2, ;

: 1! a! $0288 2, drop ; forth

: r #0 + + ;

: 9600 #12 ;

: 38400 #3 ;

: 115200 #1 ;

: b/s $83 #3 r p! 38400 #0 r p! #0 #1 r p! #3 #3 r p! ;

: init b/s ( 16550 ) #1 #2 r p! #0 #4 r p! ;

: xmit ( n ) #5 r p@ $20 and drop if #0 r p! ; then pause xmit ;

: cts #6 r p@ $30 and $30 xor drop if cts ; then xmit ;

: st #6 r p@

: xbits $30 and $10 / dup #1 and 2* 2* + 2/ ;

: st! #4 r p! ;

: ?rcv #5 r p@ #1 and drop if #0 r p@ then ;

: rcv ?rcv if ; then pause rcv ; lblk @ edit

\ Block 101

\

\ : 1@ a-n ( fetch byte from byte address )

\ : 1! na ( store byte to byte address )

\ : r n-p ( convert relative to absolute port address. Base port on stack at compile time. Compiled as literal at yellow

\-green transition )

\ : 9600

\ : 115200 ( baud-rate divisors. These are names, not numbers )

\ : b/s ( set baud rate. Edit to change )

\ : init ( initialize uart )

\ : xmit n ( wait for ready and transmit byte )

\ : cts n ( wait for clear-to-send then xmit )

\ : st -n ( fetch status byte )

\ : xbits n-n ( exchange status bits )

\ : st! n ( store control byte )

\ : ?rcv ( fetch byte if ready. Set flag to be tested by ) if

\ : rcv -n ( wait for ready and fetch byte )

\ Block 102

( App: Colors ) empty

#4210752 MagentaV col #4210752 MagentaV del

: lin dup 2/ 2/ dup 2* line ;

: hex xy @ #7 and over 2/ for lin #7 + next over for lin next swap 2/ for #-7 + lin next drop ;

: +del del @ nop

: petal and col @ + $00F8F8F8 and rgb color #100 hex ;

: -del del @ $00F8F8F8 xor $00080808 + ;

: rose #0 +del #-176 #-200 +at $00F80000 -del petal #352 #-200 +at $00F80000 +del #-264 #-349 +at $F800 -del petal #176 #-200

+at $F8 +del #-176 #98 +at $F8 -del petal #176 #-200 +at $F800 +del ;

: ok show page #512 #282 at rose text col @ h. space del @ $FF and h. keypad ; nload ok h e

\ Block 103

\ ( Draws 7 hexagons. Colors differ along red, green and blue axes. )

\ : col ( color of center hexagon )

\ : del ( color difference )

\ : lin n ( draws 1 horizontal line of a hexagon )

\ : hex n ( draws top, center and bottom. Slope 7 x to 4 y is 1.750 compared to 1.732 )

\ : +del n ( increment color )

\ : -del n

\ : petal n ( draw colored hexagon )

\ : rose ( draw 7 hexagons )

\ : ok ( describe screen. Center color at top )

\ Block 104

( Colors keypad )

: in del @ 2* $00404040 min del ! ;

: out del @ 2/ $00080808 max del ! ;

: r $00F80000

: +del del @

: +col and col @ + $00F8F8F8 and col ! ;

: g $F800 +del ;

: b $F8 +del ;

: -r $00F80000 -del +col ;

: -g $F800 -del +col ;

: -b $F8 -del +col ;

: nul ;

: h keypd nul nul quit nul -r -g -b nul r g b nul out nul nul in nul nul nul nul nul nul nul nul nul nul nul nul $00250000

, $00626772 dup , , $2B00002D , #0 , #0 , #0 ,

\ Block 105

\

\ : in ( increment color difference )

\ : out ( decrement it )

\ : r

\ : g

\ : b ( increment center color )

\ : -r

\ : -g

\ : -b ( decrement it )

\ : +del ( redefine with ; )

\ : +col ( change center color )

\ : nul ( ignore )

\ : h ( describe keypad )

\ Block 106

( App: Wood ) empty #125810090 MagentaV x #-1123891786 MagentaV y

#8286477 MagentaV inc #33554432 MagentaV frame #39 MagentaV dep #65056 MagentaV hole

: h0 #400000 inc ! #15 dep !

: home inc @ scrnw #2 / * negate x     s ! inc @ scrnh #2 / * y ! ; macro

: f* $2EF7 2, #26 shr $E2C1 2, #6 1, $C20B 2, nip ;

: w! a! $00028966 3, drop ; forth

: wf+ frame @ w! #2 frame +! ;

: om negate $FF + ;

: o5 om $03 shr $07E0 xor ;

: o4 $FC and #3 shl $1F xor ;

: o3 om $F8 and #8 shl $1F xor ;

: o2 #3 shr $F800 xor ;

: o1 om $FC and #3 shl $F800 xor ;

: o0 $F8 and #8 shl $07E0 xor ;

: order jump o0 o1 o2 o3 o4 o5 o0

: hue #8 shl #26 / dup $FF and swap #8 shr order ;

: vlen dup f* swap dup f* + ;

: vdup over over ;

: vndp push push vdup pop pop ;

: itr over dup f* over dup f* negate + push f* 2* pop swap v+ over 2* + 2/ vndp + + ;

: data ; #4 +load ok draw h

\ Block 107

\ ( Display an imitation pine blockboard screen )

\

\ ( This is based on a skewed Mandelbrot set with )

\ ( modified colors )

\ Block 108

( App: Mandelbrot Set ) empty

#-204800000 MagentaV x #153600000 MagentaV y #400000 MagentaV inc

#34 MagentaV dep #33554432 MagentaV frame #0 MagentaV hole

: h0 #400000 inc ! #34 dep !

: home inc @ scrnw #2 / * negate x     s ! inc @ scrnh #2 / * y ! ; macro

: f* $2EF7 2, #26 shr $E2C1 2, #6 1, $C20B 2, nip ;

: w! a! $00028966 3, drop ; forth

: wf+ frame @ w! #2 frame +! ;

: hue ( n-n ) #8191 * ; dup dup + dup dup + + + dup dup + dup dup    ef + + ; #3142 * ; @ ;    ef

: vlen dup f* swap dup f* + ;

: vdup over over ;

: vndp push push vdup pop pop ;

: itr over dup f* over dup f* negate + push f* 2* pop swap v+ ;

: x: ( c- ) emit $3D emit ;

: data text #0 #0 at $78 x: x @ . $79 x: y @ . $69 x: inc @ . $64 x: dep @ . ; nload ok draw h

\ Block 109

\

\ Block 110

( Mandelbrot Set )

: o 0 0 dep @ #1 max for vndp itr vdup vlen $F0000000 + drop -if *next drop drop hole @ ; then drop drop pop hue ;

: mh x @ swap scrnw for o wf+ inc @ u+ next nip ;

: mv y @ scrnh for mh inc @ negate + next drop ;

: +d #2 dep +! : -d #-1 dep +! dep @ #1 max dep !

: draw vframe frame ! mv data ;

: ok c show keypad ;

: l inc @ scrnw #1 - #8 */ negate x +! draw ;

: u inc @ scrnh #1 - #8 */ y +! draw ;

: d inc @ scrnh #1 - #8 */ negate y +! draw ;

: r inc @ scrnw #1 - #8 */ x +! draw ;

: +z inc @ #3 max dup scrnw #1 - #8 */ x +! dup scrnh #1 - #8 */ negate y +! #3 #4 */ #3 max inc ! draw ;

: -z inc @ #10000000 min dup scrnw #1 - #8 */ negate x +! dup scrnh #1 - #8 */ y +! #4 #3 */ inc ! draw ;

: hh home draw ; : hh2 h0 draw ;

: h keypd nul nul quit nul -d nul nul +d l u d r -z hh hh2 +z nul nul nul nul nul nul nul nul nul nul nul nul $2500 , $2B00002D

, $13121110 , $2B30482D , #0 , #0 , #0 ,

\ Block 111

\ ( More Mandelbrot )

\ ( ludr move the cursor left right up down )

\ ( - + top row change depth detail )

\ ( - + bottom row change zoom )

\ ( h centres the image to the home location )

\ ( 0 resets depth and zoom )

\ Block 112

( App: words )

#74878 MagentaV from

#10315920 MagentaV tobl

#16 MagentaV num

#10010 MagentaV blkn

: tost ( u-- ) tobl @ ! #4 tobl +! ;

: tb $25C7C00E tost ; ( blue tab token )

: docr $9080000E tost ; ( blue cr token )

: setbl ( u-- ) blkn ! blkn @ block dup #1024 erase tobl ! #0 num ! ;

: newblk blkn @ #2 + setbl ;

: one ( -- ) from @ @ $FFFFFFF0 and $09 or tost tb #4 from +! #1 num +! ;

: chk1 num @ #6 mod #0 + 0if docr then drop ;

: chk2 num @ #96 mod #0 + 0if newblk then drop ;

: ww ( -- ) #10000 setbl

( FORTH ) $B189640B tost docr

ftha from ! fthn @ for one chk1 chk2 next

( MACRO ) newblk $8AC84C0B tost docr

maca from ! macn @ for one chk1 chk2 next

( BLUE ) newblk $C74CC80B tost docr

blua from ! blun @ for one chk1 chk2 next

#10000 edit ; ww

\ Block 113

\

\ Block 114

( App: Sounds make a noise ) empty

#25 MagentaV tempo #0 MagentaV mute #2259 MagentaV period

: tn ( ft- ) tempo @ * swap #660 #50 */

: hz ( tf- ) push #1000 #1193 pop */

: osc ( tp- ) dup period ! split $42 p! $42 p!

: tone ( t- ) mute @ #0 + drop if drop ; then $4F $61 p! ms $4D $61 p! #20 ms ;

: click #1 #90 osc ;

: t #3 tn ;

: q #8 tn ;

: c #16 tn ;

: 2tone #75 q #50 q ;

: h1 #50 c #54 q #50 q #45 c #60 c ;

: h2 #40 c #45 q #50 q #50 c #45 c ;

: h3 #54 c #60 q #54 q #50 c #45 q #40 q #50 t #45 t #50 t #45 t #45 #12 tn #40 q #40 #32 tn ;

: hh

: handel h1 h2 h3 ;

: piano #55 #7 for dup q #3 #2 */ next drop ;

: cetk #6 c #10 c #8 c #4 c #6 #32 tn ;

: bomb mute @ #0 + drop if ; then $4F $61 p! #500 for #1000 i invert + split $42 p! $42 p! #1 ms next $4D $61 p! #1 #32 tn

; handel

\ Block 115

\ ( Sounds : using the PC internal speaker )

\ : tempo ( in ms per 1/8 quaver )

\ : mute ( equals -1 to disable sound )

\ : period ( test only - value sent to hardware )

\ : tn ( ft- play f Hz for t * 11 ms )

\ : hz ( tf- play t ms at f Hz )

\ : osc ( tp- play t ms of period p )

\ : tone ( t- play the current tone for t ms )

\ : click ( makes a click )

\ : t ( triplet )

\ : q ( quaver )

\ : c ( crotchet )

\ : 2tone ( 2 tones )

\ : h1

\ : h2

\ : h3

\ : hh

\ : handel ( part of Handels Gavotte )

\ : piano

\ : cetk ( Close Encounters of the Third Kind )

\ : bomb ( - well sort of .... )

\ Block 116

( Colourblind Editor Display )

#1 MagentaV state $01 MagentaV state*

: +txt white $6D emit space ;

: -txt white $6E emit space ;

: +imm yellow $58 emit space ;

: -imm yellow $59 emit space ;

: +mvar yellow $09 emit $11 emit $05 emit $01 emit space ;

: txts string $03010100 , $07060504 , $09090901 , $0F0E0D0C , ( ; )

: tx ( c-c ) $0F and txts + 1@ $0F and ;

: .new state @ $0F and jump nul +imm nul nul nul nul nul nul nul +txt nul nul +mvar nul nul nul ;

: .old state* @ $0F and jump nul -imm nul nul nul nul nul nul nul -txt nul nul nul nul nul nul ;

here

: cb ( n-n ) #0 + 0if ; then tx

state @ swap dup state ! - drop if .old .new

state @ #0 + if dup state* ! then then ;

: cbs ( -- here ) #0 + $00 + cblind ! ;

\ Block 117

\

\ : state

\ : cb ( acts on a change of token type. It ignores extension tokens )

\ Block 118

( Graphics demo Todo: fix this! ) empty

#2 #22 +thru

: htm #116 load ( html ) ;

ok

\ Block 119

\ ( A graphics extension package )

\ : . ( Type ) ok ( after loading this block )

\ Block 120

( added macros ) forth

: mfill #24 for cr space #5 for rand32 h. space next next ;

: matrix show black screen green mfill keypad ;

\ Block 121

\ ( added macros )

\ : 1+ ( increment tos )

\ : 1- ( decrement tos )

\ : @b ( fetch byte from absolute addr. )

\ : @w ( fetch word from absolute addr. )

\ : @l ( fetch long from absolute addr. )

\ : !b ( store byte in absolute addr. )

\ : !w ( store word in absolute addr. )

\ : !l ( store long in absolute addr. )

\ : matrix ( What is the Matrix? )

\ : ver ( returns the address of the CFDOS version - use as ) ver dump

\ Block 122

( Stack juggling + misc. )

: v- ( v-v ) push invert 1+ u+ pop invert 1+ + ;

: vn push rot less if rot pop -rot ; then -rot pop ; #2222 MagentaV pen #236986408 MagentaV bs

: vloc ( xy-a ) scrnw 2* * over + + vframe + ;

macro

: @w $8B66 3, ;

: !w a! $00028966 3, drop ;

forth

: point ( xy- ) pen @ swap w! ;

: at? ( -xy ) xy @ $00010000 /mod swap ;

: @r ( a-a ) 1+ dup #4 u+ @ + ;

: !r ( aa- ) 1+ dup push negate #-4 + + pop ! ;

: select ( an- ) #5 * over + @r swap @r !r ;

\ Block 123

\ ( Stack juggling words. small and fast. )

\ : addr -a ( absolute address )

\ : rot abc-bca ( stack pictures are best .. )

\ : -rot abc-cab ( ..described with letters, in )

\ : tuck ab-bab ( ..this case. )

\ : 2swap abxy-xyab

\ : 2over abxy-abxyab

\ : 2dup ab-abab

\ : v- v1v2 - v1-v2 ( vector subtract. )

\ : vn vv-vv ( sort vectors so x1 is less x2 )

\ : vframe -addr ( address of screen. )

\ : pen -addr ( current color. )

\ : bs -addr ( base for elements )

\ : vloc xy-a ( convert xy into addr. )

\ : point xy- ( set point at xy to current pen. )

\ : at? -xy ( return current screen location. )

\ : @r a-a ( get absolute addr from jump/call )

\ : !r aa- ( set jump/call to absolute addr. )

\ : select an- ( select call n from table a. Store it in table call 0 )

\ Block 124

( new logo )

: .co $72 $6F $6C $6F $63 5* ;

: .fo $68 $74 $72 $6F $46 5* ;

: cf #27 dup at silver .co .fo #25 dup at red .co green .fo ;

: log1 show black screen text cf keypad ;

: ckb black #0 #740 at #1023 #767 box #800 #650 at #1023 #740 box ;

: grads #0 #128 for i 2* 1- rgb color dup #10 at #5 + dup #120 box next

iconw #21 * - #128 for #257 i 2* negate + dup #256 * + rgb color dup #10 at #5 + dup #100 box next drop ;

\ Block 125

\ ( New logo )

\ : log1 ( a simple text demo )

\ : ok ( the graphics demo )

\ Block 126

( Circles ) #-16977 MagentaV c-cd #0 MagentaV c-ff

: point4 #4096 * swap #4 * 2dup + 2/ negate bs @ + pen @ over w! over push + pen @ over w! + pen @ over w! pop negate + pen

@ swap w! ;

: opnts 2dup point4 2dup swap point4 ;

: d? c-cd @ ?f drop -if ; then dup invert c-cd +! 1- #1 c-ff ! ;

: cfl 1+ 1+ push pen @ swap pop 2/ for over over w! 1+ 1+ next drop drop ;

: cfl4 #4096 * swap #4 * 2dup + 2/ negate bs @ + swap 2dup cfl push + pop cfl ;

: fvrt ?f drop if cfl4 #0 c-ff ! ; then point4 ;

: fpnts 2dup c-ff @ fvrt 2dup swap cfl4 ;

: points opnts ;

: addr pop ;

: pntst addr points opnts fpnts ;

: framed pntst #1 select ;

: filled pntst #2 select ;

: circle ( rxyc- ) #0 c-ff ! pen ! #1024 * + 2* vframe + bs ! #0 swap dup negate c-cd !

: crcl less if points #1 u+ over c-cd +! d? crcl ; then points drop drop ;

\ Block 127

\ ( Circles )

\ : point4 ( .. all other words are internal. )

\ : points ( acts like a deferred word. )

\ : pntst ( table of calls to different point routines. Select alters ) points

\ : framed ( set ) circle ( to draw outlined circles. )

\ : filled ( set ) circle ( to draw filled circles. )

\ : circle rxyc- ( draw circle with radius ) r ( center ) xy ( in color ) c

\ Block 128

( lines )

#-1456 MagentaV ax #0 MagentaV ay #2048 MagentaV sx #2 MagentaV sy #31987278 MagentaV lbase

macro

: lp $8B909090 , $C88BADE8 , $205A8BAD , $232B8966 , $030578C0 , $185A0302 , $03084203 , $ECE2105A , ;

forth

: !base ( xy- ) #2048 * over + + vframe + lbase ! ;

: bline ( xy- ) abs 2* dup ay ! over 2* negate ax ! over

negate + swap 1+ pen @ ax a! lp drop ;

: ?xd ( vv-vv ) 2over 2over v- abs swap abs swap less

drop drop #-1 if 1+ then ?f drop ;

: !sy ( yn-y ) push ?f pop -if negate then sy ! bline ;

: xdom ( xyxy- ) 2swap !base #2 sx ! #2048 !sy ;

: ydom ( xyxy- ) swap 2swap swap !base swap #2048 sx !

#2 !sy ;

: aline ( vv- ) ?xd if vn 2over v- xdom ; then push push

swap pop pop swap vn 2over v- ydom ;

: line ( xy- ) at? 2over aline at ;

: frame ( xy- ) at? 2over drop over line 2over line 2swap

push drop over pop line line ;

\ Block 129

\ ( line drawing Do Not Mess With Variables. They are indexed by lp. )

\ : lp ( macro inner loop for speed. Draws point and moves location. )

\ : !base x y -- ( set base address )

\ : bline dx dy -- ( draw a line using bresenham x dominant )

\ : ?xd v1 v2 -- v1 v2 ( set flag if line is x-dominant )

\ : !sy dy n -- dy ( store n in sy set sign to match sign of dy )

\ : xdom x y dx dy ( draw an x-dominant line )

\ : ydom x y dx dy ( draw a y-dominant line )

\ : aline v1 v2 ( draw any straight line )

\ : line x y ( draw line from current at to xy. Moves at to given xy. )

\ : frame xy- ( trace outline of rectangle with corners at and xy. Pen position is not altered. )

\ Block 130

( Utils )

: xxcoy ( sf st ) $E7C1F88B , $368B560A , $B90AE6C1 , $0100 , $AD5EA5F3 , $C3AD 2,

: xrcopy ( sf sl st ) push dup push swap negate + pop swap pop over + swap for over over copy push 1- pop 1- -next drop drop

;

\ Block 131

\ ( Utils )

\ : copy from to- ( copy from to block numbers. Unlike orig copy; no change to blk )

\ : rcopy first last to- ( multiple block copy routine )

\ Block 132

( fillstack ) #1114112 MagentaV fstak $00 MagentaV fstakn

: fstini ( - ) $0400 block fstak ! 0 fstakn ! ;

: fpop ( -uuu ) fstak @ #3 for dup @ swap cell- next

fstak ! #-3 cells fstakn +! ;

: fpsh ( uuu- ) #3 for cell fstak +! fstak @ ! next

#3 cells fstakn +! ;

: fst? ( - ) fstakn @ ?f drop ; fstini

macro

: 2- 1- 1- ;

: 2+ 1+ 1+ ;

forth

: 5drop ( uuuuu- ) drop drop drop drop drop ;

: rtre ( a-n ) #2048 #1 - and negate #2048 + ;

: enstak ( dlrlr-dlrlr ) 2- #4 pick dup #3 pick + over

#3 pick + fpsh over #4 pick negate + 2+ drop

-if #4 pick negate dup #3 pick +

\ Block 133

\ ( fillstack: stack of spans to fill. )

\ : fstini ( initialize )

\ : fpop ( pop the next element from the stack )

\ : fpsh ( push element on the stack )

\ : fst? ( set 0 flag if empty )

\ : 2- ( screen pixels are 2 bytes. )

\ : 2+

\ : 5drop ( unload forth stack. )

\ : rtre a-n ( return remaining to right screen edge. )

\ : enstak dlrlr-dlrlr ( push a span or element onto the stack. Also push a left hand direction reversal and a right hand

\ reversal if needed. )

\ Block 134

( area filling ) #25702 MagentaV tfc #14660 MagentaV fc

: pset ( a-f ) dup dup w@ $FFFF and tfc @ negate + drop

if drop 0 ; then fc @ swap w! 0 1+ ;

: bcup ( a-a ) dup #2048 #1 - and 2- begin -if drop ; then

push 2- pset drop pop if 2- *end then drop 2+ ;

: ispan pset if ; then push enstak pop ;

: xgr dup negate #3 pick + drop ;

: nispan ( dlrlx- ) xgr -if 5drop pop pop pop drop drop

drop ; then pset if push nip dup pop then ;

: dosp ( dlrlx-dlrlxi ) jump nispan ispan ;

: sha2 over rtre begin ( dlrlxic ) -if drop ; then push

dosp #2 u+ pop 2- end

: sha1 ( dlr- ) over pset over ( dlrxil ) if bcup ( dlrxil ) then

swap push swap 2+ pop ( dlrlxi ) sha2 ?f drop

if enstak then 5drop ;

: sha begin fst? if fpop sha1 *end then ;

: fsln ( a-lr ) dup bcup swap dup rtre

begin -if drop ; then push pset drop if

2+ pop 2- *end then pop drop 2- ;

: afill ( xyc- ) fstini fc ! vloc dup w@ $FFFF and tfc !

fsln over over #-2048 u+ #-2048 + #-2048 -rot fpsh

#2048 u+ #2048 + #2048 -rot fpsh sha ;

: afill drop drop drop ;

\ Block 135

\ ( area filling )

\ : pset a-0/1 ( set pixel at a, if pixel equals tfc. Return 0 if not, 1 if pixel was set. )

\ : bcup a-a ( adjust a until left edge is found. Limited to screen edge. )

\ : ispan ( stack if the right edge is found. )

\ : xgr ( Set neg flag if x is greater then parent-r )

\ : nispan ( exit if beyond right edge of span, else start a new span. )

\ : dosp dlrlx - dlrlxi ( jump table. )

\ : sha2 ( let x go over each pixel and set it or start/end new spans. )

\ : sha1 ( starting at left edge, find the new left edge and init x to next pixel. stack if run into right screen edge while

\ in span. )

\ : sha ( pop the next span and color it. )

\ : fsln a-lr ( Starting at screen address a, find the left edge and right edge of the seed line. Color it in the proces

\s. )

\ : afill xyc ( starting with screen location xy, and color c, fill the color found there with c until the color found change

\s. )

\ Block 136

( random ) #-1896373196 MagentaV rsav #-526774649 MagentaV rseed

: rand ( -- ) time rsav ! $E09A0E87 rseed ! ;

: ror ( u-u ) $D3ADC88B , $C3C8 2,

: random ( w-w ) push rseed @ #0 #32 for 2* swap 2* swap -if rsav @ xor then next nip #15 ror dup rsav ! abs pop mod abs

; rand

: tt $0100 random ;

\ Block 137

\ ( random )

\ : rand - ( set random variables )

\ : ror nm-n ( rotate n m times right )

\ : random n-0..n-1 ( return a random number range 0..n-1 limited to a 16 bit number. )

\ Block 138

( demos )

: xlate #384 + #512 u+ ;

: xat xlate at ;

: xline xlate line ;

: 4lines over #0 xat #0 over xline over - #0 xline negate #0 swap xline #0 xline ;

: art #70 for #71 i - #5 * i #5 * 4lines next ;

: radius #8 ;

: lrc push dup dup + negate pop + random + ;

: shade 2over #2 + 2over drop #3 + #0 circle circle ;

: dotty filled #100 for radius random dup #397 lrc #621 + over #176 lrc #121 + $FFFF random shade next ;

: blbx black #6 #121 at #404 #299 box ; #-17 MagentaV xyzz

: fillit #-1 xyzz +! xyzz @ #200 + drop -if blbx 0 xyzz ! then framed #3 for #8 random #2 + dup #398 lrc #6 + over #178 lrc

#121 + $FFFF circle next

; #6 #210 $FFF0 random afill ;

\ Block 140

( new logo 2 )

: lnes framed #20 for i 2* #40 + #250 #584 $FF07 circle next filled #30 #250 #584 $F800 circle framed $FFFF pen ! #620 #120

at #1020 #300 frame #5 #120 at #405 #300 frame ;

: ok show black screen grads lnes text cf dotty fillit ckb keypad ; ( ok )

\ Block 141

\ ( New logo )

\ : log1 ( a simple text demo )

\ : ok ( the graphics demo )

\ Block 142

( html0 ) #80 load #2222119 MagentaV h-dd #0 MagentaV ppt macro

: 2/s ?lit $E8C1 2, 1, ; forth

: temit h-dd @ !b #1 h-dd +! ;

: tspc $20 temit ;

: .dc ?f #1 -if - then swap abs

: dcl #10 /mod swap $30 + push ?f 0if drop ?f drop -if $2D temit then pop temit ; then dcl pop temit nop ;

: .hx $39 over #15 and $30 + less nip if $27 + then push #4 2/s 0if drop pop temit ; then .hx pop temit nop ;

: strt dup @b $FF and if temit 1+ strt ; then drop drop ;

: str: pop strt ;

: header str: $6D74683C , $3C0A3E6C , $6B6E696C , $6C657220 , $7974733D , $6873656C , $20746565 , $65707974 , $6574223D ,

$632F7478 , $20227373 , $66657268 , $3D 1, $6C6F6322 , $6F66726F , $2E687472 , $22737363 , $703C0A3E , $0A3E 3,

: trailer str: $74682F3C , $0A3E6C6D , $00 1,

\ Block 143

\ ( html0. Block 80 has ascii conversion tables. )

\ : h-dd ( data destination. ) ppt ( pre- parsed type. )

\ : 2/s ( macro, right shift by n. )

\ : temit c- ( emit char to target. )

\ : tspc ( emit space )

\ : .dc n- ( signed decimal print. Recursive! )

\ : dcl ( dec print loop. )

\ : .hx n- ( unsigned hex print. Also recursive. Both routines have no leading zeroes. )

\ : strt a- ( Print bytes from address until first null byte. )

\ : str: ( Output what follows up to null byte. )

\ : header ( Lay down html header to display blocks. The header is very minimal. It expects colorforth.css in the same direct

\ory. )

\ : trailer ( Closing html stuff. )

\ Block 144

( html1 )

: .code 1- drop -if ; then str: $6F632F3C , $003E6564 ,

: .all str: $646F633C , $6C632065 , $3D737361 , $00 1,

: same? ppt @ over ppt ! swap over - 1+ + drop ;

: comn same? 0if drop tspc pop drop ; then .code .all ;

: .def str: $3E666564 , $20 2,

: .com #2 comn str: $3E6D6F63 , $20 2,

: .chx #3 comn str: $3E786863 , $20 2,

: .exe #4 comn str: $3E657865 , $20 2,

: .xhx #5 comn str: $3E786878 , $20 2,

: .cpm #6 comn str: $3E6D7063 , $20 2,

: .var #7 comn str: $3E726176 , $20 2,

: .txt #8 comn str: $3E747874 , $20 2,

: .txc #9 comn str: $3E637874 , $20 2,

: .tac #10 comn str: $3E636174 , $20 2,

\ Block 145

\ ( html1 )

\ : .code n- ( output /code in brackets if n is larger then 0. )

\ : .all ( common part to start a new code tag. )

\ : same? n-o ( set ppt to the new type. Return the old type with flags set from comparison. )

\ : comn n- ( if this is a new tag, close prev tag and print common part. If not: print space AND EXIT CALLER )

\ : .def ( Each of these words correspond to a )

\ : .com ( .. code tag as defined in colorforth.css )

\ : .chx ( .. The numbers are positional, and bare )

\ : .exe ( .. no correspondence to the pre parsed )

\ : .xhx ( .. types. They will output if a change )

\ : .cpm ( .. in tag is required. Comn will exit )

\ : .var ( .. by doing a pop-drop if the tag is the )

\ : .txt ( .. same. )

\ : .txc

\ : .tac

\ Block 146

( html2 )

: .str ch if temit .str ; then drop drop ;

: bs1 #0 ppt ! str: $3E72683C , $6C627B0A , $206B636F , $00 1,

: bs2 str: $643C0A7D , $63207669 , $7373616C , $786F623D , $0A3E 3,

: bend ppt @ .code str: $69642F3C , $000A3E76 ,

: .br 1- drop -if ; then str: $3E72623C , $0A 2,

: pp0 .str ;

: pp1 .exe .str ;

: pp3 ppt @ dup .code .br #1 ppt ! .all .def .str ;

: pp4 .com .str ;

: pp7 .cpm .str ;

: pp9 .txt .str ;

: ppa .txc .str ;

: ppb .tac .str ;

: ppc .var .str 1+ dup @ .com .dc ;

\ Block 147

\ ( html2 )

\ : .str n- ( Unpack n and print as ascii. )

\ : bs1 ( clear the type and print html stuff for the start of a block. )

\ : bs2 ( second half of block header. )

\ : bend ( Block end html stuff. )

\ : .br n- ( Html line break, if n larger then 0 )

\ : pp0 ( The preparsed words in a block are )

\ : pp1 ( .. printed by the ppn words. Eg pp0 is )

\ : pp3 ( .. word continuation pp1 is for executed )

\ : pp4 ( .. words, etc. They unpack and print. )

\ : pp7 ( .. They also print html tags. )

\ : pp9

\ : ppa

\ : ppb

\ : ppc

\ Block 148

( html3 )

: dbn push 1+ dup @ pop ?f drop ;

: sln dup 2/ 2/ 2/ 2/ 2/ swap #16 and drop ;

: xnb if .xhx .hx ; then .exe .dc ;

: cnb if .chx .hx ; then .com .dc ;

: pp2 dbn xnb ;

: pp5 dbn cnb ;

: pp6 sln cnb ;

: pp8 sln xnb ;

: ppdo jump pp0 pp1 pp2 pp3 pp4 pp5 pp6 pp7 pp8 pp9 ppa ppb ppc ;

: index dup #15 and dup push or pop ;

: dblk dup bs1 .dc bs2 block begin dup @ ?f 0if drop drop bend ; then index ppdo 1+ end

: hbuf #2000 block ;

: html hbuf #4 * h-dd ! header swap over for over i - 1+ + over + dblk next drop drop trailer hbuf h-dd @ #3 + #4 / over

- 1+ + #3 for tspc next ;

\ Block 149

\ ( html3 )

\ : dbn an-an ( Fetch next word. Set hex flag. )

\ : sln n-n ( Make full word and set hex flag. )

\ : xnb n- ( print n as hex/dec executed number. )

\ : cnb n- ( print n as hex/dec compiled number. )

\ : pp2 an-a ( A double executed number. )

\ : pp5 an-a ( A double compiled number. )

\ : pp6 n- ( A single compiled number. )

\ : pp8 n- ( A single executed number. )

\ : ppdo ( Table of words. The index is the pre- parsed type type. )

\ : index n-ni ( extract index from n. )

\ : dblk b- ( print block b in html. )

\ : hbuf -a ( start of buffer. )

\ : html bn-al ( Output n blocks starting with block b in html. Leaves addr and length on the stack, so it can be saved using

\ ) file put ( on a floppy. )

\ Block 150

( html3 )

: dbn push 1+ dup @ pop ?f drop ;

: sln dup 2/ 2/ 2/ 2/ 2/ swap #16 and drop ;

: xnb if .xhx .hx ; then .exe .dc ;

: cnb if .chx .hx ; then .com .dc ;

: pp2 dbn xnb ;

: pp5 dbn cnb ;

: pp6 sln cnb ;

: pp8 sln xnb ;

: ppdo jump pp0 pp1 pp2 pp3 pp4 pp5 pp6 pp7 pp8 pp9 ppa ppb ppc ;

: index dup #15 and dup push or pop ;

: dblk dup bs1 .dc bs2 block begin dup @ ?f 0if drop drop bend ; then index ppdo 1+ end

: hbuf #2000 block ;

: html hbuf #4 * h-dd ! header swap over for over i - 1+ + over + dblk next drop drop trailer hbuf h-dd @ #3 + #4 / over

- 1+ + #3 for tspc next ;

\ Block 151

\ ( html3 )

\ : dbn an-an ( Fetch next word. Set hex flag. )

\ : sln n-n ( Make full word and set hex flag. )

\ : xnb n- ( print n as hex/dec executed number. )

\ : cnb n- ( print n as hex/dec compiled number. )

\ : pp2 an-a ( A double executed number. )

\ : pp5 an-a ( A double compiled number. )

\ : pp6 n- ( A single compiled number. )

\ : pp8 n- ( A single executed number. )

\ : ppdo ( Table of words. The index is the pre- parsed type type. )

\ : index n-ni ( extract index from n. )

\ : dblk b- ( print block b in html. )

\ : hbuf -a ( start of buffer. )

\ : html bn-al ( Output n blocks starting with block b in html. Leaves addr and length on the stack, so it can be saved using

\ ) file put ( on a floppy. )

\ Block 152

( simpler and slower bresenham line drawing. For reference. ) #-360 MagentaV ax #0 MagentaV ay #2 MagentaV sy #0 MagentaV sw

: bpoint push 2dup sw @ ?f drop if swap then point pop ;

: bline abs 2* dup ay ! over 2* negate ax ! over negate + swap 1+ for bpoint ?f +if sy @ u+ ax @ + then ay @ + push #1 u+

pop next drop drop drop ;

: ?xd 2over 2over v- abs swap abs swap less drop drop #-1 if 1+ then ?f drop ;

: !sy push ?f pop -if negate then sy ! bline ;

: xdom #0 sw ! #1 !sy ;

: ydom #1 sw ! #1 !sy ;

: aline ?xd if vn 2over v- xdom ; then push push swap pop pop swap vn 2over v- ydom ;

\ Block 154

\ Block 155

\ ( fillstack: stack of spans to fill. )

\ : fstini ( initialize )

\ : fpop ( pop the next element from the stack )

\ : fpsh ( push element on the stack )

\ : fst? ( set 0 flag if emtpy. )

\ : pick ( copy n from the stack. )

\ : 2- ( screen pixels are 2 bytes. )

\ : 2+

\ : 5drop ( unload forth stack. )

\ : rtre a-n ( return remaining to right screen edge. )

\ : enstak dlrlr-dlrlr ( push a span or element onto the stack. Also push a left hand direction reversal and a right hand

\ reversal if needed. )

\ Block 160

( Timing ) empty macro

: out $E1E6 2, ; forth

: tare time invert #1000 for next time + ;

: tare+ time invert push #1000 for dup next c pop time + ;

: test +     s #1000 for out next time + ; ( next 3 loop 5.7 /next 2 /swap 25 swap 7.2 ) macro

: c! $C88B 2, drop here ;

: loop $49 1, $75 1, ( e2 ) here invert + 1, ; forth

: try time invert #1000 c! loop time + ;

\ Block 162

( Spy ) empt $03F8 #54 load init

: ry #5 r p@ ; nload init

: buffer #2000 block ; #2000 #1 wipes #0 MagentaV buf #0 buf !

: b! swap $FF and + buf @ buffer + ! #1 buf +! ;

: dev r2 if dup xmit $0100 b! dev ; then ;

: pc ?rcv if dup x2 0 b! pc ; then ;

: relay s2 st s2! st! dev pc ;

: .1 $0F and digit ;

: .byte dup $10 / .1 .1 ;

: traffic text buffer buf @ #1 max #400 min for dup @ green $0100 ? if red then .byte #1 + next drop ;

: ok show black screen relay traffic keypad ;

: k show black screen relay keypad ;

: q #6000 for relay next ;

: test st! st ; #84 load

\ Block 164

( Serial 2 )

: r $02F8 + ;

: b/s $83 #3 r p! 9600 #262 #0 r p! #0 #1 r p! #3 #3 r p! ;

: init b/s ( 16550 ) #1 #2 r p! #0 #4 r p! ;

: x2 #5 r p@ $20 and drop if #0 r p! ; then x2 ;

: c2 #6 r p@ $30 and $30 or drop if c2 ; then x2 ;

: s2 #6 r p@ xbits ;

: s2! #4 r p! ;

: r2 #5 r p@ #1 and drop if #0 r p@ ; then ;

\ Block 166

( Dynapulse 200m )

: send pop swap for dup 1@ x2 #1 + next drop ;

: reset #2 send $2323 ,

: 1st #12 send $37269A12 , $39027AFD , $23C75680 ,

\ Block 168

( Test sidt and lidt )

#7168 MagentaV vidt sidt vidt !

: resi cli vidt @ lidt ;

\ Block 169

\ ( This block is used by the next block as the interrupt vector table. )

\ Block 170

( Interrupts ) macro

: 1ld ( n ) ?lit $B9 1, , ;

: p! ( na ) a! $EE 1, drop ;

: 2push $5250 2, ;

: 2pop $585A 2, ;

: forth: 2push $00BE5651 3, ivec $0100 + a, ;

: ;forth $595E 2, 2pop ;

: clear $20E620B0 , ;

: 8clear $A0E620B0 , $20E6 2, ;

: i; $CF 1, ; forth

: interrupt ( n ) 2* 2* 2* ivec + here $FFFF and $00080000 + over ! here $FFFF0000 and $8E00 + swap #4 + ! ;

: ifill ( an ) for dup interrupt #1 + next drop ; $00 $70 ifill

: ignore i; $20 $08 ifill

: ignore 2push clear 2pop i; $28 $08 ifill

: ignore 2push 8clear 2pop i; $00 interrupt

: 0div $7FFFFFFF 1ld i;

\ Block 171

\

\ : idt -a ( table of 2-word interrupts. Edit convenient block number )

\ : 1ld n ( load register 1 with literal )

\ : lidt ( load interrupt descriptor table from byte address on stack )

\ : 2push ( save registers 0 and 2 )

\ : 2pop ( restore 2 and 0 )

\ : forth: ( save registers used by Forth )

\ : ;forth ( restore registers used by Forth )

\ : clear ( store 20 to port 20 to clear irq 0-7 )

\ : 8clear ( also 20 to port a0 to clear irq 8-f )

\ : i; ( return from interrupt - restore flags )

\ : !idt b ( execute lidt )

\ : interrupt n ( construct interrupt to ) here. ( Avoid yellow-green literal with red comment )

\ : ifill an ( n entries in default interrupt table )

\ : ignore ( clear ) ( grey = $01644001  ) ( interrupt. Doesnt clear the device )

\ : 0div ( make divisor +infinity, quotient 0 )

\ Block 172

( Admtek Comet An983b ) macro

: align here #7 and #3 xor drop if nop align ; then ; forth

: array pop 2/ 2/ ;

: us ( n ) khz @ #1000 #3 * / * for next ;

: r ( n-a ) $DB000000 + 2/ 2/ ;

: rom ( a-n ) $A4 + r @ ;

: 3rom ( nnn ) #4 rom #0 rom dup #16 for 2/ next swap ;

: reset #1 $00 r ! #1000 us ;

: frag #0 , $02000000 , $00 , here #4 + , ;

: tx align array frag frag frag frag frag frag

: n tx #1 + ;

: a tx #2 + ; #16 MagentaV f

: fr! f @ + ! ;

: first ( an ) #0 f ! $20000000 or

: send ( an ) $01000000 or n fr! a fr! $80000000 tx fr! #4 f +! ;

: last ( an ) $42000000 or send #1 us

: poll #-1 $08 r ! ;

\ Block 173

\

\ : array -a ( returns word-aligned address in dictionary )

\ : us n ( delay n microseconds. Edit cpu clock rate )

\ : r n-a ( word address of register. Edit base address from ) north ( PCI device configuration )

\ : rom a-n ( fetch 2 bytes of ethernet id )

\ : 3rom -nnn ( 3 byte-pairs of id. )

\ : reset ( controller )

\ : tx -a ( transmit descriptor ring )

\ : n -a ( fragment length/control )

\ : a -a ( fragment address )

\ : send an ( fragment into descriptor queue )

\ : first an ( fragment. )

\ : last an ( fragment. Start transmission )

\ Block 174

( Receive ) #281880 MagentaV rxp

: rx align array $80000000 , $01000600 , $2000 block #4 * dup , here #4 + , $80000000 , $01000600 , $0600 + , rx #4 * ,

: init reset rx #2 * 2* $18 r ( receive ) ! #1 us tx #2 * 2* $20 r ( transmit ) ! #1 us $00202002 ( start ) $30 r ! #1 us

$00010040 $38 r ! sti #-1 $28 r ! ;

: link #3 + @ 2/ 2/ ;

: own? @ #0 or drop ;

: /int rxp @ $80000000 over ! link own? -if #-1 $28 r ! then ;

: rcvd rx nop

: wait dup own? -if link wait ; then dup rxp ! #2 + @ ;

: reg dup r @ h. space #2 h.n cr ;

: regs $B8 reg $A0 reg $98 reg $94 reg $78 reg $60 reg $48 #10 for dup reg #-8 + next drop ;

: ok show $00400000 rgb color screen text regs keypad ;

: rx1 $2000 block dump ;

: rx2 $2000 block $0180 + dump ; ok

\ Block 175

\

\ : rx -b ( receive descriptor ring )

\ : init ( ialize controller. Set tx/rx address/on and perfect match )

\ : link a-b ( next link in descriptor ring )

\ : own? a ( is this descriptor owned? )

\ : /int ( give up ownership of received packet , clear interrupt if no packet remains )

\ : rcvd -a ( return address of recieved packet )

\ : wait -b ( till packet received )

\ : reg a ( display register and address )

\ : regs ( display interesting registers )

\ : ok ( diagnostic display )

\ Block 176

( App: Ethernet ) empty

( interrupts ) #170 load

( hardware interface ) #172 load #174 load macro

: w $66 1, ;

: w@ $8B 2, ;

: w! $0289 2, drop ;

: *byte $C486 2, ; forth

: n@ w w@ $FFFF and *byte ;

: 2! a! w w! ;

: n! a! *byte w w! ;

: n, *byte 2, ;

: string pop ;

: packet string #-1 dup , 2, 3rom 2, 2, 2, #0 n,

: length ( n ) packet #12 + n! ;

: broadcast #-1 dup dup packet nop

: 3! swap over 2! #2 + swap over 2! #2 + 2! ;

: ethernet ( n ) length packet #14 first ;

: +ethernet ( -a ) rcvd #14 + ; fixthis

#2 #16 +thru breakhere ( todo fix this )

$2A interrupt

: serve forth: receive /int 8clear ;forth i; init ok discover

\ Block 177

\

\ : empty ( redefined to disable interrupts )

\ : w ( 16-bit prefix )

\ : w@ b-n ( fetch 16-bits from byte address )

\ : w! nb ( store 16-bits )

\ : *byte n-n ( swap bytes 0 and 1 )

\ : n@ b-n ( fetch 16-bit network-ordered number )

\ : 2! nb ( store 16-bit number )

\ : n! nb ( store 16-bit number in network order )

\ : n, n ( compile 16-bit number in network order )

\ : string -b ( returns byte address )

\ : packet -b ( ethernet packet header )

\ : dest -b ( destination field in packet )

\ : src -b ( source field )

\ : length n ( store length into packet )

\ : 3! nnnb ( store 3-word MAC )

\ : ethernet n ( send header with type/length )

\ : @ethernet -b ( return payload address of received packet )

\ Block 178

( ARP for a single correspondent ) macro

: move ( sdn ) $C189 2, drop $00C78957 3, drop $00C68956 3, $A4F3 2, $5F5E 2, drop ; forth

: . ( n ) 1, ;

: message string $01 n, $0800 n, $06 . $04 . $01 n,

: me 3rom 2, 2, 2, ( IP ) #0 . #0 . #0 . #0 .

: to #0 #0 #0 2, 2, 2, ( IP ) #0 . #0 . #0 . #0 .

: sender #8 + ;

: target #18 + ;

: dir #6 + ;

: ip #6 + w@ ;

: ar ( n ) message dir n! $0806 ( ARP ) ethernet message #28 last ;

: arp cli broadcast #1 ar sti ;

: -arp ( b-b ) dup #-2 + n@ $0806 or drop if ; then pop drop

: me? dup target ip message sender ip or drop if ; then dup sender packet #6 move

: query? dup dir n@ #1 or drop if ; then sender message target #10 move #2 ar ;

\ Block 179

\ ( Set ip addresses with Edit. Normal order, net bytes first )

\ : move sdn ( move n bytes from source to destination. Register 1 is used, 6 and 7 are saved )

\ : . n ( compile byte. Resembles URL punctuation )

\ : message -b ( 28-byte string )

\ : me ( comment marking my mac/ip address )

\ : to ( comment marking correspondent )

\ : sender

\ : target

\ : dir -b ( fields in either ) message ( or received message )

\ : ip b-n ( fetch ip address )

\ : ar n ( send query 1, or reply 2 )

\ : arp ( broadcast query )

\ : -arp b-b ( return if not ARP. Otherwise process and skip out. )

\ : me? b ( return if broadcast not for me. Save sender only in packet )

\ : query? b ( if a request, reply )

\ Block 180

( ipv4 )

: header align string $4500 n, #0 n, #1 n, #0 n, $FF11 n, #0 n, #0 , #0 ,

: length ( n ) header #2 + n! ;

: +id header #4 + dup n@ #1 + swap n! ;

: -sum for dup n@ u+ #2 + next drop dup $00010000 / + invert ;

: sum header #10 + n! ;

: checksum 0 sum #0 header #10 -sum sum ;

: source header #12 + ;

: destination header #16 + ;

: ip ( n-n ) dup #20 + $0800 ethernet length +id checksum header #20 send ;

: +ip dup #-2 + n@ $0800 or drop if pop ; then #20 + ;

\ Block 181

\ ( Set ip addresses with Edit. Normal order, net bytes first )

\ : header -a ( 40-byte ipv6 header )

\ : length n ( store 2-byte length in header )

\ : dest -a ( 4-byte destination ip address )

\ : src -a ( source ip )

\ : ip n ( send ip header embedded in ethernet packet )

\ : +ip b-b ( skip out if not IP. Otherwise return payload address )

\ Block 182

( UDP )

: xid 3rom + + ;

: b@ ( b-n ) w@ $FF and ;

: header string xid n, #0 n, #8 n, #0 n, #0 n,

: length ( n ) #8 + header #4 + n! ;

: port header #2 + n! ;

: from? over #-8 + n@ or drop ;

: udp ( n ) dup #8 + ip length ;

: +udp ( b-b ) dup #-11 + b@ #17 or drop if pop ; then #8 + ;

\ Block 183

\

\ : b@ b-n ( fetch byte )

\ : header -a ( 8-byte udp header )

\ : length n ( store length in header )

\ : port p ( set destination port )

\ : from? ap ( udp packet from port ) p ( ? )

\ : udp n ( send ip header for n-byte packet )

\ : +udp b-b ( skip out if not UDP. Otherwise return payload address )

\ Block 184

( DNS resolver ) $0CF42A44 MagentaV server #1671948608 MagentaV host

: msg string #0 , #1 n, #0 2, #0 , #1 n, #1 n,

: ptr? dup n@ $C000 and $C000 or drop ;

: skip ptr? if dup b@ if + #1 + skip ; then drop #1 + ; then #2 + ;

: length dup negate swap skip + ;

: 4! a! w! ;

: query server @ destination 4! #53 port dup length dup #16 + udp drop header #8 send msg #12 send send msg #12 + #4 last

;

: answer dup #12 + skip #4 + swap #6 + n@ ;

: resolve ( a-h ) #0 host ! query

: wait host @ #0 or if ; then drop wait ;

: rr+ #8 + dup n@ + #2 + ;

: -dns #53 from? if ; then pop drop answer

: rr #-1 + -if #-1 host ! ; then swap skip dup n@ #1 or drop if rr+ swap rr ; then

: address #10 + dup w@ host ! ;

\ Block 185

\ ( Assumtions )

\ : 1 ( a response contains one entry in the question section )

\ : 2 ( the first address in the answer section, if any, sufficiently resolves the query )

\ : server ( name server )

\ : host ( the resolved IP address )

\ : skip a-b ( skip past a domain field )

\ : length a-n ( length of a domain in bytes )

\ : query a- ( send DNS query to the DNS server )

\ : answer a-bn ( give the answer section and the number of resource records )

\ : resolve a-h ( resolve domain name to host address )

\ : wait -h ( wait for a response from the server )

\ : rr+ a-b ( skip a resource record )

\ : -dns ( dns packet recieved , search for address )

\ : rr a-b ( process resource record )

\ : address a-b ( set the host address )

\ Block 186

( Domain names ) #62 load macro

: 1! a! $0288 2, drop ;

: interp qdup $F889 2, ; forth

: word ch if 1, #1 u+ word ; then drop drop ;

: . here #0 1, interp #0 over @ #-16 and word #1 u+

: words over @ $0F ? if drop nip swap 1! ; then word #1 u+ words ;

: end #0 1, ;

: cf string . ( www ) . ( colorforth ) . ( com ) end

: google string . ( www ) . ( google ) . ( com ) end

: none string . ( none ) end

\ Block 187

\

\ : 1! xa- ( write byte at byte address )

\ : interp -a ( word address of next word to be interpreted )

\ : word w- ( compile packed word as ASCII characters )

\ : . ( compile counted ASCII string )

\ : words an- ( compile extentions words as ASCII )

\ : end ( of domain )

\ : none ( test of a non-existant domain )

\ Block 188

( DHCP client )

: fill for #0 , next ;

: msg align string $00060101 , xid , #5 fill 3rom 2, 2, 2, #0 2, #50 fill $6382 n, $5363 n, $00010135 3, $06030237 , #12

1, . ( colorforth ) $FF 2, #0 , $3204 n, #0 , $FF 1,

: eq over over or drop ;

: skip over #1 + b@ #2 + u+ ;

: find over b@ if eq if $FF or if drop skip find ; then then drop drop #2 + ; then drop #1 u+ find ;

: your #16 + w@ ;

: ack dup #6 find w@ server ! #3 find w@ message target #6 + 4! your dup source 4! message sender #6 + 4! #1 ar ;

: -dhcp #67 from? if ; then dup #4 + w@ xid or drop if ; then dup #240 + dup #53 find w@

: type #2 or if #7 or drop if ack then drop ; then drop

: offer #54 find w@ msg #261 + 4! your msg #267 + 4!

: request #272 $3604 $0103 msg #241 + n!

: bootp msg #259 + n! broadcast #-1 destination 4! #67 port udp header #8 send msg swap last ;

: discover #260 $FF00 bootp ;

\ Block 189

\

\ : xid -v ( a unique identifier used in all DHCP correspondence with this client )

\ : fill n ( fill ) n ( words )

\ : msg ( the DHCP message , both discover and request are contained , discover is ends at ) $FF 2,

\ : eq xy-xy ( test equality )

\ : skip at-bt ( skip DHCP option )

\ : find at-b ( find option of type ) t ( in option list )

\ : your a-h ( IP address )

\ : ack ao ( server acknowledge , assign your IP , router IP , and DNS server IP )

\ : -dhcp a ( receive DHCP packet with ) xid

\ : type aot ( recieve offer ) 2 ( or ack ) 5

\ : offer ao ( recieved an offer , send a request )

\ : request ( request the offered parameters )

\ : bootp nt ( send a discover or request message )

\ : discover ( broadcast a discover message )

\ Block 190

( ICMP )

: header string $0800 n, $00 n, $00 ,

: icmp dup #-34 + b@ #1 or drop if ; then ;

: ping #8 ip header #8 last ;

\ Block 191

\ ( Client can get or put blocks to server )

\ : payload n-bn ( 2 bytes were appended to UDP header for block number )

\ : +put nn ( send block number. Append block as last fragment. Packet length distinguishes two messages )

\ : it b ( move 1024 bytes from packet to offset block )

\ : -got b-b ( if a 2-byte message, return. Otherwise move block to archive - 300+ - and skip out )

\ : receive ( check and decode received packet. ) +test ( returns if true, ) -test ( returns if false. Otherwise they ) pop

\ ( - skip-out - return from ) receive. ( Resulting stack need not be empty, since ) /forth ( will restore pre-interrupt

\ stack. ) pop ( must be in a word called by ) receive, ( it cant be nested )

\ : +get b ( send requested block from archive )

\ : get n ( send block number to request. Interrupt disabled lest reply interfer )

\ : put n ( send block )

\ : archive ( send blocks 0-161 - 9 cylinders ) icmp dhcp

\ Block 192

( Blocks to/from server )

: payload ( n-bn ) header #8 + n! header #10 ;

: +put ( nn ) #1026 udp over payload send + block 2* 2* #1024 last ;

: it ( b ) dup #2 + swap n@ #300 + block 2* 2* #1024 move ;

: -got ( b-b ) dup #-4 + n@ #2 #8 + or drop if it pop ; then ;

: receive +ethernet -arp +ip +udp -dns -dhcp -got

: +get ( b ) n@ #300 +put ;

: ... ( interrupt-protect words that transmit )

: get ( n ) cli #2 udp payload last sti ;

: put ( n ) cli #0 +put sti ;

: archive #161 for i put #1000 us -next ;

\ Block 193

\ ( Client can get or put blocks to server )

\ : payload n-bn ( 2 bytes were appended to UDP header for block number )

\ : +put nn ( send block number. Append block as last fragment. Packet length distinguishes two messages )

\ : it b ( move 1024 bytes from packet to offset block )

\ : -got b-b ( if a 2-byte message, return. Otherwise move block to archive - 300+ - and skip out )

\ : receive ( check and decode received packet. ) +test ( returns if true, ) -test ( returns if false. Otherwise they ) pop

\ ( - skip-out - return from ) receive. ( Resulting stack need not be empty, since ) /forth ( will restore pre-interrupt

\ stack. ) pop ( must be in a word called by ) receive, ( it cant be nested )

\ : +get b ( send requested block from archive )

\ : get n ( send block number to request. Interrupt disabled lest reply interfer )

\ : put n ( send block )

\ : archive ( send blocks 0-161 - 9 cylinders ) icmp dhcp

\ Block 194

( Format floppy ) empt forth #1 MagentaV hd

: array pop 2/ 2/ ;

: com align array $1202004D , $6C 2,

: done $03F4 a! p@ $D0 or drop if done ; then ;

: byte ( n ) ready p! ;

: sectors ( nn-n ) #18 for over byte hd @ byte dup #18 mod #1 + byte #2 byte #1 + next drop ;

: head ( nn-n ) dup hd ! $0400 * $1202004D + com ! seek com #6 command dup 2* - #1801 + sectors done ;

: cylinders ( n ) #0 swap for #0 head #1 head #1 + next stop drop ;

: format #12 cylinders ;

\ Block 195

\ ( Increase speed from 2 cylinders/s to 3 )

\ : array -a ( return next word address )

\ : com -a ( address of command string )

\ : done ( wait till last sector formatted. Till ready to read )

\ : byte n ( send byte to fdc when ready )

\ : sectors nn-n ( send 4 format bytes to each of 18 sectors. Sector number from 1 to 18 )

\ : head nn-n ( set head number. Issue seek and format commands. Starting sector number depends on cylinder, allowing 2 sector

\ times to step heads. Cylinder 1: 17 18 1 2 ... 16. 1801 + adjusts for 1s complement and for unsigned mod )

\ : cylinders n ( format both heads of each cylinder, starting at 0 )

\ : format ( standard number of cylinders. Smaller is faster )

\ Block 196

( Hard disk ) empt macro ( use this at your own ) risk

: 2/s ?lit $F8C1 2, 1, ;

: p!+ $42EE 2, ;

: 1! $91 1, drop ;

: insw 1! $97 1, $006DF266 3, $97 1, ;

: outsw 1! $96 1, $006FF266 3, $96 1, ; forth

: 2dup over over ;

: bsy $01F7 p@ $80 and drop if bsy ; then ;

: rdy ( -n ) $01F7 p@ #8 and drop if $01F0 a! #256 ; then rdy ;

: sector $01F3 a! swap p!+ #8 2/s p!+ #8 2/s p!+ #8 2/s $E0 or p!+ drop p!+ drop 2* 2* ;

: read ( an ) $20 sector #256 for rdy insw next drop ;

: write ( an ) bsy $30 sector #256 for rdy outsw next drop ; nload

\ Block 198

( boot: 3f fat0: 5f fat1: 25a5 dir: 2 cl forth: 8e6d cl )

: reg dup p@ $FF and #2 h.n space #3 h.n cr ;

: regs #7 for i $01F0 + reg -next ;

: ok show blue screen text regs keypad ;

: cl $20 * $4AAB + ;

: buffer $2000 block ;

: ?fort dup @ $54524F46 or drop ;

: cl0 dup #5 + @ $00010000 * swap #6 + @ #16 2/s $FFFF and or ;

: find ( -n ) buffer dup #2 cl read #256 for ?fort if #8 + *next drop ; then cl0 pop drop ;

: fort $8E6D cl ;

: +2 $8000 u+ $0100 + ;

: reads for 2dup read +2 next drop drop ;

: writes for 2dup write +2 next drop drop ;

: get buffer fort #9 reads ;

: cf! #0 fort #2 writes ;

\ Block 200

( Deskjet ) empty #2 +load

: nb #768 #3 * ; #4 +load

: pixels for pix next drop drop ;

: drow string $33622A1B , $622A1B4D , $5730 2,

: rpt drow #10 type drop ;

: columns for $0264 #2 wipes dup buffer #8 * #768 pixels line rpt rpt #2 + next drop ;

: res #300 2, #300 2, #2 2, ;

: esci string $306C261B , $6F2A1B4C , $1B4D312E , $3033742A , $2A1B5230 , $55342D72 , ( 32672a1b 4025736 res res res res

) $32722A1B , $53343033 , $30722A1B , $722A1B41 , $000C4362 3,

: print esci #37 type $F0000000 #767 #1024 * #2 * + #1024 columns #6 type drop ;

: tx string $3F and if $3F or if ; then $C0 or ; then ;

: text tx map ! print ;

: it table map ! print ;

\ Block 202

( Printer ) macro

: p@ $EC 1, ;

: p! $EE 1, ;

: @w $8B66 3, ;

: @b $8A 2, ;

: +a $C2FF 2, ;

: bts $0010AB0F 3, drop ;

: 2/s ?lit $F8C1 2, 1, ; forth

: ready p@ $80 and if ; then ready ;

: delay for next ;

: emit $0378 a! p! +a ready +a $8D or p! #30 delay #1 or p! drop ;

: type for dup @b emit #1 + next ;

: buffer $0264 block #4 * ;

: string pop ;

: !b dup - #7 and a! dup #3 2/s bts #1 + ;

: three !b

: two !b

: one !b

: nul drop ;

: white $FFFF and dup $FFFF or drop if - then ;

\ Block 204

( Deskjet )

: -nb nb negate u+ ;

: bcmy string $10243800 , $3033 , $00200022 , $10000011 , $C00F , $4003 , $00 , $00 , $0008000A , $00 , $00800002 , $00 ,

$04000005 , $00 , $00 , $C0000001 ,

: ye nb #3 * u+

: all over over #3 and jump nul one two three

: ma -nb #2 2/s all ;

: cy -nb #2 2/s all ;

: bl -nb #2 2/s all ; #1050918 MagentaV map

: 6b $C618 and #3 2/s dup #3 2/s or $03C3 and dup #4 2/s or $3F and ;

: table string bcmy + @b ;

: ex map @ push ;

: pix over @w 6b ex $FF and if ye ma cy bl then drop #3 + #1024 #-2 * u+ ;

: arow string $30622A1B , $4D 1,

: trbp string $32622A1B , $00563838 3,

: trbr string $32622A1B , $00573838 3,

: color #7 type drop nb #8 / type ;

: line arow #5 type drop buffer #3 for trbp color next trbr color drop ;

\ Block 206

( x18 simulator ) empty macro

: 2/s ?lit $F8C1 2, 1, ; forth

: state $1FFF block ; nload

: reset r #26 for $00100000 over ! #1 + next drop $0180 mem @ ir ! $0181 pc ! $00 slot ! ;

: un. #5 for #37 emit next ;

: undef $00100000 ? if drop un. ; then #5 h.n ;

: r. ( a-a ) dup @ undef cr #1 + ;

: stack sp @ $08 for dup ss r. drop #-1 + next drop ;

: return rp @ #8 for #1 + dup rs r. drop next drop ;

: ok show black screen text green return r r. blue r. r. white r. r. green r. r. drop stack keypad ;

reset ok

\ Block 207

\

\ : 2/s n ( shift right n bits )

\ : state -a ( address of state vector for current computer )

\ : reset ( set registers undefined, execute from ROM )

\ : un. ( display undefined register )

\ : h.n nn ( display n hex digits of number )

\ : undef n ( bit 20 set means undefined )

\ : r. ( display register )

\ : stack ( display stack, top at top )

\ : return ( display return stack, top at bottom )

\ : ok ( display registers, b a blue, pc ir white )

\ Block 208

( Registers )

: r state ;

: b state #1 + ;

: ar state #2 + ;

: pc state #3 + ;

: ir state #4 + ;

: t state #5 + ;

: s state #6 + ;

: slot state #7 + ;

: ss #7 and #8 + state + ;

: rs #7 and #16 + state + ;

: rp state #24 + ;

: sp state #25 + ;

: mem $2000 block + ; #4 +load #2 +load

: s1 ir @ #8 2/s inst ;

: s2 ir @ #3 2/s inst ;

: s3 #0 slot ! ir @ #4 and drop if ret then pc @ mem @ ir ! #1 pc +!

: s0 ir @ #13 2/s inst ;

: step slot @ jump s0 s1 s2 s3

: steps for step next ;

\ Block 209

\ ( Name 26 registers in state vector )

\ : ar -a ( A register. Cannot be named a because Pentium macro takes precedence )

\ : s0-s3 ( execute instruction from slot 0-3 )

\ : step ( execute next instruction )

\ : steps n ( execute n instructions )

\ Block 210

( Instructions )

: nul ;

: call pc @ +r

: jmp ir @ $01FF and pc ! ;

: jz t @ dup or

: jc drop if #3 slot ! ; then jmp ;

: jns t @ $00020000 and jc ;

: ret -r pc ! ;

: @b b @

: @x mem @ +t ;

: @+ ar @ #1 ar +! @x ;

: n pc @ #1 pc +! @x ;

: @a ar @ @x ;

: !b b @ #1 b +!

: !x -t swap mem ! ;

: !+ ar @ #1 ar +! !x ;

: !a ar @ !x ;

: inst ( n ) #1 slot +! $1F and jump jmp jmp call call jz jz jns jns @b @+ n @a !b !+ nul !a -x 2*x 2/x +* orx andx nul +x

r@ a@ t@ s@ r! a!x nul t!

\ Block 211

\ ( Define action of each instruction )

\ : inst n ( jump vector for 32 instruction codes )

\ Block 212

( Instructions )

: +r ( n ) r @ rp @ #1 + dup rp ! rs ! r ! ;

: -r ( -n ) r @ rp @ dup rs @ r ! #-1 + rp ! ;

: +t ( n ) t @ s @ sp @ #1 + dup sp ! ss ! s ! t ! ;

: -t ( -n ) t @ s @ t ! sp @ dup ss @ s ! #-1 + sp ! ;

: -x t @ $0003FFFF or t ! ;

: 2*x t @ 2* $0003FFFF and t ! ;

: 2/x t @ dup $00020000 and 2* or 2/ t ! ;

: +* t @ #1 ? if s @ + then 2/ t ! ;

: orx -t t @ or t ! ;

: andx -t t @ and t ! ;

: +x -t t @ + $0003FFFF and t ! ;

: r@ -r +t ;

: a@ ar @ +t ;

: t@ t @ +t ;

: s@ s @ +t ;

: r! -t +r ;

: a!x -t ar ! ;

: t! -t drop ;

\ Block 213

\

\ : +r n ( push onto return stack )

\ : -r -n ( pop from return stack )

\ : +t n ( push onto data stack )

\ : -t -n ( pop from data stack )

\ : -x ( some instructions named with terminal x to avoid Pentium conflict )

\ Block 214

( x18 target compiler ) empt #2097556 MagentaV h #2097555 MagentaV ip #2 MagentaV slot macro

: 2*s ?lit $E0C1 2, 1, ; forth

: memory $2000 block ;

: org ( n ) memory + dup h ! ip ! #0 slot ! ;

: , ( n ) h @ ! #1 h +! ;

: s3

: s0 h @ ip ! #13 2*s , #1 slot ! ;

: s1 #8 2*s

: sn ip @ +! #1 slot +! ;

: s2 #3 2*s sn ;

: i, slot @ jump s0 s1 s2 s3

: 25x #174 load ; #8 +load #2 +load #4 +load n x18 call class 25x

\ Block 215

\ ( Prototype for target compilers )

\ : h ( address of next available word in target memory )

\ : ip ( address of current instruction word )

\ : slot ( next available instruction slot )

\ : 2*s n ( shift left n bits )

\ : memory -a ( host address for target memory )

\ : org n ( set current target memory location )

\ : , n ( compile word into target memory )

\ : s0-s3 ( assemble instruction into slot 0-3 )

\ : i, ( assemble instruction into next slot )

\ : 25x ( compile code for multicomputer )

\ Block 216

( Instructions )

: nop $1E i, ;

: adr ( n-a ) slot @ #2 or drop if nop then i, ip @ ;

: call defer ( a ) #2 adr +! ;

: if ( -a ) #4 adr ;

: -if ( -a ) #6 adr ;

: then ( a ) h @ $01FF and swap +! ;

: @+ $08 i, ;

: @b $09 i, ;

: n defer #8 f@ execute $0A i, , ;

: @ $0B i, ;

: !+ $0C i, ;

: !b $0D i, ;

: ! $0F i, ;

: - $10 i, ;

: 2* $11 i, ;

: 2/ $12 i, ;

: +* $13 i, ;

: or $14 i, ;

: and $15 i, ;

: + $17 i, ;

\ Block 217

\ ( Words being redefined for the target computer. These Pentium words can no longer be executed. Although Pentium macros

\ still take precedence during compilation, they will no longer be used. )

\ : adr n-a ( assembles instruction, but not in slot 2, where address goes. Instruction address left on stack )

\ : call ( deferred to class. Executed for target defined words )

\ : then a ( puts address in low 9 bits of previous instruction word )

\ : n ( executed for green short-numbers. All 18-bit target numbers are short. Executes white short-number to put interp

\reted number on stack. Then assembles literal instruction with number in next location )

\ Block 218

( Instructions )

: pop $18 i, ;

: a $19 i, ;

: dup $1A i, ;

: over $1B i, ;

: push $1C i, ;

: a! $1D i, ;

: drop $1F i, ;

: ; #4 ip +! ;

\ Block 219

\ ( More target instructions )

\ : ; ( since it will be executed, it does not conflict with the Pentium macro )

\ Block 220

( 25x ROM ) $0180 org $00 dup - dup - dup - dup - dup - dup - dup - dup - dup push push push push push push push push push

a! a nop

\ Block 222

( Target )

: defer ( -a ) pop ;

: execute ( a ) push ;

: class ( a ) last #1 + ! ;

: f! ( an ) sp + ! ;

: f@ ( n-a ) sp + @ ; #1445 MagentaV ?com #1369 MagentaV csho

: empty empt #0 class csho @ ?com @

: functions ( aa ) #4 f! #6 f! ;

: x18 ( a ) #4 f@ ?com ! #6 f@ csho ! #1 f@ functions ;

\ Block 224

\ Block 225

\

\ Block 226

( Realtek rtl8139b ) macro

: move ( sdn ) $C189 2, drop $00C78957 3, drop $00C68956 3, $A4F3 2, $5F5E 2, drop ; forth

: 1us #1

: us ( n ) #550 #3 / * for next ;

: r ( n-a ) $02000000 device $14 + pci + 2/ 2/ ;

: rom ( a-n ) r @ ;

: 3rom ( nnn ) #4 rom #0 rom dup #16 for 2/ next swap ;

: tx ( -b ) $2000 block #4 * ;

: rx ( -b ) tx #1536 + ; #1 MagentaV ds #42 MagentaV fr

: n ( -a ) ds @ $10 r + ;

: send ( an ) fr @ tx + swap dup fr +! move ;

: first ( an ) n @ $2000 and drop if ds dup @ #1 + #3 and swap ! #0 fr ! send ; then first ;

: last ( an ) send tx ds @ $20 r + ! fr @ #60 max n ! ;

: reset $10000000 $34 r ! #100 us ;

: init rx $30 r ! 1us reset $0C000000 $34 r ! 1us $8A $44 r ! #3 ds ! $FB dup $21 p! $A1 p! sti

: /int $FFFF0001 $3C r ! ;

: rcvd ( -b ) $38 r @ dup $00010000 / $1FFF and $FFFFFFF0 + $38 r ! $10 + $1FFF and rx #4 + + ;

\ Block 227

\

\ : move sdn ( move n bytes from source to destination. Register 1 is used, 6 and 7 are saved )

\ : us n ( delay n microseconds. Edit cpu clock rate )

\ : r n-a ( word address of register )

\ : rom a-n ( fetch 2 bytes of mac )

\ : 3rom nnn ( 3 byte-pairs of mac )

\ : tx -a ( transmit buffer. 1536 bytes. Fragments must be assembled for transmission )

\ : rx -b ( receive buffer. 8k + 1532 byte overrun )

\ : ds -a ( must cycle thru 4 tx descriptors )

\ : fr -a ( must accumulate fragments in tx buffer )

\ : n -a ( tx status/length. Writing starts transmission )

\ : send an ( fragment into transmit buffer )

\ : first an ( fragment. Wait till buffer empty )

\ : last an ( fragment. Start transmission )

\ : reset ( controller )

\ : init ( ialize controller. Set tx/rx address/on and mac/broadcast. Enable irq10 )

\ : rcvd -b ( received packet. Register 38 is 10 bytes before start of next packet. Register 3a is end of current packet

\ )

\ Block 228

( Display registers )

: reg ( a ) dup r @ h. space #2 h.n cr ;

: regs $48 #19 for dup reg #-4 + next drop ;

: ok show red screen text regs keypad ;

\ Block 229

\

\ : reg a ( display register and address )

\ : regs ( display interesting registers )

\ : ok ( diagnostic display )

\ : 48 ( counter. Neat! )

\ : 44 ( rx configuration )

\ : 40 ( tx configuration )

\ : 3c ( interrupt )

\ : 38 ( rx count/address )

\ : 34 ( command )

\ : 30 ( rx 8k ring buffer )

\ : 2c-20 ( tx address )

\ : 1c-10 ( tx status )

\ : c-8 ( multicast id, unused )

\ : 4 ( mac 54 )

\ : 0 ( mac 3210 )

\ Block 230

( Ethernet ) empty #124 load

: empty empt logo cli ; macro

: w $66 1, ;

: w@ $8B 2, ;

: w! w $0289 2, drop ;

: *byte $C486 2, ; forth #126 load #128 load

: n@ w w@ $FFFF and *byte ;

: 2! a! w! ;

: n! a! *byte w! ;

: n, *byte 2, ;

: string pop ;

: packet string #-1 dup dup 2, 2, 2, 3rom 2, 2, 2, #0 n,

: length ( n ) packet #12 + n! ;

: 3! swap over 2! #2 + swap over 2! #2 + 2! ;

: ethernet ( n ) length packet #14 first ;

: +ethernet ( -a ) rcvd #14 + ; #132 load #134 load #136 load #138 load $72 interrupt

: serve forth receive /int 8clear /forth i; init ok

\ Block 231

\

\ : empty ( redefined to disable interrupts )

\ : w ( 16-bit prefix )

\ : w@ b-n ( fetch 16-bits from byte address )

\ : w! nb ( store 16-bits )

\ : *byte n-n ( swap bytes 0 and 1 )

\ : n@ b-n ( fetch 16-bit network-ordered number )

\ : 2! nb ( store 16-bit number )

\ : n! nb ( store 16-bit number in network order )

\ : n, n ( compile 16-bit number in network order )

\ : string -b ( returns byte address )

\ : packet -b ( ethernet packet header )

\ : dest -b ( destination field in packet )

\ : src -b ( source field )

\ : length n ( store length into packet )

\ : 3! nnnb ( store 3-word MAC )

\ : ethernet n ( send header with type/length )

\ : @ethernet -b ( return payload address of received packet )

\ Block 232

( ARP for a single correspondent )

: . ( n ) 1, ;

: message string $01 n, $0800 n, $06 . $04 . $01 n,

: me 3rom 2, 2, 2, ( IP ) #0 . #0 . #0 . #2 .

: to #0 #0 #0 2, 2, 2, ( IP ) #0 . #0 . #0 . #1 .

: sender #8 + ;

: target #18 + ;

: dir #6 + ;

: ip #6 + w@ ;

: ar ( n ) message dir n! $0806 ethernet message #28 last ;

: arp cli #-1 dup dup packet 3! #1 ar sti ;

: -arp ( b-b ) dup #-2 + n@ $0806 or drop if ; then pop drop

: me? dup target ip message sender ip or drop if ; then dup sender packet #6 move

: query? dup dir n@ #1 or drop if ; then sender message target #10 move #4 ar ;

\ Block 233

\ ( Set ip addresses with Edit. Normal order, net bytes first )

\ : . n ( compile byte. Resembles URL punctuation )

\ : message -b ( 28-byte string )

\ : me ( comment marking my mac/ip address )

\ : to ( comment marking correspondent )

\ : sender

\ : target

\ : dir -b ( fields in either ) message ( or received message )

\ : ip b-n ( fetch ip address )

\ : ar n ( send query 1, or reply 4 )

\ : arp ( broadcast query )

\ : -arp b-b ( return if not ARP. Otherwise process and skip out. )

\ : me? b ( return if broadcast not for me. Save sender only in packet )

\ : query? b ( if a request, reply )

\ Block 234

( ipv6 )

: header string $01000060 , $00 n, $17 . #64 .

: to $00 , $00 , $00 , ( IP ) #0 . #0 . #0 . #2 .

: me $00 , $00 , $00 , ( IP ) #0 . #0 . #0 . #1 .

: length ( n ) header #4 + n! ;

: dest header #20 + ;

: src header #36 + ;

: ip ( n ) $86DD ethernet length header #40 send ;

: +ip ( b-b ) dup #-2 + n@ $86DD or drop if pop ; then #40 + ;

\ Block 235

\ ( Set ip addresses with Edit. Normal order, net bytes first )

\ : header -a ( 40-byte ipv6 header )

\ : length n ( store 2-byte length in header )

\ : dest -a ( 4-byte destination ip address )

\ : src -a ( source ip )

\ : ip n ( send ip header embedded in ethernet packet )

\ : +ip b-b ( skip out if not IP. Otherwise return payload address )

\ Block 236

( UDP )

: b@ ( b-n ) w@ $FF and ;

: header string #0 n, #0 n, #8 n, #0 n, #0 n,

: length ( n ) #8 + header #4 + n! ;

: udp ( n ) dup #8 + ip length ;

: +udp ( b-b ) dup #-34 + b@ $17 or drop if pop ; then #8 + ;

\ Block 237

\

\ : b@ b-n ( fetch byte )

\ : header -a ( 8-byte udp header )

\ : length n ( store length in header )

\ : udp n ( send ip header for n-byte packet )

\ : +udp b-b ( skip out if not UDP. Otherwise return payload address )

\ Block 238

( Blocks to/from server )

: payload ( n-bn ) header #8 + n! header #10 ;

: +put ( nn ) #1026 udp over payload send + block 2* 2* #1024 last ;

: it ( b ) dup #2 + swap n@ #300 + block 2* 2* #1024 move ;

: -got ( b-b ) dup #-4 + n@ #2 #8 + or drop if it pop ; then ;

: receive +ethernet -arp +ip +udp -got

: +get ( b ) n@ #300 +put ;

: ... ( interrupt-protect words that transmit )

: get ( n ) cli #2 udp payload last sti ;

: put ( n ) cli #0 +put sti ;

: archive #161 for i put #1000 us -next ; lblk @ edit

\ Block 239

\ ( Client can get or put blocks to server )

\ : payload n-bn ( 2 bytes were appended to UDP header for block number )

\ : +put nn ( send block number. Append block as last fragment. Packet length distinguishes two messages )

\ : it b ( move 1024 bytes from packet to offset block )

\ : -got b-b ( if a 2-byte message, return. Otherwise move block to archive - 300+ - and skip out )

\ : receive ( check and decode received packet. ) +test ( returns if true, ) -test ( returns if false. Otherwise they ) pop

\ ( - skip-out - return from ) receive. ( Resulting stack need not be empty, since ) /forth ( will restore pre-interrupt

\ stack. ) pop ( must be in a word called by ) receive, ( it cant be nested )

\ : +get b ( send requested block from archive )

\ : get n ( send block number to request. Interrupt disabled lest reply interfer )

\ : put n ( send block )

\ : archive ( send blocks 0-161 - 9 cylinders )

\ Block 240

( ipv4 )

: header align string $4500 n, #0 n, #1 n, #0 n, $FF00 n, #0 n, #0 , #0 ,

: length ( n ) #20 + header #2 + n! ;

: +id header #4 + dup n@ #1 + swap n! ;

: checksum ;

: source header #12 + ;

: destination header #16 + ;

: ip ( n-n ) dup #20 + $0800 ethernet length +id checksum header #20 send ;

\ Block 242

( Howerds test block ) empty macro

: gtend $7E 1, here invert + 1, ;

: init $B803F0BA , $EEEE0055 , ; forth

: h $01E5 ; ( h last class macros forths )

: allot ( n- ) h +! ;

: mk2 here $10 + ; $40 allot

: mk $01E2 ;

: class $01E9 ;

: macros $01EA ;

: forths $01EB ;

: mk macros @ mk2 ! forths @ mk2 #1 + ! h @ mk2 #2 + ! ;

: mt mk2 @ macros ! mk2 #1 + @ forths ! mk2 #2 + @ h ! ;

: reload #0 push ;

: qkey #3 for i next ; #57 MagentaV ky

: key pause $64 p@ #1 and drop if $60 p@ dup $3A - drop -if ky ! ; then drop then key ;

: kk key ky @ #57 - drop if kk then ;

: pt $03F0 ; here $04 / $12345678 , ,

: conf cli init $00 pt p! pt $01 + p@ $01 pt p! pt $01 + p@ ;

\ Block 243

\

\ : kk ( shows key values . press esc to exit )

\ Block 244

( IR remote ) empty macro

: 2/s ?lit $F8C1 2, 1, ;

: p@ $EC 1, ;

: p! $EE 1, drop ;

: 1@ $8A 2, ;

: 1! a! $0288 2, drop ; forth

: ba #10 /mod $011F a! p! $0118 + a! ;

: b@ ba #0 p@ ;

: b! ba p! ;

: us #748 * time + -

: till dup time + drop -if till ; then drop ;

: ms #1000 * us ;

: array pop #2 2/s ;

: nul ; #3 MagentaV onf #145 load #146 load #50 load #147 load #148 load #149 load #150 load #151 load #152 load #153 load

#155 load #154 load

: h keypd nul nul quit bye +db -db mute nul +xx -ch jp vcr tv0 dvd cd fm nul nul nul nul nul nul nul nul nul nul nul nul

$00152500 , $00091016 , $11001016 , $0E0A1002 , #0 , #0 , #0 ,

\ Block 245

\ ( smsc ircc2.0 IR Consumer mode ) $32 #10 b! #0 #12 b! #0 #20 b!

\ : buffer #200 block #4 * ;

\ : reset $10 #7 b! $80 #4 b! ;

\ : on $40 #5 b! ;

\ : off #2 #4 b! #200 ms ;

\ : emit #6 b@ $40 and drop if emit ; then #0 b! ;

\ : rdy #6 b@ $80 and drop ;

\ : get #0 b@ over 1! #1 + ;

\ : bytes for

\ : byte rdy if get dup buffer #4096 + or drop if byte ; then drop pop drop ; then next drop ;

\ : r #200 #1 wipes $80 dup #4 b! #5 b! buffer #1000000 bytes #0 #5 b! ;

\ : word - #4 for dup emit #8 2/s next drop ;

\ : cmd for word next #1

\ : sp for #0 word next ;

\ : rate #22 b! #21 b! ;

\ : sync $80 #20 b! ;

\ Block 246

( App: Slime : simple game ) empty ( sounds ) #4 +load

macro

: @w $8B66 3, ; forth #2 MagentaV speed #13631840 MagentaV alice #29360784 MagentaV bob #0 MagentaV once #-1048576 MagentaV da

#-16 MagentaV db #17 MagentaV delay #25 MagentaV /del #-1 MagentaV off #0 MagentaV done

: mova da @ alice +! ;

: movb db @ bob +! ;

: qpel ( a- ) @ $00010000 /mod at vframe xy @ $00010000 /mod swap $0400 * + $02 * + @w $FFFF and #0 + if #1 done ! #1 off

! white bomb then ;

: clr #13 #65536 * #16 * #320 + alice ! #28 #65536 * #16 * #688 + bob ! #16 da ! #-16 db ! #0 delay ! #1 off ! #0 done !

#1 #1000 tn

: bgnd silver screen #16 #16 at black #1008 #672 box

: draw $FFFF color alice mova qpel #132 emit red bob movb qpel #133 emit ;

: tick off @ #0 + drop if ; then delay @ #-1 + delay ! -if /del @ delay ! draw click then ;

: b. ( c- ) $30 + 2emit ;

: ok show silver once @ #0 + drop if clr #0 once ! then silver #0 #708 at #600 #768 box #48 #708 at $00FFFF00 color #135

mute @ #0 + drop if #1 + then 2emit #0 emit speed @ #1 + b. tick keypad ; nload x ok h

\ Block 247

\ ( slime ) empt macro

\ : @w ( 16bit fetch )

\ : speed ( selected speed )

\ : alice ( 16:16 bit xy coordinate of left slug )

\ : bob ( 16:16 bit xy coordinate of right slug )

\ : once ( is set to initialise the game )

\ : mova ( move alice by the value in da )

\ : movb ( move bob by the value in db )

\ : delay ( counts the ticks for each move )

\ : /del ( the reset value for delay )

\ : qpel ( check for slime coloured pixel )

\ : clr ( set alice and bob to start positions )

\ : bgnd ( draw the background )

\ : draw ( the slugs )

\ : tick ( do this every screen update )

\ : ok ( the screen display )

\ Block 248

( Slime keypad )

: +speed #1

: +/-s speed @ + #0 max #9 min speed ! #10 speed @

invert + dup * #7 + #2 / #2 invert + /del ! ;

: -speed #-1 +/-s ;

: down #16 #65536 * da ! ; : up #-16 #65536 * da ! ;

: r #16 da ! ; : l #-16 da ! ;

: d2 #16 #65536 * db ! ; : u2 #-16 #65536 * db ! ;

: r2 #16 db ! ; : l2 #-16 db ! ;

: nul ;

: go #0 off ! ; : stop #-1 off ! ;

: x #1 once ! ;

: t off @ #0 + drop if #0 off ! ; then #-1 off ! ;

: help #249 edit ;

: mutet mute a @ invert mute ! ;

: h keypd nul quit t nul nul nul nul nul l2 u2 d2 r2 x nul stop go nul nul nul nul l up down r -speed help mutet +speed $742E

, #0 , $13121110 , $31302078 , #0 , $13121110 , $2B6E682D ,

\ Block 249

\ ( Slime keypad )

\ : ludr ( move Alice and Bob left up down up )

\ : x ( reset the game )

\ : 0 ( stop the game )

\ : 1 ( start the game )

\ : - ( decrease the speed )

\ : h ( to see this help screen )

\ : m ( mute the sound - on/off )

\ : + ( increase the speed )

\ : . ( quit )

\ : t ( toggle on/off )

\ : slime: ( two players control Alice and Bob. The first to hit any slime or the edges loses. )

\ : credits: ( Coded by Howerd Oakford from an idea by Alan Crawley and Paul Chapman )

\ : tested: ( by Hannah Oakford )

\ : type slime ( to play again )

\ Block 250

( Sounds ) #20 MagentaV tempo #-1 MagentaV mute #90 MagentaV period

: tn ( ft- ) tempo @ * swap #660 #50 */

: hz ( tf- ) push #1000 #1193 pop */

: osc ( tp- ) dup period ! split $42 p! $42 p!

: tone ( t- ) mute @ #0 + drop if drop ; then $4F $61 p! ms $4D $61 p! #20 ms ;

: click #1 #90 osc ;

: t #3 tn ;

: q #8 tn ;

: c #16 tn ;

: 2tone #75 q #50 q ;

: h1 #50 c #54 q #50 q #45 c #60 c ;

: h2 #40 c #45 q #50 q #50 c #45 c ;

: h3 #54 c #60 q #54 q #50 c #45 q #40 q #50 t #45 t #50 t #45 t #45 #12 tn #40 q #40 #32 tn ;

: hh

: handel h1 h2 h3 ;

: piano #55 #7 for dup q #3 #2 */ next drop ;

: cetk #6 c #10 c #8 c #4 c #6 #32 tn ;

: bomb mute @ #0 + drop if ; then $4F $61 p! #500 for #1000 i invert + split $42 p! $42 p! #1 ms next $4D $61 p! #1 #32 tn

;

\ Block 251

\ ( Sounds )

\ : tempo ( in ms per 1/8 quaver )

\ : mute ( equals -1 to disable sound )

\ : period ( test only - value sent to hardware )

\ : tn ( ft- play f Hz for t * 11 ms )

\ : hz ( tf- play t ms at f Hz )

\ : osc ( tp- play t ms of period p )

\ : tone ( t- play the current tone for t ms )

\ : click ( makes a click )

\ : t ( triplet )

\ : q ( quaver )

\ : c ( crotchet )

\ : 2tone ( 2 tones )

\ : h1

\ : h2

\ : h3

\ : hh

\ : handel ( part of Handels Gavotte )

\ : piano

\ : cetk ( Close Encounters of the Third Kind )

\ : bomb ( - well sort of .... )

\ Block 252

( App: colorforth editor ) empty nload qinit

: eddd jblk @ ok h ( drop ) ;

: edd ( b- ) jblk @ jlast ! jblk ! eddd ; blk @ jblk ! #206 jlast ! eddd

\ Block 253

\ ( The colorforth editor in colorforth )

\ Block 254

( Editor circular buffers ) #0 MagentaV cbn #0 MagentaV ends

: data ( - ) cbn @ $01 invert and cbn ! ;

: ptrs ( - ) cbn @ $01 or cbn ! ;

: heads ( - ) cbn @ $02 invert and cbn ! ;

: tails ( - ) cbn @ $02 or cbn ! ;

: cb@ ( -c ) ends @ cbn @ #8 * rshift $FF and ;

: cb! ( c- ) $FF and cbn @ #8 * lshift ends @ $FF cbn @ #8 * lshift invert and or ends ! ;

: cbnum ( -n ) cbn @ heads cb@ tails cb@ - $FF and swap cbn ! ;

: cbuf ( -a ) r@ $0100 / #2 + cbn @ $01 and + block ;

: tl- ( -n ) cbnum ?f drop 0if $00 ; then tails cb@ cbuf + @ cb@ #1 + cb! ;

: tl+ ( n- ) tails cbnum $FF - drop 0if tl- drop then cb@ $01 - cb! cb@ cbuf + ! ;

: hd@ ( -n ) heads cb@ cbuf + @ ;

: hd- ( -n ) cbnum $00 - drop 0if $00 ; then hd@ cb@ #1 - cb! ;

: hd! ( n- ) heads cb@ cbuf + ! ;

: hd+ ( n- ) cbnum $FF - drop 0if tl- drop then heads cb@ $01 + cb! hd! ; #4 +load

\ Block 255

\

\ : cbn ( bit 0 selects one of two circular buffers. Bit 1 selects head or tail value )

\ : cb@

\ : cb! ( read/write a byte to one of the 4 in ends selected by cbn )

\ : ptrs ( selects the pointer buffer )

\ : data ( selects the data buffer )

\ : heads ( selects the head value )

\ : tails ( selects the tail value )

\ : cbnum ( gives the number of items in the currently selected buffer )

\ : cbuf ( returns the address of the start of both buffers - the next 2 blocks )

\ : tl+

\ : tl-

\ : hd+

\ : hd- ( add or subtract from the head or tail of the currently selected buffer )

\ : ... ( note the tl- in hd+ . if the buffer is full we remove the oldest from the tail )

\ Block 256

( r App:ay buffer string Undo Display r    i r    s r    t r    l r    f r    d r    0 r    o r    ; r   r r   rr r   rt

r    e r   re r   ra r   rn r   ri r    a r   rc r   rl r   rf r   rd r    n r   r8 r   r; r   t r   tr r    i r   to r   te

r   ta r   tn r    s r   ts r   tc r   tl r   tf r    c r   t0 r   t8 r   t; r   o r    l r   ot r   oo r   oe r   oa r    f

r   oi r   os r   oc r   ol r    d r   od r   o0 r   o8 r   o; r    0 r   er r   et r   eo r   ee r    8 r   en r   ei r   es

r   ec r    ; r   ef r   ed r   e0 r   e8 r   r r   a r   ar r   at r   ao r   rr r   aa r   an r   ai r   as r   rt r   al

r   af r   ad r   a0 r   ro r   a; r   n r   nr r   nt r   re r   ne r   na r   nn r   ni r   ra r   nc r   nl r   nf r   nd

r   rn r   n8 r   n; r   i r   ir r   ri r   io r   ie r   ia r   in r   rs r   is r   ic r   il r   if r   rc r   i0 r   i8

r   i; r   s r   rl r   se r   sn r   ss r   sl r   rf r   s8 r   m r   mt r   me r   rd r   ms r   ml r   md r   m8 r   r0

r   ct r   ce r   cn r   cs r   r8 r   cd r   c8 r   y r   yt r   r; r   yn r   ys r   yl r   yd r   t r   l r   lt r   le

r   ln r   tr r   ll r   ld r   l8 r   g r   tt r   ge r   gn r   gs r   gl r   to r   g8 r   f r   ft r   fe r   te r   fs

r   fl r   fd r   f8 r   ta r   wt r   we r   wn r   ws r   tn r   wd r   w8 r   d r   ds r   ti r   vs r   p r   ps r   b

r   ts r   h r   hs r   x r   xs r   tc r   us r   q r   qs r   0 r   tl r   1 r   1s r   2 r   2s r   tf r   3s r   4 r   4s

r   5 r   td r   6 r   6s r   7 r   7s r   t0 r   8s r   9 r   9s r   j r   t8 r   - r   -s r   k r   ks r   t; r   .s r   z

r   zs r   ; r   : r   !s cccc cccc r   !s r   + r   +s r   @ bbbb r   @ r   ot bbbb r   *s )

\ Block 257

\

\ Block 258

( Display Undo string buffer ) #0 MagentaV jcur #64 MagentaV jblk

: sze ( -n ) $E0 ;

: qinit #0 ends ! $00 ptrs hd! $10000009 data hd! ;

: qnew ( - ) #0 ptrs hd+ ;

: qnum ( -c ) ptrs hd@ ;

: qpop ( -n ) data hd- ptrs hd@ #1 - if hd! ; then hd- drop drop ;

: qpush ( n- ) data hd+ ptrs hd@ $FF - drop 0if drop then ptrs hd@ #1 + hd! #0 #0 MagentaV pos #0 MagentaV lpos

: 2toc ( n-a ) jblk @ block pos @ + + ;

: xtoc? ( -n ) #1 2toc @ $0F and ;

: rtocs ( - ) jcur @ pos !

: ntocs ( -n ) #0 2toc @ $0F and #12 - ?f drop 0if #2 ; then #1 xtoc? ?f drop 0if #1 + then $FF and ;

: ltocs ( -n ) #0 pos !

: ltcs pos @ jcur @ - drop -if pos @ lpos ! ntocs pos +! ltcs drop then jcur @ lpos @ - ;

: mx ( n- ) jcur @ + #0 max #255 min jcur ! ;

: ml ltocs negate mx ;

: mu #8 for ml next ;

: mr rtocs mx ;

: md #8 for mr next ; nload

\ Block 259

\

\ : qinit ( initialises the queue pointers )

\ : qnew ( starts a new string entry )

\ : qnum ( -c number of cells in the top string )

\ : qpop ( -n returns the top cell of the top string )

\ : qpush ( n- stores n in the top string )

\ : ntocs ( number of tokens in the top string )

\ : qq ( n- ) qnew for qnum @ $0100 * $10000009 + qpush next cbnum drop ;

\ : qqq qinit #50 for #5 qq next #3 qq ;

\ : vvv ptrs cbnum data cbnum ;

\ : kk c vvv qpop ptrs hd@ ;

\ : gg cbuf dump ;

\ Block 260

( Editor Display ) #0 MagentaV cblind

: cb cblind @ #0 + drop ; #16 MagentaV state $10 MagentaV state*

: yellow $00FFFF00 color ;

: +txt white $6D emit space ;

: -txt white $6E emit space ;

: +imm yellow $58 emit space ;

: -imm yellow $59 emit space ;

: +mvar yellow $09 emit $11 emit $05 emit $01 emit space ;

: txts string $03010100 , $07060504 , $09090901 , $0F0E0D0C , ( ; )

: tx ( c-c ) $0F and txts + 1@ $0F and ;

: .new state @ $0F and jump nul +imm nul nul nul nul nul nul nul +txt nul nul +mvar nul nul nul ;

: .old state* @ $0F and jump nul -imm nul nul nul nul nul nul nul -txt nul nul nul nul nul nul ;

: state! ( n-* ) dup #0 + drop 0if drop ; then tx cb 0if drop ; then state @ swap dup state ! - drop if .old .new state @

#0 + if dup state* ! then drop then ; nload

\ Block 261

\

\ : state

\ : state! ( acts on a change of token type. It ignores extension tokens )

\ Block 262

( Editor Display ) macro

: @b $8A 2, ; forth #160 MagentaV jcnt #206 MagentaV jlast #2 MagentaV jcol

: bksp xy @ #22 $00010000 * negate + xy ! ;

: ?.cur jcnt @ #1 + #255 min jcnt ! jcur @ jcnt @ negate + #1 + drop 0if $00FF4040 color bksp $30 emit white then ;

: x xy @ $00010000 / ;

: ?cr x #1000 negate + drop -if ; then

: ncr xy @ #30 + $FFFF and $00030000 xor xy ! ;

: emt ?cr emit ;

: emit emt ;

: emitw unpack if emit emitw ; then space drop drop ;

: emitcs unpack if #48 + emit emitcs ; then space drop drop ;

: dig pop + @b $FF and emit ;

: edig dig $1B1A1918 , $1F1E1D1C , $13052120 , $0E04100A ,

: odig dup $0F and swap 2/ 2/ 2/ 2/ $0FFFFFFF and ; nload

\ Block 263

\

\ : ncr ( new cr -does not get confused with original )

\ Block 264

( CAPITALS HPO 2004 Editor Display )

: .hex odig if .hex edig ; then drop edig ;

: .dec #-1 ? -if negate #35 emit then

: n #10 /mod #-1 ? if .dec edig ; then drop edig ;

: num if $00C0C000 and color cb if #24 emit #21 emit then .hex space ; then color .dec space ;

: txt $00FFFFFF color emitw ;

: blu $FF color emitw ;

: cap $00FFFFFF color unpack #48 + emit emitw ; $00 MagentaV caps?

: caps $00FFFFFF color emitcs #-1 caps? ! ;

: ex bksp caps? @ ?f drop if caps ; then emitw ;

: gw $FF00 color emitw ;

: cw $FFFF color emitw ;

: yw $00FFFF00 color emitw ;

: coly #2 jcol ! ;

: colr #4 jcol ! ;

: colg #5 jcol ! ;

: colm #13 jcol ! ;

: colc #8 jcol ! ;

: colb #14 jcol ! ;

: rot $8B045E8B , $046E892E , $C38B0689 , $C3 1, #1220107268 MagentaV last nload

\ Block 265

\

\ : caps

\ : caps? ( is true if the extension token is CAPITALS )

\ : txt? ( returns true if the last token was text )

\ : .hex

\ : .dec

\ Block 266

( Editor display )

: short push dup 2/ 2/ 2/ 2/ 2/ swap $10 and drop pop num ;

: ys $00FFFF00 short ;

: long push #1 u+ $10 and drop dup @ pop num ;

: yn $00FFFF00 long ;

: gs $FF00 short ;

: gn $FF00 long ;

: var $00FF00FF color emitw #0 gn ;

: x xy @ $00010000 / ;

: rcr x #0 xor drop if cr then ;

: rw xy @ $FFFCFFFD + drop if rcr then $00FF0000 color cb if #41 emit space then emitw ;

: nuld drop ;

: .word ( w- ) dup #-16 and swap $0F and if $00 caps? ! then dup state! jump ex yw yn rw gw gn gs cw ys txt cap caps var

blu nuld nuld ( ; )

: t #0 jcnt ! jblk @ block text #3 lm #1024 rm #3 #3 at $10 state ! $10 state* !

: n dup @ #-1 ? if ?.cur .word #1 + n ; then drop drop $0F state! ; white #103 emit ;

: ok show $00200040 color screen t keypad ; nload

\ Block 267

\ ( CAPITALSALLTHEWAY! )

\ Block 268

( Editor aaaa bbbb cccc dddd keypad insertion )

: ripple ( a- ) dup dup @ over #1 + @ rot ! swap #1 + ! ;

: toc ( -a ) jblk @ block jcur @ + ;

: toend ( -n ) sze jcur @ - #0 max sze min ;

: del toc @ qpush toc toend for dup ripple #1 + next #0 swap ! drop ;

: dels jcur @ ?f drop 0if ; then ml qnew rtocs for del next ;

: ins ( n- ) sze jcur @ - ?f drop -if ; then jblk @ block sze + toend for #1 - dup ripple next ! ;

: undo qpop ins ;

: undos qnum ?f 0if drop ; then for undo next mr ; #25 MagentaV ky

: key pause $64 p@ #1 and drop if $60 p@ dup $3A - drop -if ky ! ; then drop then key ;

: lst ( n- ) jblk ! ok key drop ; nload

\ Block 269

\ ( Editor main keypad )

\ : ripple ( a- swaps the values at a and a+1 )

\ : bpush

\ : bpop ( push and pop the edit stack TBD )

\ : del ( removes the cell at the current cursor )

\ : dels ( removes the extension cells and one non extension coll before the cursor )

\ : undo ( puts back one cell )

\ : undos ( puts back one word which may have extension cells )

\ Block 270

( Editor keypad cursor )

: btog ( n-n ) dup #1 and drop if #1 invert and dup jblk ! ; then #1 xor dup jblk ! ;

: cbtog cblind @ invert cblind ! ;

: lastb ( n-n ) jlast @ dup jblk ! swap jlast ! ;

: blkld jblk @ $FFFFFFFE and #-32 + drop -if ; then jblk @ load ;

: -blk ( n-n ) #-2 + #18 max dup jblk ! ;

: +blk ( n-n ) #2 + #252 min dup jblk ! ;

: accep drop xx ;

: h keypd nul dels accep undos coly colr colg btog ml mu md mr -blk colm colc +blk colb nul nul nul cbtog nul nul lastb blkld

nul nul nul $00072515 , $2D0D010B , $0110160C , $2B0A0923 , $023A3800 , $03000029 , $3C ,

\ Block 272

( App: Conways Game of Life ) empty nload

: 1cell ( n-- ) #32 /mod adj adj over over at #16 u+

#16 + box ;

: nocell ( n-- ) drop ;

: draw ( n-- ) dup old @ #1 and jump nocell 1cell

: allcls ( -- ) #1023 for i draw -next ;

: gen ( -- ) #1023 for i tick swap new ! -next #1023 for i new @ i old ! -next ;

: locn ( --n ) row @ #32 * col @ + ;

: cur ( -- ) locn dup old @ $FF * $00FF0000 + color 1cell ;

: back ( -- ) black screen $00303010 color #40 #40 at

#583 dup box ;

: g ( -- ) show back green allcls gen keypad ;

: s ( -- ) gen show back blue allcls cur keypad ;

: clear ( -- ) #1500 #8 wipes #16 row ! #16 col ! s ;

: t ( -- ) locn old dup @ #1 xor swap ! ;

: col! ( n-- ) col +! col @ #31 and col ! ;

: l1 ( -- ) #-1 col! ;

: r1 ( -- ) #1 col! ;

: row! ( n-- ) row +! row @ #31 and row ! ;

: up1 ( -- ) #-1 row! ;

: dn1 ( -- ) #1 row! ;

: h keypd nul nul quit nul nul nul nul nul l1 up1 dn1 r1 nul nul nul nul glide glid2 glid3 glid4 clear s g t nul nul nul

rando $2E00 , #0 , $13121110 , #0 , $1C1B1A19 , $74677378 , $52000000 , clear glide g h

\ Block 273

\

\ : s ( stop )

\ : g ( go )

\ : t ( toggle the square )

\ : ludr ( left up down right )

\    ( press s to stop then draw a shape using ludr and t to toggle )

\    ( then press g to go or s to single step )

\ : 1234 ( create gliders which move to the four corners counting clockwise from the top left )

\ ( R loads random numbers )

\ Block 274

( Conways Game of Life ) #16 MagentaV row #16 MagentaV col

: old ( n-a ) cells #1500 block + ;

: new ( n-a ) cells #1504 block + ;

: rando ( -- ) #0 old $03FF for rand over ! cell+ next drop ;

: pos swap #32 /mod swap ;

: val #32 * + swap over old @ #1 and + ;

: up pos swap #31 + #31 and val ;

: dn pos swap #1 + #31 and val ;

: lt pos #31 + #31 and swap val ;

: rt pos #1 + #31 and swap val ;

: nul ;

: n2 #0 ;

: s2 dup old @ #1 and ;

: y2 #1 ;

: tick dup #0 up lt dn dn rt rt up up nip jump n2 n2 s2 y2 n2 n2 n2 n2 n2

: adj ( nn--nn ) swap #17 * #40 + ;

: st ( rc- ) col @ + swap row @ + #32 * + old #1 swap ! ;

: glide ( -- ) #0 $02 st #0 #1 st #0 #0 st #1 #0 st #2 #1 st ;

: glid2 #0 #0 st #0 #1 st #0 #2 st #1 #2 st #2 #1 st ;

: glid3 ( -- ) #0 #2 st #1 #2 st #2 #2 st #2 #1 st #1 #0 st ;

: glid4 ( -- ) #0 #0 st #1 #0 st #2 #0 st #2 #1 st #1 #2 st ;

\ Block 276

( Wave audio SB, 8 bit, mono, no DMA ) empt macro

: pb@ 0 $EC 1, ;

: pb! $EE 1, drop ;

: /8 $0008F8C1 3, ; forth

: +base $0220 + ; ( * )

: ?rd $0E +base a!

: *?rd pb@ $80 ? drop if ; then *?rd ;

: ?wr $0C +base a!

: *?wr pb@ $80 ? drop if *?wr then ;

: dsp@ ?rd $0A +base a! pb@ ;

: dsp! ?wr pb! ;

: ?init dsp@ $AA or drop if ?init ; then ;

: 0dsp #6 +base a! #1 pb! #30 for pb@ drop next #0 pb! ?init $D1 dsp! ; 0dsp

: *dac! $10 dsp! dup dsp! /8 ;

: dac! *dac! *dac! *dac! *dac! drop ;

: length #2 + dup #-1 + @ 2/ 2/ ;

: ?data dup @ $61746164 or drop if length + ?data ; then length ;

: sound #100 block #3 + ?data ; ( * )

: play for dup @ dac! #1 + next drop ;

\ Block 277

\

\ : pb@ ( -n get byte from port )

\ : pb! ( n- put byte to port )

\ : /8 ( n-n shift 8 bit right )

\ : +base ( n-n add base adress )

\ : ?rd ( wait for DSP read ready )

\ : ?wr ( wait for DSP write ready )

\ : dsp@ ( -n read DSP )

\ : dsp! ( n- write DSP )

\ : ?init ( wait until initialized )

\ : 0dsp ( reset 3 us DSP, turn on speaker )

\ : dac! ( n- write 4 byte to DAC )

\ : length ( a-an return length of record )

\ : ?data ( a-an search data record )

\ : sound ( -an return address and length of sound data )

\ : play ( an- play sound )

\ Block 278

( App: colorforth Explorer ) empty #9 MagentaV strt

: ?sze ( a- ) dup #510 block - drop ;

: crs ( n- ) ?f if for cr next #0 then drop ;

: docrs cr strt @ negate #0 max crs ;

: up1 ( a-a ) ?sze +if ; then $0400 + dup @ $FFFFFFF0 and $5C58BC80 - drop 0if ; then up1 ;

: upn ( n-a ) #0 max #64 block up1 swap ?f if for up1 next ; then drop ;

: ln ( a- ) #4 for cell+ dup @ dup $0F and $01 - ?f drop

0if drop leave then if dotsf then next drop ;

: .line ( a- ) ?sze +if drop ; then cr dup ablk . ln ;

: lines ( -- ) strt @ #0 max upn #20 strt @ negate #0 max - ?f if for blue dup .line up1 next then drop ;

: marker iconh #11 * #4 - ;

: qok show $4228 color screen #240 #0 at cblk block ln $00 color #0 marker at #1023 marker #30 + box #0 #0 at

docrs lines keypad ;

nload

\ Block 279

\ ( Scans the first cell of each block for App: )

\ ( and displays the first 4 words after App: )

\ : +

\ : - ( step through the applications )

\ : ? ( displays the applications first shadow block )

\ : o ( loads the application )

\ : . ( requires ) .word ( from the editor )

\ : up1 ( takes the address of the start block and steps through even blocks until it finds a token App: )

\ Block 280

( explorer )

: go strt @ #9 + upn ablk noshow ld ;

: md strt @ #1 - #-9 max strt ! ;

: mu strt @ #1 + #512 #4 / min strt ! ;

: qed strt @ #9 + upn ablk dup blk ! edit ;

: qh keypd nul quit go nul qed nul nul qed nul nul nul nul md nul nul mu nul nul nul nul nul nul nul nul nul nul nul nul

$002D2500 , $3F202065 , $00 , $2B20202D , $00 , $00 , $00 , qok qh

\ Block 281

\ ( Scans the first cell of each block for App: )

\ ( and displays the first 4 words after App: )

\ : +

\ : - ( step through the applications )

\ : ? ( displays the applications first shadow block )

\ : o ( loads the application )

\ : . ( requires ) .word ( from the editor )

\ : up1 ( takes the address of the start block and steps through even blocks until it finds a token App: )

\ Block 282

( fix the font )

: glyph ( c--a ) #48 * font @ + ;

: fix ( from to -- ) swap glyph swap glyph $30 cmove ;

: fixa $27 $3A fix $5B $01 fix $5C $02 fix $5D $03 fix

$20 $04 fix ;

: fixb $28 $5B fix $29 $5D fix ;

\ Block 283

\

\ Block 284

( grey = #-29727166  )oken ( test ) ( grey = #19138560  ) ( all tokens )

: tok ( t-n ) $11110000 + ;

: loc #2000 block ;

: set $10 for i #1 - tok loc i + ! next #9 tok loc ! #0 loc #17 + ! loc dump ; set

\ Block 288

( App: Timer Interrupt ) empty

( Interrupts ) #170 load

#0 MagentaV ticks

: !pit $34 $43 p! ( lo ) $A9 $40 p! ( hi ) $04 $40 p! ; !pit

: pic1! $21 p! ; : pic2! $A1 p! ;

: p@ p@ ; : p! p! ;

: ttb $20 p@ $21 p@ $A0 p@ $A1 p@ ;

: bpic cli ( init ) $11 dup $20 p! $A0 p!

( irq ) $00 pic1! $08 pic2! ( master ) #4 pic1! ( slave ) #2 pic2! ( 8086 mode ) #1 dup pic1! pic2! ( mask irqs ) $8F pic2

! $B8 pic1! ;

: npic cli ( init ) $11 dup $20 p! $A0 p!

( irq ) $20 pic1! $28 pic2! ( master ) #4 pic1! ( slave ) #2 pic2! ( 8086 mode ) #1 dup pic1! pic2! ( mask irqs ) $8F pic2

! $B8 pic1! ;

npic ( Note: npic will break bochs )

$20 interrupt

: timer0 forth: #1 ticks +! clear ;forth i;

( cli to disable interrupts , sti to enable )

sti

: test cli #0 ticks ! #1 secs sti #100 secs cli ;

: tm cli #0 ticks ! ;

lblk @ edit

( Type bye after loading in bochs !!!! )

\ Block 289

\ ( Timer Interrupt )

\ : !pit ( sets up the programable interval timer to )

\    ( 1 khz for a 1 ms tick )

\    ( for a clock of 14.31818 / 12 or 1.19318167 Mhz )

\    ( +/- 400 Hz this is actually 0.99985 +/- 0.0004 )

\    ( ms or about 0.015 percent fast. )

\ : pic1! ( write an octet to interrupt controller 1 )

\ : pic2! ( write an octet to interrupt controller 2 )

\ : !pic ( sets up the PIC chips )

\ $20 interrupt ( is the timer interrupt )

\ : timer0 ( the Forth code to run every timer tick )

\    ( use ) sti ( to enable interrupts, ) cli ( to disable )

\ : test ( run a 100 second test to time the timer )

\    ( interrupt with respect to the Real Time Clock. )

\ : tm ( measure cpu ms in timer ticks )

\ Block 290

( App: Sounds ) jmk #20 MagentaV tempo #0 MagentaV mute #1807 MagentaV period

: tn ( ft- ) tempo @ * swap #660 #50 */

: hz ( tf- ) push #1000 #1193 pop */

: osc ( tp- ) dup period ! split $42 p! $42 p!

: tone ( t- ) mute @ #0 + drop if drop ; then $4F $61 p! ms $4D $61 p! #20 ms ;

: click #1 #90 osc ;

: t #3 tn ;

: q #8 tn ;

: c #16 tn ;

: 2tone #75 q #50 q ;

: h1 #50 c #54 q #50 q #45 c #60 c ;

: h2 #40 c #45 q #50 q #50 c #45 c ;

: h3 #54 c #60 q #54 q #50 c #45 q #40 q #50 t #45 t #50 t #45 t #45 #12 tn #40 q #40 #32 tn ;

: hh

: handel h1 h2 h3 ;

: piano #55 #7 for dup q #3 #2 */ next drop ;

: cetk #6 c #10 c #8 c #4 c #6 #32 tn ;

: bomb mute @ #0 + drop if ; then $4F $61 p! #500 for #1000 i - + split $42 p! $42 p! #1 ms next $4D $61 p! #1 #32 tn ; 2tone

jmt

\ Block 292

( App: Test block : ) empty #0 MagentaV strt #62976 MagentaV lstup

: sze #256 block ;

: crs ( n- ) ?f if for cr next ; then drop ;

: up1 ( a-a ) dup sze - drop +if ; then $0100 + dup @ $FFFFFFF0 and $5C58BC80 - drop 0if dup lstup ! ; then up1 then ;

: .line ( a- ) dup sze - drop +if drop ; then cr dup $0100 / . #4 for #1 + dup @ dotsf next drop ;

: upn ( n-a ) ?f if #0 swap for up1 next ; then #0 ;

: lines strt @ negate #0 max crs strt @ #0 max upn #16 for up1 blue dup .line next drop drop ;

: ok show $00444444 color screen #240 #0 at r@ $0100 / block #4 for #1 + dup @ dotsf next drop $00 color #0 #266 at #1023

#296 box #0 #0 at lines keypad ;

: go strt @ #9 + upn $0100 / ld xx ;

: md strt @ #1 - #-8 max strt ! ;

: mu strt @ #1 + #256 min strt ! ;

: ?? strt @ #9 + upn $0100 / #1 + lst xx ;

: h keypd nul nul accept nul go nul nul ?? nul nul nul nul md nul nul mu nul nul nul nul nul nul nul nul nul nul nul nul

$00 , $2F000003 ,

\ Block 293

\ ( compilation )

\ : . ( the code for ) 2tone ( only exists for as long as it is needed )

\ Block 294

( App: Serial terminal ) empty #52 load #48 load

#65 MagentaV char #0 MagentaV qchar #0 MagentaV pos

: - ( nn-n ) negate + ;

: 0eq ( n- ) ?f if #0 #0 + drop ; then #1 #0 + drop ;

: 0neq #0 + drop ;

: eq ( nn- ) - 0eq ;

: crr pos @ $1E + $FFFF and pos ! ;

: cls black screen #0 pos ! ;

: act qchar @ 0eq if ; then pos @ $00010000 /mod swap at blue char @ chc emit xy @ pos ! char @ #13 eq if crr then char @

#12 eq if cls then #0 qchar ! ;

: wait pause qchar @ 0neq if wait then ;

: ch ( c- ) rkey? if rkey $FF and char ! #-1 qchar ! then ;

: ok c cls act #0 pos ! show ch act $00 #650 at $00202020 color #1024 #768 box keypad ;

\ Block 295

\ ( The next two blocks are a 256 character 8*8 pixel font )

\ : . ( display characters statically on the screen )

\ : . ( type ) ok ( then ) #65 ch #13 ch #66 ch

\ Block 296

( App: Mouse test ) empty vars dump mark

: kk vars dump hex $04 for ekey ;is ekey ekey ekey ekey ekey c next ;

: tt

: ps2 $D4 $60 pc! ;

: mm kstat ;

:  ;iso

\ Block 297

\

\ Block 298

\ Block 299

\

\ Block 301

\ ( Help screen )

\ ( F1 ) show this help screen or the start shadow

\ ( F2 ) toggle number base between decimal and hex

\ ( F3 ) toggle seeb display of blue words ( - )

\ ( F4 ) editor, toggle colorforth / colorblind mode

\ Block 302

( App: Floppy disk driver ) macro

: - $35 1, $FFFFFFFF , ;

: delay $E1E6 2, ;

: p@ a! dup $EC 1, delay ;

: p! a! $EE 1, delay drop ;

: 1@ $8A 2, ;

: 1! a! $0288 2, drop ; forth

: on $1C $03F2 p! ;

: off $00 $03F2 p! ;

: err -if off warm ; then drop ;

: msr $03F4 p@ $C0 and ;

: out $00100000 for msr $80 or drop if *next $00 - ; then $03F5 p! pop drop 0 ;

: in $00100000 for msr $C0 or drop if *next $01 - ; then $03F5 p@ pop drop 0 ;

: cmd for out err next ;

: conf $00 $70 $00 $13 $04 cmd ; $03 $A2 $03 $03 cmd ;

: sense $08 $01 cmd ; nload off

\ Block 303

\

\ : - ( ones complement, sets flags )

\ : delay ( dummy write, some hardware seems to need this )

\ : on - ( activate floppy )

\ : off - ( turn motor off, reset FDC )

\ : err n - ( warm start if SF set )

\ : msr - n ( get main status register )

\ : out n - ? ( write a byte to the FIFO, return error on timeout )

\ : in - n ? ( read a byte from the FIFO, return error on timeout )

\ : cmd x n - ( send n bytes to the FIFO )

\ : conf - ( some FDC commands, )

\ : spec -

\ : sense - ( see documentation for details )

\ Block 304

( Floppy disk driver )

: clrfifo in -if drop ; then drop drop clrfifo ;

: clrintr sense in err $80 and drop if clrfifo ; then clrfifo clrintr ;

: wait sense in err $80 and drop if clrfifo wait ; then clrfifo ;

: cal $00 $07 $02 cmd wait ;

: reset /flop $03 $A2 $03 $03 cmd $00 $70 $00 $13 $04 cmd ;

: init on pause spec conf clrintr cal ;

: xfer for in err over 1! $01 + next drop ;

: rd init push $FF $1B $12 $02 $01 $00 pop $00 $E6 $09 cmd block $04 * $0400 $12 * xfer off ;

: readid $00 $4A $02 cmd $07 for in err next clrintr ;

: version $10 $01 cmd in err $90 or drop if $02 - ; then 0 ;

\ Block 305

\

\ : clrfifo - ( discard all remaining input from the FIFO )

\ : clrintr - ( clear all pending interrupts )

\ : wait - ( wait for interrupt )

\ : cal - ( calibrate: move head to track 0 )

\ : reset - ( put FDC back to original state )

\ : init - ( initialize controller )

\ : xfer a n - ( reads n bytes from the FIFO to byte address a )

\ : rd b c - ( reads cylinder c to block b )

\ : readid ( for debugging )

\ : version - ? ( tests if your FDC supports enhanced commands )

\ Block 314

( EEEEEEE qkqq )

\ Block 332

\ Block 333

\

\ Block 337

\

\ Block 344

\ Block 400

( App: words )

#4621480 MagentaV from

#10308116 MagentaV tobl

#298 MagentaV num

#10000 MagentaV blkn

: tost ( u-- ) tobl @ ! #4 tobl +! ;

: ww ( -- ) #0 num ! #10000 blkn ! blkn @ block tobl !

ftha from !

fthn @ for from @ @ #9 or tost $25C7C00E tost

#4 from +! #1 num +!

num @ $06 mod #0 + 0if $9080000E tost then drop

num @ $06 #20 * mod #0 + mod 0if blkn @ #1 + blkn ! blkn @ block tobl ! #0 num ! then drop

next

#10000 edit

;

\ Block 401

\ ( Help screen )

\ ( F1 ) show this help screen or the start shadow

\ ( F2 ) toggle number base between decimal and hex

\ ( F3 ) toggle seeb display of blue words ( - )

\ ( F4 ) editor, toggle colorforth / colorblind mode

\ ( F5 ) rsn...

\ ( F6 ) shows the last block edited

\ Block 416

\ Block 428

\ Block 432

\ Block 433

\

\ Block 434

\ Block 459

\

\ Block 491

\

\ Block 500

( App: Icons font editor ) empty

#0 MagentaV ic

: showall ( -- ) #0 #0 #448 at #256 for dup emit #1 + next

drop ic @ #42 /mod #24 * #448 + swap #16 *

2dup at #16 #24 v+ red box ;

\ Block 501

\ ( Draw big-bits icon )

\ : @w a-n ( fetch 16-bit word from byte address )

\ : !w na ( store same )

\ : *byte n-n ( swap bytes )

\ : ic -a ( current icon )

\ : cu -a ( cursor )

\ : sq ( draw small square )

\ : xy -a ( current screen position, set by ) at

\ : loc -a ( location of current icons bit-map )

\ : 0/1 n-n ( color square depending on bit 15 )

\ : row a-a ( draw row of icon )

\ : +at nn ( relative change to screen position )

\ : ikon ( draw big-bits icon )

\ : adj nn-nn ( magnify cursor position )

\ : cursor ( draw red box for cursor )

\ : ok ( background task to continually draw icon, icon number at bottom )

\ Block 503

\

\ Block 504

( App: Stack usage analyser )

: oneline ( a-- ) cr dup $6600 - $08 * $6800 + blue h.4 space yellow $20 type ;

: ok show blue page text #8 lm #1024 rm

stacks $10 for dup oneline $20 + next drop

keypad ;

ok

\ Block 506

( App: Server tasks )

#0 MagentaV var1

: ttsv1 serv1 #1000 ms #1 var1 +! ;

: ksv1 isrv1 ;

ttsv1

#0 MagentaV var2

: ttsv2 serv2 #2000 ms #1 var2 +! ;

: ksv2 isrv2 ;

ttsv2

: ttclr #0 var1 ! #0 var2 ! ;

: ttstop isrv1 isrv2 ;

lblk @ edit

\ Block 507

\ ( Server tasks )

\ ( demonstrates how to run colorforth words in one of the two background tasks )

\

\ : ttsrv1 ( counts up var1 )

\ : ttsrv2 ( counts up var2 )

\

\ : ksrv1 ( kills the serv1 task by giving it a null )

\    ( action )

\ : ksrv2 ( the same for serv2 )

\ : ttclr ( clears both counter variables )

\ : ttstop ( stops both tasks from running )

\ Block 508

( Arithm )

\ Block 509

\

\ Block 510

( Sandbox  o98 any old ASCII  ovk@ )

: rrrr push ;

: tttt pop ;

: test ( your code here ) ;

\ Block 511

\ ( Help screen )

\ ( F1 ) show this help screen or the start shadow

\ ( F2 ) toggle number base between decimal and hex

\ ( F3 ) toggle seeb display of blue words ( - )

\ ( F4 ) editor, toggle colorforth / colorblind mode

\ ( F5 ) rsn...

\ ( F6 ) shows the last block edited

\ Done.

# Appendix D “Coloring Forth”

Coloring Forth

Because we must deal with the unknown, whose nature is by definition speculative and outside the flowing chain of language, whatever we make out of it will be no more than probability and no less than error.

–         EDWARD SAID

Beginning, Intention and Method

Motivation

Core Words

Specials

bye

Quit ColorForth.

| Stack | No effect |
| --- | --- |

ColorForth Source

bye

Assembler

Bye: push 0 call ExitProcess

Macros

macro2 dd offset semi dd offset cdup dd offset qdup dd offset cdrop dd offset then dd offset begin

; semi

Overview

Semicolon – terminates current definition.

Implementation

Implementation is non-trivial – it provides both tail-recursion support and some optimization. If the last compiled item was a call to a word, it’s being replaced with a jmp. Otherwise, ret is compiled.

List variable contains address of last compiled word.

H stands for HERE

H dd 0 list dd 0, 0 semi: mov edx, [H] sub edx, 5 cmp [list], edx jnz @f cmp byte ptr [edx], 0e8h jnz @f inc byte ptr [edx] ; jmp ret @@: mov byte ptr [5+edx], 0c3h ; ret inc [H] ret

Sample

Forth x 1 2 + ; y x x ;

Code

x: 008A07B5 8D 76 FC lea esi,[esi-4] 008A07B8 89 06 mov dword ptr [esi],eax 008A07BA B8 01 00 00 00 mov eax,1 008A07BF 05 02 00 00 00 add eax,2 008A07C4 C3 ret y: 008A07C5 E8 EB FF FF FF call x 008A07CA E9 E6 FF FF FF jmp x

This sample deserves some explanation.

EAX contains the topmost element of data stack. ESI points to the second element. In order to put the value 1 onto stack, we have to store current topmost element in memory and decrement stack pointer, and only then load the constant into EAX. This operation takes 2 Pentium commands, 5 bytes, XXX ticks . Not so bad.

As we can see, 2 + is compiled into something shorter and better, we’ll look at literal optimization later.

Now, for the word X the semicolon ; has compiled ret, while for Y the second call to X has been replaced with a jump.

dup cdup

Implementation

As we can guess from the code below, cdup stands for “compile dup”. It compiles 5 bytes onto the top of the dictionary, and we have seen these 5 bytes above. So, dup is implemented as a macro, which compiles code to push the topmost element from EAX onto in-memory stack.

cdup: mov edx, [H] mov dword ptr [edx], 89fc768dh mov byte ptr [4+edx], 06 add [H], 5 ret

Traditional Forth implementation could look like this.

hex : dup 89fc768d , 06 c, ; immediate

?dup qdup

Implementation

qdup: mov edx, [H] dec edx cmp [list], edx jnz cdup cmp byte ptr [edx], 0adh jnz cdup mov [H], edx ret

This code looks whether the last compiled instruction was 0adh, which stands for

lods dword ptr [esi]

And this is nothing else than a drop – move second element into the top of stack register, and increment stack pointer. So, ?dup works as dup, though if last compiled instruction was drop, it shifts HERE one byte back – actually, “uncompiles” the drop.

This trick is used for optimizing macros – immediate words – that compile code with stack notation ( -- n), for example, 0, A, and pop, following words, which take one item off stack ( n -- ) – for example, push (traditional >R), ! or A!.

0 ?dup c031 2, ; a ?dup c28b 2, ; pop ?dup 58 1, ; ! ?lit if ?lit if 5c7 2, swap a, , ; then 589 2, a, drop ; then a! 950489 3, 0 ,drop ; push 50 1, drop ; a! ?lit if ba 1, , ; then d08b 2, drop ;

The macro 0 puts zero onto stack. It compiles into

xor eax, eax

A puts the value of the address register onto stack. Corresponding Pentium code is

mov eax, edx

Let’s consider an example

x1 here 2/ 2/ dup push a! 0 a ! pop @ ;

This code moves values 1 and 0 into the cell on the top of the dictionary.

| here 2/ 2/ push drop a! 0 dup a ! pop @ ;. | 008A07CF E8 8B 12 B6 FF call here (00401a5f) 008A07D4 D1 F8 sar eax,1 008A07D6 D1 F8 sar eax,1 008A07D8 8D 76 FC lea esi,[esi-4] 008A07DB 89 06 mov dword ptr [esi],eax 008A07DD 50 push eax 008A07DE AD lods dword ptr [esi] 008A07DF 8B D0 mov edx,eax 008A07E1 B8 00 00 00 00 mov eax,0 008A07E6 8D 76 FC lea esi,[esi-4] 008A07E9 89 06 mov dword ptr [esi],eax 008A07EB 8B C2 mov eax,edx 008A07ED 8B D0 mov edx,eax 008A07EF AD lods dword ptr [esi] 008A07F0 89 04 95 00 00 00 00 mov dword ptr [edx*4],eax 008A07F7 58 pop eax 008A07F8 8B 04 85 00 00 00 00 mov eax,dword ptr [eax*4] 008A07FF C3 ret. |
| --- | --- |

Implicit dups and drops are highlighted with light blue. In future I will be replacing Pentium instructions with dup and drop macros. It’s interesting to notice that dup is 5 bytes, white drop – only one.

This example, though a bit artificial, illustrates how address register and store-fetch pair works, as well as introduces “address as offset from 0 in cells” and “address as offset from 0 in bytes”. I’ll be calling these cell address and byte address.

.